STRUCTURAL INSPECTION

Date:
June 09, 2006,
Type:
Primary residence
RE:
Post Hurricane Katrina Structural Inspection

Structure:
Wood frame, CMU Piers, Vinyl siding, Shingle roof
To:

David Seagrave
Reference:
251 Evella Dr.






Slidell, La






985-960-3186
History:

Mr. Seagrave contacted Dammon Engineering to perform a structural inspection on the above residence.  His concern is that his raised single story home on rebar reinforced CMU piers was destroyed during hurricane Katrina.  The home was built in the 1960’s and had approximately 900 s.f. elevated living area.  The main floor was at approximately 13 ft 6 inches.
Findings:

During a visual inspection of the home, the following was noted:

1. The concrete foundation and cable reinforcement are all that remain of this home.

Nearby Property  
1. Approximately 200 homes along Carr Drive, Evella Dr and Jacqueline Dr. were existing prior to the storm, only a very small percentage of the homes are habitable or in varied states of disrepair.
2. Many of these homes had their main living floor at or above sixteen ft. NGVD, and as such this part of their homes that were built on columns was above the flood and, having breakaway walls between the columns, could have survived a flood of this nature, but not the damaging winds.
3. There are neighboring homes that had their first floor at or near the same elevation as this one that only sustained one half inch of water in the home due to broken windows or doors. Wind blown siding and roof shingles were apparent to neighboring homes.
4. Approximately 21 homes located at the intersection of Debbie Dr. and Jacqueline and Evella Dr. are all destroyed. The homes that remained were built to current construction codes or better.
5. Allstate Insurance determined that the home at 255 Evella, was destroyed totally by wind.
Analysis

1. Investigation of how the hurricane acted on this and other homes on Carr Dr. incorporates testimony from persons who rode out the storm in a home that was no more than one mile from this dwelling, and by others, including myself, who rode out the storm in Slidell, Lake Catherine, as well as videos of the storm in progress taken on Carr Drive in Slidell.

2. Many of the homes on Carr Dr. had their floor elevation (first habitable floor) at or above elevation 16, above the FEMA requirement for this area and some three to four feet above the flood water that came after noon on August 29, 2005.
3. There was no tidal wave, but a fast rising tide that his these houses.  Without a tidal wave to crush the house, other means have to be blamed, namely the high winds and tornadic activity.

4. Following the house debris being scattered around the area, the flood waters took the debris inland.  
The winds of hurricane Katrina were blowing from the northeast and north until the eye of the storm passed.  .
Hurricane force winds ripped the Northshore/Carr Drive area starting before dawn on Monday, August 29, 2005, some six hours prior to the flood that ensued later that morning.  The western edge of the eye of the hurricane passed over this area later that morning, probably about 10:30 a.m.  These winds out of the northeast and north were gusting and causing roofs and whole houses to shutter and come apart under relentless wind pressures of from 30 psf to 45 psf, during the pre-dawn and early morning hours.  These gusting wind speeds caused variable wind pressures that were capable of shaking and dismantling the houses, long before flooding arrived, the flood peaking after noon on Monday, August 29.  Wind direction during this period was first out of the northeast prior to the eye passing and then the north and northwest following the eye passing.

Winds on Lake Pontchartrain are confirmed by the Louisiana State Office of Climatology to have reached 130 mph with gusts well over that speed.  With this, variable forces of up to 40,000 lbs. acted on the walls, windows, doors, etc., having the capability of shaking the building apart and blowing it into Lake Pontchartrain.

This building has been exposed to hurricanes and floods in the past.  Hurricane Isidor, with its similar flood characteristics, did no structural damage to the residence.  Hurricane Katrina surge waters had reached the shallow marshes and Lake Borgne long before reaching this area, and surges that were seen along the Mississippi Coast did not happen here, as they had been dampened by traveling across long stretches of these shallow areas, and were being dampened and blown to New Orleans and Chalmette by the winds out of the northeast.

The homes on Carr Drive had two and three story buildings, and many of the taller buildings had columns and breakaway walls to guard against water damage.  They were also demolished.  There is only one answer – the wind did the major damage, and the water carried off the remnants.

Reports from people who rode out the hurricane in Rigolets Estates, Fort Pike, Irish Bayou, Lakeview Drive, Carr Drive and myself on Bayou Liberty, all attest to the fact that there was no great surge or wall of water that would have pushed over buildings, but instead a fast rising tide in all these locations, that followed the relentless wind.  These people also reported that the flood water did not reach 16 feet above sea level, and that they saw airborne roofs and other parts of buildings, boat houses, etc., fly by them in their respective locations.

Conclusion:

It is my professional opinion based on eyewitness reports of the several references listed below as having been eyewitness, and my own observations during the storm, that this home was destroyed prior to the major flooding that occurred during the hurricane, and that the wind tore the house from its support columns after subjecting it to such wind pressures that were above those called for in the design of buildings prior to the requirements of the International Code for Construction of dwellings. In addition tornadic activity was present during this hurricane and should not be over looked.
Eyewitnesses:

1.
Mr. Leonard Brown – Rigolets Estates


Marlin Drive @ Moray Drive

Rigolets Estates

Slidell, Louisiana

2. Joe Perez

(Volunteer Fire Department)

St. Nicholas Church

Lake Catherine Hwy 90

New Orleans, Louisiana

3.
Kennard Jackley (video)


265 Carr Drive


Slidell, Louisiana

4.
Emmett G. (Pete) Dammon


34487 Torregano Rd. (Bayou Liberty)


Slidell, Louisiana

5.
Jimmy Ray Lemoine


Irish Bayou


New Orleans, Louisiana

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

Attachments:
Maps



Hurricane Katrina Effects on Lake Pontchartrain


This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


