STRUCTURAL INSPECTION

Date:
October 12, 2005

Type:
Primary residence

RE:
Post Hurricane Katrina Structural Inspection

Structure:
Wood frame on concrete /Wood Piers, seal tab roof shingle



Vinyl Siding

Owner(s): 
Janie Lampkin








Property: 
272 Marina Dr.



Slidell, La 70458



985-781-1906

History:

Mrs. Lampkin contacted Dammon Engineering to perform a structural inspection on the referenced residence.  She was concerned about the foundation/structure of there home after hurricane Katrina‘s high water She reported that the floor was sagging in and around the kitchen and dinning room.

Findings:

After a visual inspection of the home it was noted that there was substantial damage to the roof shingles and felt paper. The vinyl siding was blown off of the home in several locations on the north wall. Sofit and facial siding was also missing in many areas of the home. Water damage was noted at the upstairs ceiling,

The home is built on concrete poured footings with 2X4 trusses all running East and West. Floor joist are 2X12 12ft long running North and South. The floor joist are placed 12 inches on center.

The floor in the kitchen/dining room is saging from the center wall out either side out five feet.  A differential reading of the floor shows a -.75.

 Conclusion:

The water that entered the home weakened the floor joist supporting the center load-bearing wall, in the kitchen/dining room, causing the floor to sag. Additional floor joist and piers should be added and the home leveled.

It is my professional opinion that once the repairs are done this dwelling will be structurally sound.

The damage to the structure and the foundation is a direct result of rising water and wind..

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


