STRUCTURAL INSPECTION

Date:
November 30, 2005

Type:
Primary Residence

RE:
Post Hurricane Katrina Structural Inspection

Structure:        2 Story Wood frame, vinyl siding.

Owner(s): 
Preston Armond

Property: 
3021 Meadow Lake East 



Slidell La 70461

History:

Mr. Armond contacted Dammon Engineering to perform an inspection on the above referenced residence.  He was concerned about the structural integrity of the residence after a new roof was being installed and was not properly covered prior to the heavy rainstorm. 

Findings:

During a visual inspection of the residence, the following was noted:

A) The sheetrock on the 2nd floor front bedroom windows have leaked and are stained. The water ran down the windows onto the baseboards and carpet. 

B) The sheetrock ceiling and carpet in the master bedroom and bathroom are stained from the rainwater. 

C) The sheetrock ceiling and the carpet in the right rear bedroom is stained.

D) The sheetrock ceiling in the hall bathroom is stained.

E) The sheetrock ceiling in the kitchen is stained.

F) The sheetrock ceiling in the dining room is stained.

G) The rainwater has damaged the insulation in the attic.

H) The light fixtures in the master bedroom and master bathroom had rainwater running out of them.

Analysis

1. All the insulation in the attic should be removed and sprayed for mold, and the wood structure allowed too dry properly. Install new insulation.

2. Remove the sheetrock in the master bedroom ceiling and install new. Tape, float, and apply texture and paint to match existing color.

3. The remaining stained sheetrock ceilings and walls shall be painted with kilz paint and then painted to match the existing.

4. The light fixtures that are affected by the rainwater shall be replaced.

5. All carpet and flooring affected by the rainwater shall be removed and replaced with new to match existing. Prior to installing new carpet, kill all mold and allow proper drying time.

6. All trim and baseboards that were affected by rainwater shall be cleaned and painted.

Conclusion:

It is my professional opinion that after all of the repairs are completed this dwelling will be structurally sound. 

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings, nail patterns and all other work should meet the new International Building Code requirements for that work.

Information homeowners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


