STRUCTURAL INSPECTION

Date:
November 4, 2005

Type:
Rental property

RE:
Post Hurricane Katrina Structural Inspection

Structure:
Wood frame, on concrete block and red brick piers/silver metal roof /Wood siding

Owner(s): 
Oneal Gallaway

Property: 
342290 Laurent



Slidell, La 70458



985-7290-2014

History:

Mr. Gallaway contacted Dammon Engineering to perform a structural inspection on the referenced residence.  He was concerned about the fact that their home came off of its foundation during hurricane Katrina. The owner stated that floodwaters rose to seven feet or more inside the residence and that tornados where sighted in the area of the residence.

Findings:

During a visual inspection of the home, the following was noted:

1. The home came off of it piers and rotated clockwise shifting approximately thirty feet.

2. The home cracked in half and the floors have piers pushing up through the floor.

Analysis

1. The hurricane winds blew first from the northeast, the trees on this property substantiate this. The house likely was floated by the floodwaters and blown by the wind until it hit the trees that stopped it.

Conclusion:

It is my professional opinion that this dwelling is structurally repairable but if the cost to raise and move the home back to its original position approaches fifty five percent of the cost of demolition and reconstruction then the later should be considered.

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Building Code requirements for that work.

Information homeowners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


