chuck
STRUCTURAL INSPECTION

Date:
January 25, 2006

Type:

Primary residence

RE:

Post-Hurricane Katrina Structural Inspection

Structure:
Wood frame/Brick Shingle Roof

Owner(s):
Ms. Terry Lambert

Property:
418 Moonraker



Slidell, La 



285-1556

History:

Ms. Lambert contacted Dammon Engineering to perform a structural inspection on the referenced residence.  Mrs. Lambert had concerns about the amount of force the walls took on the south side of the home during Hurricane Katrina.  The garage that is in the rear of the home is completely gone.  The elevation of the slab is 13.6’.  There was approximately two feet of floodwater inside the home during the hurricane.
Findings:

1. A visual inspection of the residence, starting outside in the rear of the home, revealed that all the back windows have been blown out due to the combined force of wind and water.

2. The roofing shingles on the house received substantial damage from the wind and had to be replaced.

3. The detached two-car garage was completely blown away.  Only the foundation remains.

4. The in ground pool system is damaged and the pool is full of debris.
5. A visual inspection of the interior ceiling showed signs of previous water leaks from the roof.

6. At the time of the inspection, the bottom 4’ of sheetrock and most of the debris had been removed from the interior of the home.
Analysis:

1. The home was damaged by both wind and water.  The wind first blew building components off the structure; the water then intruded the home bringing in debris.
2. The damaged walls in the rear of the home should be rebuilt.

3. The garage was damaged by both wind and water.  The wind removed the structural components which were holding the building together.  The rising water removed the remaining loose material, leaving only the foundation.
4. All building components have exceeded the wind loads that they are designed for and should be replaced.

Conclusion:

It is my professional opinion that the main framing of this home is structurally sound, and once the above work is accomplished, the home will be structurally sound.

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Building Code requirements for that work.

Information homeowners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

Attachments:
1.  Hurricane Katrina Effects on Lake Pontchartrain


2.  Map of North Lake Pontchartrain

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


