STRUCTURAL INSPECTION

Date:
August 29, 2006

Type:
Primary residence

RE:
Post Hurricane Katrina Structural Inspection

Structure:
Raised Wood Frame Home/Beams and Pilings/Asbestos Siding/Asbestos Shingle Roof

Owner(s): 
Kenny Kararick


Mailing Address: 
Kenny Kararick



4500 Touché Rd.





35436 Hwy 433

New Orleans, La 





Slidell, La 70460

History:

Mr. Kararick contacted Dammon Engineering to perform a structural inspection on the above residence.  The house had 1,943 s.f. of area and was built in 1954 on eight inch piles with the floor approximately six feet above the ground.  With the ground being approximately elevation 7, NGVD, this put the main floor at approximately elevation 12.  The property is at the end of the Touché road with water surrounding three sides.  South of the property is unprotected marshland and Lake Catherine.  Mr. Kararick stated that after hurricane Katrina the house was missing and only the woodpiles remained of the foundation.  He stated he has photos of the trees that fell pointing to the west.  Appliances from the home were found in the canal north of the home and bathroom fixtures were found in the canal south of the home.  Homes in the immediate area that partially survived Katrina sustained severe damage to the east sides of the structures.

Findings:

During a visual inspection of the property, the following was noted:

1. No pilings or joist remained on the property at the time of inspection.

2. The concrete stair pad was all that remained.

Analysis

The winds of Hurricane Katrina were blowing from the northeast and north until the eye of the storm passed.  The disbursed items from the home and the direction the trees fell support the theory that the structure was destroyed by the wind prior to the flood arriving, and that the flood took what was left of the broken building from its supports and into the canal, which is north and east of the property.

Hurricane force winds ripped the Lake Catherine area starting much before dawn on Monday, August 29, 2005, prior to the flood that ensued later that morning.  The western edge of the eye of the hurricane passed over this area later that morning, probably about 8:30 a.m.  The winds out of the northeast and north were gusting and causing roofs and whole houses to shutter and come apart under relentless wind pressures of from 30 psf to 45 psf, during the pre-dawn and early morning hours.  These gusting wind speeds caused variable wind pressures that were capable of shaking and dismantling the houses, long before flooding arrived, the flood peaking around noon on Monday, August 29.  Wind direction during this period was first out of the northeast and north prior to the eye passing and then from the northwest and then west to southwest following the eye passing.

Winds on Lake Pontchartrain are confirmed by the Louisiana State Office of Climatology to have reached 130 mph with gusts well over that speed.  With this, variable forces of up to 40,000 lbs. acted on the walls, windows, doors, etc., having the capability of shaking the building apart and blowing it into the canal.

This building has been exposed to hurricanes and floods in the past.  Hurricane Isidor, with its similar flood characteristics, did no structural damage to the residence.

Hurricane surge waters had reached the shallow marshes and Lake Borgne long before reaching this area, and surges that were seen along the Mississippi Coast did not happen here, as they had been dampened by traveling across long stretches of these shallow areas, and were being dampened and blown to New Orleans and Chalmette by the winds out of the northeast.  When the water did come, it rose fast, but without a major wave to knock down houses.

The homes in this area had two and three story buildings, and some of the taller buildings had columns and breakaway walls to guard against water damage.  These homes were generally above the flood height, but the wind, being there first did its damage to the houses that were not built to any particular code.  They were also demolished.  There is only one answer – the wind did the major damage, and the water carried off the remnants.

Reports from people who rode out the hurricane in Rigolets Estates, Fort Pike, Irish Bayou, Lakeview Drive, Carr Drive and myself on Bayou Liberty, all attest to the fact that there was no great surge or wall of water that would have pushed over buildings, but instead a fast rising tide in all these locations, that followed the relentless wind.  These people also reported that the flood water did not reach 16 feet above sea level, and that they saw airborne roofs and other parts of buildings, boat houses, etc., fly by them in their respective locations.

Conclusion:

It is my professional opinion that the house was first battered and demolished by the winds of Hurricane Katrina before the flood waters came, washing away much of the debris into the canal.

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Building Code requirements for that work.

Information homeowners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

Attachments:
Effects of Hurricane Katrina



Map

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


