STRUCTURAL INSPECTION

Date:
March 31, 2006
Type:
Business
RE:
Pre purchase inspection
Structure:
Single story wood frame, concrete foundation, corrugated metal siding, composition shingle roof

To:

Steve Ladner
Reference:
60181 Hwy 11.






Slidell, La



774-3487



History:

Mr. Ladner contacted Dammon Engineering to perform a structural inspection on the above business location. 
Findings:

During a visual inspection of the business, the following was noted:

1. The roof consists of a composition shingle roof with a metal deck roof installed over the shingles.  Seventy five percent of the metal roof has been blown off.
2. There is damage to the roof shingles and roof decking.

3. Approximately forty percent of the soffit and fascia is damaged and/r broken.

4. The ceilings are water damaged.
5. The concrete foundation is in good condition.
6. The visual inspection of the property revealed no signs of termites.

Conclusion:

The damage to the structure is repairable. There may be additional damage not seen until the coverings are removed. It is my professional opinion that once all repairs are accomplished this dwelling will be structurally sound.

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Code requirements for that work.

Information owners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


