Kirt

STRUCTURAL INSPECTION

Date:
October 5, 2005

Type:
Residential

RE:
Structural Inspection

Structure:
Wood frame on conventional slab, brick and vinyl siding, seal tab roof
Owner(s): 
Clark Gristina

Property: 
78220 Coyne Rd.



Covington, La



(985) 632-3333

History:

Mr. Gristina contacted Dammon Engineering to perform a structural inspection on the referenced residence.  He was concerned about the cracks in the concrete foundation.

Findings:

A visual inspection of the exterior of the residence revealed that the walkways on the east side of the home showed settlement cracks. Approximately 15 ft from the northeast corner of the home and near the first window, there is a crack running west from the edge of the walkway up the foundation and continuing under the carpet in the interior of the home.  On the west side of the home there are surface cracks in the vicinity of the location as if the crack continues across the entire foundation.  Removal of all floor coverings in the affected location would be required to confirm the continuance of the crack running east to west.

Analysis:

It appears that settlement in the foundation has occurred on the north wall of the home.  This could be caused by inadequate compaction of the soil prior to construction.  It is recommended that pilings are placed on the north side of the foundation to prevent further settlement of the foundation. 

Conclusion:

It is my professional opinion that with this work accomplished, the building will be structurally sound.
When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Code requirements for that work.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


