Structural Inspection Follow-up

Wednesday, May 21, 2008

Following are specific concerns that were stated in the inspections dated October 11, 2005 and October 12, 2005.  Mr. Pete Conner contacted Dammon Engineering on this date to request a follow up inspection on areas of concern as listed:

Building A.  on East wall revealed substantial damage on the front exterior wall.  Condition now is--------

Building A.  on North wall near parking lot sustained substantial damage to the   front exterior wall     on all three levels.  Was apparently severely infested by termites.  Condition now is---------------------   

 Does company now have a termite exterminator company employed?  Was wall completely and correctly rebuilt?

Are hurricane straps correctly installed at all required locations?   

Are beams now properly attached to the piles?

Was stucco on north side of building either repaired or replaced?

What was the disposition of the two piles on southwest corner of the structure that were showing signs of deterioration at the ground level on our previous inspection?

Building B (South)

Was any correction to the piles supporting the walkway between buildings completed?

(Walkway has shifted and beams are off center)  

Rot was observed on the 3rd floor balcony roof, and also on beams supporting the third floor walkway at the northwest corner.  What is disposition?

Beam at northwest corner of structure appeared to have warped over time.  Although still sitting on the piles it needed attention.  What has been done?

Building A.  was not deemed to be structurally sound.  What has been done to correct these findings?  Was concrete floor removed and replaced?  Was the east wall removed and replaced with new material?  Have beams been attached to the pilings as per requirements of the International Building Code?

Has ALL rotted wood in all buildings been replaced?

Building B.  Has the beams at the northwest edge of the structure been repaired by attaching it to the piling?

Have ALL floor beams been properly attached to the pilings?

Are buildings now being treated for termites as required by the IBC?

