[image: image1.png]DAMMON ENGINEERING, INC.

dammonengineering.com

ENGINEERS ARCHITECTS
CONSULTING DESIGN STUDIES EXPERT WITNESS
1095 Florida Ave. P.O. Box 2830 985-649-5832
Slidell, LA 70458 Slidell, LA 70459 FAX 985-641-5950

Structural Inspection

November 15, 2005

Post-Katrina Structural Inspection

Materials: Wood Frame/Vinyl Siding/Metal Roof on Wood Columns, Strapped to House

Frame
Owner(s): Garry and Judy Mackles Mailing Address: 17 Country Club
Location: 11033 Tahiti Street Laplace, LA
Bay St. Louis, MS 39520-7604 985-652-2261/504-231-4762

History:

Ms. Mackles contacted Dammon Engineering, Inc. to inspect their hurricane damaged property
on the Jordan River in Bay St. Louis and to try to determine if their vacation home was destroyed
by the flood that accompanied Hurricane Katrina, or if it was the victim of high winds, or a
tornado.

It was reported that hurricane winds racked the area on Monday morning August 29, prior to the
flood, which rose very fast to approximately the level of the floor of the main living level of the
Mackles home.

Findings:

1. Generally speaking, as can be seen in the photos, the house is gone, removed from the
pilings and framework that supported it, and deposited almost three blocks away, while
another unidentifiable house is sitting on the front of their property.

2. The walls of one room, which was on the ground level of the structure, remained intact
but open to the sky. Cabinets and other items in this room that were nailed or screwed to
the interior walls and studs were pulled loose and are missing.

3. See accompanying sketch. The family TV was found approximately 200 feet southeast
of the property, the main pile of rubble (furniture, framing, roof, etc.) was found
approximately three blocks southwest of the house, and items such as the 10° x %"
ground rod for their electrical service meter was found in the yard, northeast of the house,
opposite from the side where the service was located. {The meter panel and ground rod
were missing from the piling it had been located on.) Appliances were found in the
general area of the main rubble pile, but not all together.

4. The path from the home site to the main rubble pile is generally along a canal that runs
more or less east-west at their house, with little or no obstruction of trees, buildings, etc.




[image: image2.png]Analysis:

The high winds of Katrina, blowing counter clockwise around the eye of the storm, would have
first presented winds generally out of the east or southeast, with this area in the eastern section of
the track of the eye. The wind would later switch to come from the south and then the west.
These winds were blowing in the 180-200 mile per hour range with higher gusts when this
property was in the northeast quadrant of the storm.

The wind preceded the water, as the wind must do to make the flood waters rise. Because of the
different directions of many of the findings of Mr. & Mrs. Mackles’ belongings, it is very likely
that a tornado or tornadic winds first did the damage, ripping the house from its foundation and
depositing in it in already rising and fast moving water, so that it traveled to the area where most
of the house now lies.

Conclusion:
It is my professional opinion that the high winds of Katrina wrecked the house prior to any
involvement of the flood water.

Sincerely,

IS

& ,/»‘ZZzég,w ;{’/ %(\—p—ﬁ—a -
Emmett G ¢) Damman, PTE.

This inspection is limited to the apparent visuai conditions of the structural components of this building. it does not cover, nor attempts to cover, any
components, ilems, and/or conditions which, by their nature or focation, are concealed or are difficult or hazardous to inspect, or which require the
moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut. No opinions are
expressed regarding conditions which could be discovered only by the disassembly of any component parts, special festing, or removal of any
goncealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under
unusual weather canditions, such as, but nat limited 1o floods, heavy rains or snows, high winds, temperature extremes, or any act of God. Specific
hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, sofl, environmental, radon, carbon
monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected. Our liability shall be
limited to reimbursement of the total cost of inspection.




