DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

February 8, 2007
STRUCTURAL CERTIFICATION

For:

Sonny Drouilhet

Location:
135 N Roadway


New Orleans, La 70460

To Whom It May Concern:

Building Description:  Two story boat house with living space/wood frame on pilings/metal roof
Scope: 

This inspection is limited to a visual inspection of the pilings above the water level. No inspection of the mechanical or electrical systems, interior and exterior walls, ceilings, floors, or roof was performed.

History:

Mr. Drouilhet contacted Dammon Engineering to perform a structural inspection on the piling system of the referenced residential property.  Mr. Drouilhet stated that water did enter the building to an approximate height of four feet during Hurricane Katrina.  The building is approximately thirty five years old.

Findings:

This inspection covered visible components of the pilings above the water level on this day and their % of capability to support the home with bearing loads as compared to their original size.
1. Piling #1 has deteriorated to fifty percent. Recommend repairing by concrete encasement with steel reinforcing.
2. Piling # 16 has a split running vertically above and below the water line approximately ½” deep. Recommend repairing by concrete encasement with steel reinforcement
3. The remaining pilings have deteriorated approximately twenty percent or less. No action need be taken at this time, but to guarantee longevity all piles should be treated with concrete and steel reinforcement.
4. The cross braces have rotted and are falling off of the piles. Replacement of the cross braces is recommended.

Conclusion:

I certify the piling system for the above residence is structurally sound and capable of being restored to near its original design load values.

Sincerely,

Emmett Dammon

La. License 8796
[image: image1.jpg]AVMAVOd HLIYON

avaH N8

'SONMId 0L Q3HJVLLY
1ON AONV d3L10d SI ONIOVYd SSO40 3HL
‘aoom d3AvO3a ANV d3XVOS d3LvM
Y04 ONIMOTIV SINIWINSYIN IN3HdIND 3IHL
NOY4 d310vd1l8ns 38 Q1NOHS HONI 3NO -~ @
RELER
Y3LVM 3HL 3A08V d313AWVIA LSITIVANS 3HL LV
J3¥NSYIN SYM JONIHAWNDYIO INIFJ¥ND 3HL
Jlld 3HL 40 dOL JHL 1V d3dNSVIW
SYM 3ONIYANNOHIO TYNIOIMO 3IHL @
3AIL MOT LY N3MVL 3¥3M SINIWIINSYIN
‘Av030 ANV NOISOdd3
¥04 Q3LS3L Fd3IM STTUd FHL- 'L

S31ON

na

aoo9 ’ ; L¢371d
aooo ) : 0c¢31ld

Q009 i i 6131Id
Qo009 L : 8131ld
aoo9 : : [131d

NI . : 913Id
Qo009 : : GLITd
Q009 ) ; y131id
Q009 : : C13Tid
aooo L y Z131id
Q009 : ) LL3Id
Q009 : 0L31d
Q009 631d

Q009 : : g3ld
4009 i : (3id
Q009 : : 937Id
lefelebs} : S3Id
Q009 ; $31d
Q009 y ¢3id

N . Z37id

¥00d ! 1371d

ANNJYID WNOdID
OILIONOD|LNIYINO[INIHENO | IWNIDINO| WNIDINO| #311d

g

|
m
!
M
m
w
:
m

DMpDOY "N Gl

ONIJTINION  NOW

INI

i

N seL

KAompooy
oypInoag Auuos

Qe 0ol 6 @ ©

¥ZI0L V1 'SUDSHO MON





