Carport Foundation Inspection

Friday, November 16, 2007

Owner:

W. A. Clarke
Contact #:
985-847-0407
Location:
215 Tierra Street

Slidell, LA 70460

History:

Mr. Clark contacted Dammon Engineering to request an inspection of a concrete slab so he may obtain an Engineers stamp.  He has plans to build a carport on this foundation.

Findings:

The slab is 25’x 25’ in size.  There is a 12” deep, 8” wide footing around the perimeter and down the center of the slab.

The footings each have four #5 rebars and the 5” slab has 6”x6” highway mesh.
This slab was poured approximately 6-7 years ago, has weathered well and appears to be in excellent condition.  Mr. Clark has plans designed for construction of a carport.

Analysis:

This foundation construction exceeds the requirements for use as a carport and should present no problem being used for this purpose.

It is my professional opinion that the foundation is designed and constructed in excess of the minimum requirements for the use intended.  The inspection was performed by and this report was written by me and is true and correct to the best of my knowledge and belief.  This report shall serve as my affidavit that the foundation has been erected in accordance with the reviewed plans, and with Chapter 4, Table R403.1 provisions of the 2006 International Residential Building Code and that I hereby assume full responsibility for the compliance with all provisions of the technical code for the foundation and all other pertinent laws or ordinances as pertains to the foundation of this structure.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

La. License 8796

