DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS




ENGINEERS


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


554 Old Spanish Trail



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

Structural Inspection

February 18, 2011

For: 


Mr. Ed Fitzgerald

Property Inspected:
305 Dover Street

       


Slidell, La. 70458 

Construction:

Single story, wood frame with brick veneer and composition shingle roof, the home is approximately 45 years old.

Scope:

This inspection is limited to a visual inspection of the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. Fitzgerald contacted Dammon Engineering to request a structural inspection of the above listed residence. Mr. Fitzgerald was concerned about the cracks in the brick veneer located on the left side of the home.

Findings:

The brick veneer on the left side of the home was noted to have hairline cracks near the rear bedroom window and the master bedroom window. These cracks do not go through to the foundation. 

Analysis:

First, it is significant to note that brick veneer is not structural and that the cracks in the brick veneer, while they are an indication that something has happened, do not necessarily indicate that something is amiss.

Secondly, houses and foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  The house continues to settle and usually by the tenth year this settlement is infinitesimal.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, tree roots, etc.

In this case, it is possible that there was initially some differential settlement.

Bricks/mortar also crack due to expansion and contraction when exposed to extreme afternoon temperature or when the sun bears directly on it.

Conclusion:

It is my professional opinion that the cracks discussed above do not pose a structural problem, and that future expansion/contraction in the wall and settlement of the slab will be infinitesimal and will not affect the structural integrity of the house.

The majority of settling usually occurs in the first year after construction and gradual settlement then follows for about ten years, after which the structure should be relatively stable.  Settling may also be the result of the local weather conditions causing expansion or shrinkage of the soil.  The brick veneer and other concrete products expand and contract, causing cracks that are not necessarily structural problems.

This residence is structurally stable and will probably not show any major settling in the future.

The cracks should be sealed with one of the commercial products designed for that purpose.  A non-shrinking mortar, or caulk, should be used to fill the openings to prevent insects from entering the residence.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


