DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS					ENGINEERS

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail				   					     985-649-5832

Slidell, LA 70458								            FAX 985-641-5950

# Structural Inspection

April 11, 2012

For: 	Dale Dixon

Century 21

Re: 	501 River Oaks Drive.

Covington, La.

Construction:

Two-story, wood frame with brick/vinyl veneer and composition shingle roof on a conventional foundation

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Dale Dixon with Century 21 contacted Dammon Engineering to request a structural inspection of the home due to a dip in the floor located right inside the front door (in the foyer) of the home.

Findings:

Immediately inside the front door of the residence, is a very noticeable lean, or slope, in the floor.

Due to the extent of the slope in the floor, A Zip Level was used to measure the difference in elevations from the rear of the home to the front of the home.

It was noted that there is a 3” difference in elevation from the rear of the home to the front of the home.  It appears that the whole house is leaning forward towards the street.  There were no signs of cracks in the foundation or the sheet rock of the home. There are some cracked ceramic tiles in the foyer along the wall next to the dining room.  There appears to be termite damage in the garage and in the rear door of the home.

Conclusion:

Houses and foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  The house continues to settle and usually by the tenth year this settlement is infinitesimal.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

The apparent settlement of this residence is such that corrective action will be necessary to level the structure.

It is recommended that a leveling company be engaged to shore up and re-level the home.

A termite company should also be contracted to treat the home.

Once the home has been re-leveled and the termites eradicated, a contractor should then be hired to repair damage created by the termites and any issues related to the settlement of the residence, such as cracked tiles, etc.

Sincerely,

Emmett G. (Pete) Dammon, P.E.