554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

info@bellsouth.net

# Structural Inspection

June 10, 2015

For: 	Linda White

61081 Donya St.

Slidell, LA 70460

Construction:

One-story, wood frame, wood & brick veneer, composition shingle roof on a concrete foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence.  No inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Linda White contacted Dammon Engineering requesting a structural inspection of the referenced residence due to an appraisal that noted cracks in the brick veneer and foundation.

Findings:

This home is approximately 50 years of age and has had the addition of living area(s) added to the original structure throughout the years. The owner stated that the residence was originally a three bedroom home with a small porch and carport. Since then, the non-living areas have been enclosed and incorporated into the home's living area.  It is evident where the concrete topping of these (previously non-living) areas was brought up to match the existing elevation of the original living area of the home.  The concrete is cracked in several places along the joints.

The front, sides and rear exterior of the home was noted to have several cracks in the mortar and brick veneer and some into the foundation.  Some repairs have been made to the exterior brick and mortar of the home. There was no evidence of sheet rock cracking around doors or windows.

Analysis:

There are no gutters and down spouts on the home, which is allowing the water to drain off the roof and drain toward the foundation;  This will cause settlement of the foundation.  A French drain around the home is needed to carry the water away from the home directly to the front ditch.

Structures typically show signs of settlement in the sheetrock at the heads of doorframes and windows, this is due to the fact that these areas offer minimal structural support.  At the time of inspection no cracks were noted in these areas.

Conclusion:

House foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

The settling of home does not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound and the home is capable of being remodeled.

However, the foundation will continue to shift with any ground movement and may experience adverse settlement if standing water continues to accumulate near the home.

It is recommended that the home be leveled/patched in the living room area prior to the remodel of the home and French Drains be installed around the home to keep the foundation dry. Termite protection should be applied in the cracks of the foundation before hand to deter termite activity.  After the home is leveled, re-point all mortar and cracked bricks.

Sincerely,

Brian A Mistich P.E.

La License No. 30187