[image: image1.jpg]Dammon Engineering, Inc.

(985) 641-5950 Fax (959 649-5832  dammonengi@ellsouthnct
1095 Florida Ave

Slidell, LA 70458

BILL TO:

Mr. William Dahl
103 Baryberry Drive
Covington, LA 70433

RE: 66 Talbot Lane/Mandeville

DESCRIPTION
Structural Inspection and Report for Above Referenced Property

" Thank You.

invoice
DATE | INVOICE #
1/11/2007 5057

AMOUNT
550.00

Total Due* €550.00




[image: image2.jpg]



[image: image3.jpg]



[image: image4.jpg]Analysis:
The post tension foundation is designed to hold the foundation together as the foundation

settles. The crack is due to settlement and likely happened during the first five years of its
life. Further settlement is unlikely because of the age of the home.

The crack in the brick ledge is common for concrete foundations and is of no structural
concern.

Conclusion:
I certify the above residence is structurally sound.

Sinccrely,

5

Emmett ammon
La. License 8796





[image: image5.jpg]DAMMON ENGINEERING, INC.

dammonengineering.com

CONSULTING DESIGN STUDIES EXPERT WITNESS
1095 Florida Ave. P.O. Box 2830 985-649-5832
Slidell, LA 70458 Slidell, LA 70459 FAX 985-641-5950

January 10, 2007

STRUCTURAL CERTIFICATION

For: Woody and Lori Dahl

Location: 66 Talbot Ln
Mandeville, La 73396

To Whom It May Concern:

Building Description: Wood frame, post tension concrete foundation, composition shingle
roof, and brick veneer on four sides. The residence is approximately 3,396 sq. ft. and fifteen
years old.

Scope:
This inspection is limited to a visual inspection of the interior and exterior walls, ceilings,

floors, roof and foundation. No inspection of the mechanical or electrical systems was
performed.

History:

Mr. Dahl contacted Dammon Engineering to perform a structural inspection on the referenced
residential property. Mr. Dahl is considering purchasing the home and his home inspection
noted a crack in the foundation, which he wished to have inspected by a professional
engineer.

Findings:
This inspection covered visible components (foundation, and wall) and their ability to support
the home.

A visible crack is seen in the foundation on the right side of the home near the bathroom
window. The crack appears to travel through the foundation and across to the A/C return air
filter grill inside the hallway of the home and past that point. Wood and carpeted floors
prevent further inspection. No cracks in the walls, ceilings or brick facade are visible. The
doors and windows open and close normally.

There is a visible crack in the brick ledge at the front right corner of the house.




