554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

May 15, 2023

Mr. Joe France

Building Safety Permits

P.O. Box 828

Slidell, La. 70459

Re:

1133 St Paul St.

Slidell, La. 70460

Dear Mr. France

Eric Myers requested that Dammon Engineering inspect the existing walls prior to closing in @ the above address.

The repairs shall be;

- Remove the bad insulation board/vapor barrier that is damaged in that area.

- Reattach brick ties with horizontal cripplers between each stud.

- Install nailers and install foam board between each stud and silicone in place.

This report does not cover any other aspects of the building such as framing, plumbing, electrical, gas, mechanical egress, general arrangement, site drainage, etc., and assumes that ground conditions are substantially the same as is under the residence.

Sincerely,

Brian A Mistich, P.E.

La. License No.  0030187