STRUCTURAL INSPECTION

Date:
July 18, 2005

Type:
Residential

For:
Bob Marloff

             1709 Fourth Street

Slidell, La 70458

Re:    Brook Wood Subdivision

         125 9th Street

         Slidell, La. 70458

History:

The home is approximately 28 years old. It is wood frame construction on a concrete foundation with brick veneer. The home was re-leveled in the right rear of the home near the kitchen sometime ago. The owner has contacted us to perform a structural inspection due to cracks in the sheet rock, and to see if the house needs to be re-leveled.

Findings:

Upon inspection, it was noted that there were a couple of small hairline cracks visible in the sheet rock wall of the kitchen and ceiling of the garage. Also the right front garage wall (exterior) was also noted to have small hair line cracks that do not go into the foundation.

Analysis:

This house, being the age it is, has gone through many years of settlement. The cracks in the gypsum walls are a direct result of this settlement and judging by the age of the paint, etc., have existed for some time.  Patching the existing cracks should prove to be a lasting remedy.

Conclusion:

It is my professional opinion that the house is structurally sound.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796
This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


