554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

February 16, 2022

Gayle Macomber

KW Realty Professionals

2053 E. Gause Blvd- Ste 100

Slidell, La. 70461

RE:

108 Athene Drive

Slidell, La. 70460

Construction:

One-story, wood frame, brick veneer, ceramic, vinyl & wood flooring with a composition shingle roof on a pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to bulking floor and cracks in sheetrock.

Findings:

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up at the front of the home in the front foyer. The slopes range in the home from 0.0” to -1.0” and are within the limits of elevation differences. Most elevation differences are due to types of flooring.

It was also noted there are stress cracks in the sheetrock walls above the doors on the right side of the home due to ground settlement in this area. The wood flooring in the front right room is buckling due to water getting underneath the wood flooring between it and the moisture barrier. (You can clearly see water stains on top of the wooden floors from either fish tanks or watering plants).

An overall visual inspection of the exterior of the home was conducted and under the home is dry; there are no noted cracks in the brick veneer. It was also noted that there are no gutters or down spouts on the home and rainwater is splashing onto the brick walls creating mold. This also allows for settlement of the soil around the home and will create uneven floors and crack sheetrock walls. Due to a high water table in this area it is necessary to keep the footings dry.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is our opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Add gutters and downspouts and tie into subsurface drainage on all downspouts that are needed to allow for proper drainage of rainwater away from the foundation.

- Repair sheetrock.

- Repair flooring

See attachments;

Elevations

Pictures

Sincerely,

Brian Mistich, P.E.