554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

September 14, 2020

For: 	Mary Landry

75487 Downs Ave

Covington, La. 70435

Ref:     23160 Lowe Davis Rd.

Covington La. 70435

Construction:

One-story, wood frame, wood veneer, wood flooring with a composition shingle roof on a CMU block & brick pier foundation.  Home is approximately 100 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to the home being 100 + years old.

Findings:

The home has some height variations throughout the flooring in the interior. It is important to note that all floor height conditions are within acceptable tolerances.

An overall visual inspection of the exterior foundation of the home was conducted and several items were noted under the house. There have been repairs in the past that included new CMU blocking on the interior piers of the foundation and the exterior brick piers have been reworked in the past but need attention.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The exterior brick pier foundation does not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Re-point and level exterior brick piers.

2)   Fill and re-grade around home to allow for proper drainage of rainwater away from the 		      foundation.

Sincerely,

Brian Mistich, P.E.