554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

April 6, 2017

For: 	Frederick Lawshe

2790 Rodeo Rd. Apt 318

Abbeville La. 70510

Ref:     605 Burnside Drive

Pearl River La.

Construction:

Two-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a conventional foundation, approximately 37 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection that noted a elevation deference in the 2nd floor master bedroom. The current owner stated that they have not done any work on the home since they have owned it and the master bedroom floor have always sloped.

Findings:

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up in the hallway at zero inches and measured into the master bedroom.

The area in question with the most notable slope is in the master bedroom area right in the door entrance to the master bedroom. It was noted to have a -1.6” difference in elevation. This master bedroom is supported with a beam under it in the living room. In the living room you can see the beam and it has a slight dip in it. The interior of the home in these locations wasn’t noted to have any cracks in the sheet rock walls or ceiling.

No cracks in the foundation were noted in the interior of the home due to the floor coverings. No cracks in the foundation were noted in the exterior of the home.

Recommendation:

There are no visible signs of distress in the wall, ceiling or wood flooring in the dining room that would indicate a continual sagging.  This is also true for the master bedroom access area.    The access area did not appear to be soft or unstable.  The owner indicated that the sagging area has existed in the house since they owned it.

It is recommended that if the supporting beam is uncovered, either from below the floor or above, that the beam be inspected.  It most likely that the beam warped during construction, and at that time, was never corrected.  Sag may exceed current new construction parameters, but is more an esthetic concern than an indication of beam malfunction.

It appears the structural integrity of this area of the home is currently sound.

Sincerely,

Brian Mistich, P.E.