554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

April 03, 2018

For: 	Collen C. Lester

76 Northam CT

Slidell, La. 70458

Ref:     682 Dale Drive

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a conventional foundation.  Home is approximately 47 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to perform a structural inspection of the referenced home due to a home inspection that noted cracks in the brick veneer.

Findings:

An overall visual inspection of the exterior of the home was conducted and few items were noted. There are a few of hairline cracks in the brick veneer and mortar on the right, rear and left side of the home. The crack in the rear behind the A/C does protrude into the foundation but appears to get smaller as it goes down. A couple of the corners of the foundation are missing and this is normal for when the form boards are removed during construction. The interior of the home has several hairline cracks in the sheetrock ceilings and walls noted in different rooms.

It was also noted that there are no gutters or down spouts on the home. The rear

yard appears to be level and does not allow rain water to flow to the street to keep the foundation dry.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

1) Add gutters and down spouts to carry the rain water to the street

2) The yard should be re-graded to allow for proper drainage of rainwater away from the foundation.

3)  Seal cracks in the brick veneer, mortar with a non-shrinking grout, to prevent insects from entering the home.

Sincerely,

Brian Mistich, P.E.