DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

STRUCTURAL INSPECTION

October 24, 2008

For:
 Donald L Murphy

             116 Charwood Drive

              Pearl River, La.70452

REF:     L.H. Jake Dunham,Jr.

             103 Barbary Drive

              Slidell, La.  

Construction:

Two stories raised Wood frame home with wood siding/fascia and a composition shingle roof.

Scope:

This inspection is limited to a visual inspection of the interior and exterior walls, ceilings, floors, roof, and foundation of the residence.  No inspection/test of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. Dunham contacted Dammon Engineering to perform a structural inspection on the referenced residential property for the purpose of selling it to the above-mentioned name. The home is approximately 23 years of age. This is a dome shaped home and has had several additions added onto it in the past years. 

Findings:

This inspection covered visible components (foundation. Wall and roof framing and their ability to support the residence.

Description





Findings:  

1.  Number of Levels




2

2.  Interior Gutted




No

3.  Debris Cleaned




Yes

4.  Floor Framing




Wood

5.  Roof Framing




Wood

6.  Wall Framing




Wood

7.  Rot or Termites Present * 



Not Found

8.  Is Structure Elevated




Yes

9.  Sign of Structural Distress or Damage

No

10. Failures or Deterioration of Foundation

No 

Analysis:

There is no insulation under the floors of the home. The attic is well insulated. The roof is approximately 5 years old and appears to be in good condition. It was also noted that some deck boards would need to be replaced in the future. The rear steps need to be secured to a footing for proper support. The second floor ceiling was also noted to have some tape seams coming apart. This is normal due to expansion and contraction of the home.

Conclusion:

With normal maintenance repairs to the home it is in good condition and is structurally sound.  

*Note:  No termite inspection was performed.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

