DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS




ENGINEERS


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


554 Old Spanish Trail.








     985-649-5832

Slidell, LA 70458







            FAX 985-641-5950

Structural Inspection

December 15, 2011

Michael Barbato

221 Nicklaus

Slidell, Louisiana 70458

For:
Construction: Two Story, Wood Frame with Brick/Stucco Veneer and Composition Shingle Roof on a Post Tension Foundation.

Scope:

This inspection is limited to a visual inspection of the interior and exterior walls, floors, roof, and foundation of the residence.  No inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Laura Ryals with Century 21 Realtors contacted Dammon Engineering to request a structural inspection of the residence on behalf of the owner.  In the appraiser’s report, cracks in the exterior stucco were noted and the appraiser requested an opinion on those cracks. 

Findings:

Hair line cracks were noted on the exterior of the residence.  The cracks run horizontally in the exterior stucco of the front and rear porches of the home.  An inspection of the interior sheet rock walls and ceiling was conducted and both were found to be in the excellent condition.  The exact cause of the cracks was undetermined, but likely was caused by normal expansion and contraction of the stucco.
First, it is significant to note that stucco veneer is not structural and that the cracks in the stucco veneer, while they are an indication that something has happened, do not necessarily represent a structural concern.

Secondly, houses and foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  The house continues to settle and usually by the tenth year this settlement is infinitesimal.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, tree roots, etc.

Conclusion:

It is my professional opinion that the residence is structurally sound and further settling and expansion/contraction should be negligible on existing components of the home.
The cracks in the exterior stucco should be repaired by a professional stucco installation technician.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.

