Structural Inspection

May 14, 2008

For:
Mrs. Trudy Cartledge

329 Yenni Drive

Kenner, Louisiana

Requested By:  Mr. Jason Migliorie

Phone:
985-205-0029

Fax:
504-468-6002

Structure:

Single story wood frame, brick veneer, composition roofing on a concrete foundation,

Scope:

This inspection is limited to a visual inspection of the interior and exterior walls, floors, roof, and foundation of the residence.  No inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect or engineering.

History:

Mr. Migliorie contacted Dammon Engineering on behalf of Mrs. Cartlege to conduct a structural analysis of the above listed residence for the purpose of obtaining building permits to add a second story to a part of the residence.

Findings:

We found a residence in excellent condition.  It is constructed on a foundation of wooden pilings with a concrete slab.  Although the numbers, capacity, and location of existing pilings is not known, if they were installed to meet the safety factor of 3 to 1 (as required by most codes) the addition of the load of the new second floor should not diminish the capacity enough to cause a problem, as the new area will only add a floor and ceiling, as a slab load will not be added, nor a roof, as the existing roof will be removed and the new added.

Description




Findings

1.  Number of levels




One

2.  Interior gutted




No (Residence is occupied)

3.  Debris removed




None to remove

4.  Floor framing




Not applicable

5.  Roof framing




Wood rafters good condition

6.  Wall framing




Wood 2x4 16” o.c.

7.  Signs of rot or termites



None seen*

8.  Signs of structural damage or distress

None, structure is in very good condition

9.  Structure elevated 




No, concrete slab

10.Failure or deterioration of foundation

None

*No termite inspection was performed

Analysis:

Although some settlement may occur, as long as the new loads are placed on load bearing walls as shown on the drawings, I believe this is a safe addition to the residence to the best of my knowledge and belief.  

Conclusion:

We recommend approval of this project.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

