DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS




ENGINEERS


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

Structural Inspection

August 6, 2010

Mr. Larry Holcomb

National Specialty Aggregate

4310 South 80th street

Mesa, AZ  85212-9639

Property: Pontchartrain Materials Corporation


       37210 Ben Thomas Road


       Slidell, Louisiana 70460

Construction:
The building is described as a single story, fully sprinkled pre-engineered metal building with CMU/metal siding and metal roof on a concrete foundation. 

Scope:

This inspection is limited to a visual inspection of the interior and exterior walls, floors, roof, and foundation of the structure.  No inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers (NABIE) Standards and Practices for Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. Larry Holcomb contacted Dammon Engineering to request a structural inspection of the building.  The structure is approximately 40 years of age and approximately 57,300 sq. ft. with 5,400 sq. ft. of office space.

Findings/Exterior-
The roof appears to have been coated in the past to preserve it.  The coating is now becoming compromised due to its age and natural wear.  There are several places that the coating has either cracked off or worn down allowing corrosion to start to take place on the panels (rust was only noted in a few locations).  A tar coating has been applied in a few areas that have apparently leaked in the past.
Several of the vent covers are gone and the screens are damaged.  The roof in the rear of the building has a piece of equipment next to it that has worn into the metal cap.

Roof drainage appears to be adequate.  There was no standing water visible on the roof at time of inspection, nor any visible indication of current roof drainage issues.  The gutters require extensive cleaning to fully assess their condition, but gutters and down spouts all show signs of wear and damage.
The exterior of the building has been painted in the past and is showing signs of wear.  The metal siding in some areas is damaged and/or rusting at the bottom edge.  Most of the metal awnings are damaged, apparently from multiple impacts from forklifts.  

Findings/Interior-

No inspection was made of the mechanical or electrical systems, but the building is currently occupied and the following was noted:

The air conditioning appeared to be functioning normally at the time of inspection.

The lighting and air conditioning is adequate for its current use.

The plumbing in the office portion of the facility (bathroom and kitchen) appeared to be functioning properly at the time of inspection.  All fixtures and faucets in this area were in operational condition and currently in use.  All cabinets and countertops in this area were in usable condition and currently in use.

The plumbing in the warehouse restrooms was not available for inspection, as it appeared that the restrooms had not been utilized for some time.

There are several hairline cracks apparent in the concrete foundation.  The hairline cracks are typical and are to be expected due to normal expansion and contraction.

Conclusion:

It is my professional opinion that the building is structurally sound.  Overall, this building is in good condition.  However, it does need general maintenance to extend the life of the building.  Please see the attached photos to better identify where maintenance and/or repairs are needed.

** No termite inspection was conducted. **
Sincerely,

Emmett G. (Pete) Dammon, P.E.

Attachments:
Disk Containing Photographs from Inspection of August 4, 2010



Floor Plan (in .pdf format)

