DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS




ENGINEERS


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

Structural Inspection

March 5, 2009

Melody Martindale

Abek Real Estate

820 Oak Harbor Blvd.

Slidell, La.

For:
3801 Charles Drive

                         Chalmette, La. 70043

Construction: Single Story, Wood Frame with brick veneer and Composition Shingle roof.

Scope:

This inspection is limited to a visual inspection of the interior and exterior walls, floors, roof, and foundation of the residence.  No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Ms. Martindale contacted Dammon Engineering to request a structural inspection of the residence for the purpose of selling. It was noted that the home was enlarged on the right side of the residence. The existing structure is approximately 35 years of age and age of the new addition is unknown. 

Findings:

The brick veneer was noted to have a crack in the rear of the residence running vertically where the new addition is located. It has since been patched. The living room trim appears to have cracked some indicating settling in that area also. The roof decking appears to have a slight lift but the rafters in the attic don’t appear to be separating from the framing.

Houses and foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  The house continues to settle and usually by the tenth year this settlement is infinitesimal.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, tree roots, improper drainage, etc.

Conclusion:

Further settling should be negligible.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

