DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS




ENGINEERS


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

Structural Inspection

March 11, 2010

For: Bryan Giacone

       424 Secluded Grove Loop

       Madisonville, LA 70447

Construction:

One story, wood frame with brick veneer/stucco and composition shingle roof

Scope:

This inspection is limited to a visual inspection of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. Giacone contacted Dammon Engineering to request a structural inspection of the above listed residence.  Mr. Giacone was concerned about the cracks in the garage foundation/bathroom tile/sheetrock and exterior cracks. 

Findings:

1. First, it is significant to note that brick veneer is not structural and that the cracks in the brick veneer, while they are an indication that something has happened, do not necessarily represent a structural concern. Secondly, houses and foundations all settle. Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  The house continues to settle and usually by the tenth year this settlement is infinitesimal. By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, tree roots, etc. There are some minor cracks in the brick façade on the right side of the house above the garage door and the front entrance stucco facade. No cracks were found in the concrete footing to support any theory of these cracks being caused by foundation movement.

2.   There is a hairline crack found in the ceramic tile in the master bathroom. There is probably a hairline crack in the concrete foundation below the tile, thus cracking the tile.  This is often found in tile floors, which is the reason for the precautionary notes on the foundation drawings.  Concrete invariably cracks (there is a book entitled “Concrete Cracks,” explaining this occurrence), due to weather or other problems at the time of pouring the slab. An electronic level was used to measure high and low spots in the slab.  The slab was found to vary only one quarter of one inch in elevation through out the home. These differences are well within those expected to be found on typical foundations. 
3.   Other cracks that were noted in the interior walls and ceiling located in the kitchen area appear to be due to expansion and contraction. 
Conclusion:

It is my professional opinion that the residence is structurally sound and further settling should be negligible.

The majority of settling usually occurs in the first year after construction and gradual settlement then follows for about ten years, after which the structure should be relatively stable.  Settling may also be the result of the local weather conditions causing expansion or shrinkage of the soil.  The brick veneer and other concrete products expand and contract, causing cracks that are not necessarily structural problems.

This residence is structurally stable and will probably not show any major settling in the future.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


