STRUCTURAL INSPECTION

Friday, June 29, 2007

Type:
Commercial

Re:
Post Katrina Structural Inspection

Structure:
Two story/wood frame/concrete foundation/vinyl siding/composition shingles

Owner:
ECO Builders

Property:
4790 Pontchartrain Blvd.

Slidell, LA 70460

History:

Mr. Elwin C. Ordoyne, III contacted Dammon Engineering to request an inspection of the above named property to verify that it is structurally sound.  It is required to have a Structural Certification prior to receiving a construction loan to repair the property.

Findings:

A visible inspection was conducted of the roof, framing, foundation and exterior siding.  No inspection of the mechanical equipment or plumbing was conducted.

Findings:

1. The roof is in good condition.

2. Exterior siding in good condition.

3. Interior has been gutted to four feet.

4. Wall studs and remaining sheetrock are in good condition.

5. Foundation shows no signs of damage, cracks, or settling.

6. The property sits level on the foundation.

7. Most debris has been removed.

8. Wooden deck and boat docks along the bayou are missing and will need to be replaced.

9. Bulkhead shows no signs of shifting or sinking.

Analysis:

Water entered the property approximately 2 ½ feet.  The property is now open to permit air to circulate freely and is free of mold.

Conclusion:

Hurricane Katrina did not seriously damage this property.  Any damage was cosmetic in nature.  Damaged sheetrock and insulation have been removed.  All interior and exterior doors have been removed.  When repairs to this property are completed this property should be then restored to its original condition.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

