DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

January 29, 2007
STRUCTURAL IINSPECTION
For:
Summer Himber

H 985-641-3396

C 985-788-3395
Location:
3046 N. Palm Dr.


Slidell, Louisiana  70458

Building Description:  wood frame, concrete foundation, composite shingle roof.
Scope: 

This inspection is limited to a visual inspection of the interior and exterior walls, ceilings, floors, roof and foundation as designed by the engineer. No inspection of the mechanical or electrical systems was performed.

The following findings were noted:

The exterior doors in the master bed room have not been adjusted.  Gaps exist under and around the doors.  These gaps will allow wind-blown water to enter the home.
The exterior doors in the living room may still be leaking water. Water stains exist in the ceiling of the first floor near the doors on the 2nd floor living room.

The vertical car rails of the elevator are still rusted and need to be refurbished and maintained.
Trim has been masked off for preparation of wall painting.
Sincerely,

Emmett G. (Pete) Dammon, P.E.

La. License 8796

