DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

November 14, 2006

STRUCTURAL INSPECTION
For:
Maurice Guillot Property Management

Location:
8 Wysteria Lane



Covington, La 

Building Description:  wood frame, conventional concrete foundation, pull tab shingle roof, brick veneer on the original construction and wood siding on the additional room. 

Scope: 

This inspection is limited to a visual inspection of the interior and exterior walls, ceilings, floors, roof and foundation of the addition to this residence.  No inspection of the mechanical or electrical systems was performed.

History:

Mr. Guillot contacted Dammon Engineering to perform a structural inspection on the referenced residential property.  Mr. Guillot stated that water did not enter the building during Hurricane Katrina.  The building is approximately thirty years of age.  A 300 sq ft. addition was added after the original construction.  

Findings:

This inspection covered visible components (foundation, wall and roof) and their ability to support the home.

1. There are two doors in the addition, one-exit door and a bathroom door.  Both doors are out of square. 

2. The concrete foundation of the addition is settling differentially from the older section, causing a separation between the addition wall and the original exterior wall.  There are visible separations between the exterior wood siding of the addition and the brick veneer of the original structure at the point of intersection.  These separations are on both sides of the addition. See Photos 1 through 8

3. There are cracked tiles in the bathroom of the addition along the original wall.  See Photo 9

4. The marble shower enclosure is separated in the corner nearest the original structure.

5. The painted gypsum walls above the marble shower enclosure in the corner nearest the original wall are separating at the taped joint. See Photo 11

6. The base moldings and painted gypsum walls in the corner near the rear door of the addition are separating from the original constructed wall. See Photo 10,12 and 13

7. As of the date of this inspection, no water leaks were visible in the ceiling.

Recommendation and Conclusion
The foundation of the addition to the residence is possibly not tied to the original slab and there has definitely been some subsidence.
It is recommended that piles be installed along the perimeter of the addition to jack the foundation back to its original level to support the weight of the addition and prevent further movement.  This should be accomplished as soon as possible to prevent further damage.  After the piles are completed the repairs to the interior of the home should be completed. 

Sincerely,

Emmett Dammon

La. License 8796

Attachment:
Photos 1-14
