DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS




ENGINEERS


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

Structural Inspection

June 29, 2009

For: 


Billy Miller, Mobile, Al. 




In care of Abek Real Estate, Burl Ruth

Property Inspected:
4632 Pontchartrain Drive




Slidell, La. 70458

Construction:

This site has two buildings located on it, one warehouse and one office warehouse. The warehouse is a metal building with metal siding and roof. The office warehouse is a metal building with metal roof /siding and stucco front.

Scope:

This inspection is limited to a visual inspection of the interior and exterior walls, floors, roof, and foundation of the building. No Inspection of the mechanical or plumbing systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys. This report is not an explanation of cause, effect, or engineering.

History:

Mr. Ruth contacted Dammon Engineering to request a structural inspection on behalf of Billy Miller for the above listed building. 

Findings:

The warehouse was noted to have a leaking roof in the middle of the bldg. and the roof insulation was falling down. It was also noted that the right front exterior and left side exterior has some siding/panels that are lightly damaged. The fascia on the rear of the bldg. was noted have torn loose. 

The exterior of the office warehouse stucco façade was noted to have a hairline crack in it. The fascia on the rear of the bldg. was noted have torn loose. Several locations the drop ceiling was noted to have water stains and in the meeting room one was torn in half with some a/c grills hanging. Also in the meeting room some outlet covers were missing. In the rear of warehouse, approximately 12” of the foundation ledge has chipped away. 

Conclusion:

It is my professional opinion that the bldg. is structurally sound and the items listed above do not pose a structural problem. 

The majority of settling usually occurs in the first year after construction and gradual settlement then follows for about ten years, after which the structure should be relatively stable.  Settling may also be the result of the local weather conditions causing expansion or shrinkage of the soil.  The brick veneer and other concrete products expand and contract, causing cracks that are not necessarily structural problems.

This residence is structurally stable and will probably not show any major settling in the future.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

