Specsheet
Revolving Tray Ovens:     115V/ 1ph/ 12 Amps     or     120V/ 1ph/ 22.5 Amps
80 quart Mixers:     200-240V/ 3ph/ 11 Amps
or    380-460V/ 3ph/ 5.2 Amps

60 quart Mixers:     230V/ 1ph/ 9.5 Amps

Pan Washer:     240V/ 1ph/ 29 Amps     or     208V/ 3ph/ 19 Amps, 240V/ 3ph/ 18 Amps
Gas Consumption- 90,000 BTUs per hour
Supply Pressure- Minimum 7” w.c. for natural, 11” w.c. for propane and 14” w.c. maximum

Dough Divider:     208-240V/ 3ph/ 20 Amps

Doughnut Fryer:     120V/ 1ph/.5 Amps      or     240V/ 1ph/ .25 Amps

Gas Consumption- 44,000-60,000 BTU per hour
Gas Connection- Natural Gas 5”-14” w.c., Propane 11”-14” w.c., Connector- ½” NPT

Doughnut Proofer:    208V/ 1ph/ 11.1 Amps

3 Door Reach in Refrigerator:     115V/ 1ph/ 15 Amps
3 Door Reach in Freezer:    208-230V/ 115V/ 1ph/ 11 Amps

Walk in Refrigerator:     115V/ 1ph/ 18.7 Amps
Walk in Freezer:     115V/ 1ph/ 16.2 Amps
Ice Maker:     208-230V/  1ph/  17.7 Amps
