SECTION 03100PRIVATE 

CONCRETE FORMWORK

Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.
1      GENERAL


1.01
SECTION INCLUDES



A.
Formwork for cast‑in place concrete, with shoring, bracing and anchorage.



B.
Openings for other work.



C.
Form accessories.



D.
Form stripping.


1.02
PRODUCTS INSTALLED BUT NOT FURNISHED UNDER THIS SECTION

A. Section 03300 ‑ Cast‑in‑Place Concrete:  Supply of 

Concrete accessories for placement by this section.


1.03
RELATED SECTIONS



A.
Section 03200 ‑ Concrete Reinforcement.

B. Section 03300 – Cast-in-Place Concrete.



C.
Reference to General Conditions. 


1.04
REFERENCES



A.
ACI 347 ‑ Recommended Practice For Concrete Formwork.



1.05
DESIGN REQUIREMENTS



A.
Design, engineer and construct formwork, shoring and bracing to conform to design and code requirements; resultant concrete to conform to required shape, line and dimension.


1.06
DELIVERY, STORAGE, AND HANDLING



A.
Store off ground in ventilated and protected manner to prevent deterioration from moisture.

1.07
COORDINATION



A.
Coordinate this Section with other Sections of work which require attachment of components to formwork.



B.
If formwork placed after reinforcement results in insufficient concrete cover over reinforcement, request instructions from Architect/Engineer before proceeding.

2      PRODUCTS


2.01
WOOD FORM MATERIALS



A.
Form Materials:  At the discretion of the Contractor.


2.02
FORMWORK ACCESSORIES



A.
Form Release Agent:  Colorless mineral oil which will not stain concrete, absorb moisture, or impair natural bonding or color characteristics of coating intended for use on concrete.


3      EXECUTION


3.01
EXAMINATION



A.
Verify lines, levels and centers before proceeding with formwork.  Ensure that dimensions agree with Drawings.


3.02
EARTH FORMS



A.
Hand trim sides and bottom of earth forms.  Remove loose soil prior to placing concrete.


3.03
ERECTION ‑ FORMWORK



A.
Erect formwork, shoring and bracing to achieve design requirements, in accordance with requirements of ACI 301.



B.
Provide bracing to ensure stability of formwork.
Shore or strengthen formwork subject to overstressing by construction loads.


C.
Arrange and assemble formwork to permit dismantling and stripping.  Do not damage concrete during stripping. Permit removal of remaining principal shores.



D.
Align joints and make watertight.  Keep form joints to a minimum.



E.
Obtain approval before framing openings in structural members which are not indicated on Drawings.


3.04
APPLICATION ‑ FORM RELEASE AGENT



A.
Apply form release agent on formwork in accordance with manufacturer's recommendations.



B.
Apply prior to placement of reinforcing steel, anchoring devices, and embedded items.



C.
Do not apply form release agent where concrete surfaces will receive special finishes or applied coverings which are affected by agent.  Soak inside surfaces of untreated forms with clean water.  Keep surfaces coated prior to placement of concrete.


3.05
INSERTS, EMBEDDED PARTS, AND OPENINGS



A.
Provide formed openings where required for items to be embedded in or passing through concrete work.



B.
Locate and set in place items which will be cast directly into concrete.



C.
Coordinate work of other Sections in forming and placing openings, slots, reglets, recesses, chases, sleeves, bolts, anchors, and other inserts.



D.
Install accessories in accordance with manufacturer's instructions, straight, level, and plumb.  Ensure items are not disturbed during concrete placement.


3.06
FORM CLEANING



A.
Clean and remove foreign matter within forms as erection proceeds.



B.
Clean formed cavities of debris prior to placing concrete.



C.
Flush with water or use compressed air to remove remaining foreign matter.  Ensure that water and debris drain to exterior through clean‑out ports.


3.07
FORMWORK TOLERANCES



A.
Construct formwork to maintain tolerances required by ACI 301.


3.08
FIELD QUALITY CONTROL



A.
Inspect erected formwork, shoring, and bracing to ensure that work is in accordance with formwork design, and that supports, fastenings, wedges, ties, and items are secure.

C. Do not reuse wood formwork more than two times for concrete surfaces to be exposed to view.  Do not patch formwork.


3.09
FORM REMOVAL



A.
Do not remove forms or bracing until concrete has gained sufficient strength to carry its own weight and imposed loads.



B.
Loosen forms carefully.  Do not wedge pry bars, hammers, or tools against finish concrete surfaces scheduled for exposure to view.



C.
Store removed forms in manner that surfaces to be in contact with fresh concrete will not be damaged.  Discard damaged forms.

END OF SECTION

1
Error! Main Document Only.
Section 03100 Page 1 of 3

