Louisiana Army National Guard

Preliminary Review Documents

141st Readiness Center (RC)

Jackson Barracks, New Orleans, Louisiana

Technical Specifications


Copyright 2004 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 08345 - VAULT DOORS 
Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project.

1. Armory door for vault.

1.2 SUBMITTALS

A. Product Data:  For each type of product indicated.

B. Shop Drawings:  Include plans, elevations, sections, details, and attachments to other work.

First paragraph below assumes manufacturer's standard-size Samples are acceptable.  Revise to suit Project.

C. Samples:  For each exposed finish.

D. Product test reports.

E. Operation and maintenance data.

1.3 QUALITY ASSURANCE

A. Installer Qualifications:  An authorized representative of fire-rated security door manufacturer for installation and maintenance of units required for this Project.

B. Comply with vault door requirements in Federal Specification AA-D-600D Door, Vault, Security.
Retaining paragraph above requires listed or labeled doors with a fire-resistance rating equivalent to adjacent fire-resistance rating of vault or file-room walls, based on testing per UL 155.  UL 155 in first paragraph below evaluates doors for Class 350 temperature rating, which is only used for protecting paper records.  See Evaluations.

C. Preinstallation Conference:  Conduct conference at Project site.

PART 2 -  PRODUCTS

2.1 VAULT DOOR
Vault doors are available with a 2-, 4-, or 6-hour rating; file-room doors are available with a 1/2- or 1-hour rating.

A. Provide GSA-Approved and labeled Class 5-A Armory Door that conforms to Federal Specification AA-D-00600.  

1. Type IL or IR (left or right opwning swing with optical device.

2. Style K- Key change combination lock

3. Design S- single lock.

4. Size:  Clear opening 40-inches (1016 mm) wide by 78-inches (1981 mm) high.  
B. Finish:  Manufacturer's standard factory-applied paint finish, applied to door, frame, and wall flanges.

C. Escape Mechanism:  Provide emergency operation of lock from vault side of door by means of inside door release fabricated from stainless-steel rod.

2.2 FABRICATION

A. Fabricate vault doors and frames rigid, neat in appearance, and free of defects, warp, or buckle.  Accurately form metal to required sizes and profiles, with minimum radius for thickness of metal.  Weld exposed joints continuously; grind, fill, dress, and make smooth, flush, and invisible.  Where practical, fit and assemble units in manufacturer's plant.  To ensure proper assembly at Project site, clearly identify work that cannot be permanently factory assembled before shipment.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Install vault door assemblies complete with doors, frames, and accessories and according to requirements of F.S. AA-D-600D.

B. Set vault door frames accurately in position; plumbed, aligned, and braced securely until permanent anchors are set.  After wall construction is complete, remove temporary braces and spreaders, leaving surfaces smooth and undamaged.

C. Adjust door hardware and operating mechanism for proper operation.

END OF SECTION 08345


VAULT DOORS 

08345 - 1

