Louisiana Army National Guard

Preliminary Review Documents

141st Readiness Center (RC)

Jackson Barracks, New Orleans, Louisiana

Technical Specifications


Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 09680 - CARPET
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. This Section includes tufted carpet.

1.2 SUBMITTALS

A. Product Data:  For each product indicated.

B. Shop Drawings:  Show the following:

1. Carpet type, color, and dye lot.

2. Seam locations.

3. Pattern type, repeat size, location, direction, and starting point.

4. Pile direction.

5. Insets and borders.

6. Edge, transition, and other accessory strips.

7. Transition details to other flooring materials.

C. Samples:  For each color and texture required.

1. Carpet:  12-inch- (300-mm-) square Sample.

2. Exposed Edge, Transition, and other Accessory Stripping:  12-inch- (300-mm-) long Samples.

D. SPiRiT Submittals:

Retain subparagraph below if low-emitting materials are required for LEED Credit EQ 4.3; coordinate with requirements selected in Part 2 for carpet, carpet cushion, and adhesives.

1. Credit IEQ 5.C4:  Manufacturers' product data for  carpet and installation adhesive, including printed statement of VOC content.

E. Product Schedule:  For carpet.

F. Maintenance data.

1.3 QUALITY ASSURANCE

A. Installer Qualifications:  An experienced installer who is certified by the Floor Covering Installation Board or who can demonstrate compliance with its certification program requirements.

B. Mockups:  Before installing carpet, build mockups to verify selections made under sample submittals and to demonstrate aesthetic effects and set quality standards for materials and execution.

Retain subparagraph below if mockups are erected as part of building rather than separately and the intention is to make an exception to the default requirement in Division 01 Section "Quality Requirements" for demolishing and removing mockups when directed, unless otherwise indicated.

1. Approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

1.4 DELIVERY, STORAGE, AND HANDLING

A. Comply with CRI 104, Section 5, "Storage and Handling."

1.5 PROJECT CONDITIONS

A. Comply with CRI 104, Section 7.2, "Site Conditions; Temperature and Humidity" and Section 7.12, "Ventilation."

1.6 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 01 Section "Product Requirements."

Revise paragraph below if manufacturers have separate warranties for various characteristics.

A. Special Warranty for Carpet:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet installation that fail in materials or workmanship within specified warranty period.  Failures include, but are not limited to, more than 10 percent loss of face fiber, edge raveling, snags, runs, loss of tuft bind strength, excess static discharge, and delamination.

Verify available warranties and insert number in subparagraph below.

1. Warranty Period:  10 years from date of Substantial Completion.

1.7 EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

A. Furnish extra materials described below, before installation begins, that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Carpet:  Full-width rolls equal to 5 percent of amount installed for each type indicated, but not less than 10 sq. yd. (8.3 sq. m).

PART 2 -  PRODUCTS

Copy applicable articles and re-edit for each product required.

2.1 TUFTED CARPET 

Delete remainder of this Article if products are limited to those listed above.

A. Basis-of-Design Product:  Subject to compliance with requirements, provide Mannington Commercial, Inc. – “Balanced” broadloom carpet or a comparable product.

1. Fiber Type:  Type 6.6 nylon.

2. Dye Method:  Solution dyed.

3. Pile Characteristic:  Enhanced graphics loop pile.

4. Average Density:  5806 oz./cu. yd. (0.215 g/cu. cm.).
5. Weight Density:  116,129 oz./cu. yd. (4.31g/cu. cm.).
6. Pile Thickness:  0.124 inches (3.15 mm) for finished carpet tile.

7. Stitches:  13.5 stitches per inch (0.53 stiches per mm).
8. Gage:  10 ends per inch (0.39 ends per mm).
9. Surface Pile Weight:  20 oz./sq. yd. (678 g/sq. m).
10. Primary Backing:  100% woven synthetic.

11. Secondary Backing:  Ultra Bac Plus
12. Width:  12 feet (3.7 m) .

B. Applied Soil-Resistance Treatment:  Manufacturer's standard material.

C. Antimicrobial Treatment:  Manufacturer's standard material.

D. Performance Characteristics:  As follows:

1. Methenamine Pill Test (ASTM-D-2859):  Passes

2. Flooring Radiant Panel Test (ASTM-E-648):  Class 1 (Direct Glue)

3. N.B.S. Smoke Chamber Test (ASTM-E-662):
Less than 450 (Flamming Mode)

4. Electrostatic Propensity:  Less than 3.0 kV per AATCC 134.

Retain subparagraph and associated subparagraphs below if low-emitting materials are required for LEED Credit EQ 4.3.  VOC limits below are based on CRI's Green Label Indoor Air Quality Testing Program.

5. VOC Limits:  Provide carpet that complies with the following limits for VOC content when tested according to ASTM D 5116:

a. Total VOCs:  0.5 mg/sq. m x h.

b. 4-PC (4-Phenylcyclohexene):  0.05 mg/sq. m x h.

c. Formaldehyde:  0.05 mg/sq. m x h.

d. Styrene:  0.4 mg/sq. m x h.

2.2 INSTALLATION ACCESSORIES

A. Trowelable Leveling and Patching Compounds:  Latex-modified, hydraulic-cement-based formulation provided or recommended by carpet manufacturer.

B. Adhesives:  Water-resistant, mildew-resistant, nonstaining type to suit products and subfloor conditions indicated, that complies with flammability requirements for installed carpet and is recommended or provided by carpet manufacturer.

Retain subparagraph and associated subparagraphs below if low-emitting materials are required for LEED Credit EQ 4.3.  VOC limits below are based on CRI's Green Label Indoor Air Quality Testing Program.

1. VOC Limits:  Provide adhesives that comply with the following limits for VOC content when tested according to ASTM D 5116:

a. Total VOCs:  10.00 mg/sq. m x h.

b. Formaldehyde:  0.05 mg/sq. m x h.

c. 2-Ethyl-1-Hexanol:  3.00 mg/sq. m x h.

C. Seam Adhesive:  Hot-melt adhesive tape or similar product recommended by carpet manufacturer for sealing and taping seams and butting cut edges at backing to form secure seams and to prevent pile loss at seams.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Comply with CRI 104 and carpet manufacturer's  written installation instructions for the following:

1. Direct-Glue-Down Installation:  Comply with CRI 104, Section 9, "Direct Glue-Down Installation."

B. Comply with carpet manufacturer's written recommendations and Shop Drawings for seam locations and direction of carpet; maintain uniformity of carpet direction and lay of pile.  At doorways, center seams under the door in closed position.

C. Extend carpet into toe spaces, door reveals, closets, open-bottomed obstructions, removable flanges, alcoves, and similar openings.

D. Install pattern parallel to walls and borders.

END OF SECTION 09680

CARPET

09680 - 3

