Copyright 2003 and 2005 AIA
MASTERSPEC Full Length
05/03 (R 05/06)

Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 064013 - EXTERIOR ARCHITECTURAL WOODWORK
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project and to distinguish exterior architectural woodwork from items made from wood but specified as finish carpentry, etc., in other Sections.

1. Exterior standing and running trim.

2. Exterior frames and jambs.

3. Exterior shutters.

4. Exterior ornamental work.

5. Shop priming exterior woodwork.

6. Shop finishing exterior woodwork.

B. Related Sections include the following:

List below only products, construction, and equipment that the reader might expect to find in this Section but are specified elsewhere.

Delete subparagraph below if no exposed framing.

1. Division 06 Section "Rough Carpentry" for exposed framing.

Revise subparagraph above or below to include wood decking and stairs, railings, seating, etc., included with decks if any.  Subparagraph below includes wood siding.

2. Division 06 Section "Finish Carpentry" for exterior carpentry exposed to view that is not specified in this Section.

1.3 SUBMITTALS

Delete first paragraph and subparagraphs below if not applicable, or supplement by inserting product list.

A. Product Data:  For each type of product and process indicated and incorporated into items of exterior architectural woodwork during fabrication, finishing, and installation.

1. Include data for wood-preservative treatment from chemical treatment manufacturer and certification by treating plant that treated materials comply with requirements.

Delete subparagraph below if fire-retardant-treated wood is not required.

2. Include data for fire-retardant treatment from chemical treatment manufacturer and certification by treating plant that treated materials comply with requirements.

B. Shop Drawings:  Show location of each item, dimensioned plans and elevations, large-scale details, attachment devices, and other components.

Retain first subparagraph below only for ornate, complex, or very high quality work.

1. Show details full size.

2. Show locations and sizes of blocking and nailers, including concealed blocking and reinforcement specified in other Sections.

Retain subparagraph below only if other requirements for WI's quality certification program are retained.

3. Apply WI-certified compliance label to first page of Shop Drawings.

C. Samples for Verification:

Delete items in subparagraphs below that are not actually needed for control of grain character, color, and finish or expand to include other items.  Sizes comply with those in AWI standard; revise if other sizes are required.

1. Lumber for exterior wood stain finish, not less than 50 sq. in. (300 sq. cm), for each species, with 1/2 of exposed surface finished.

2. Lumber for transparent finish, not less than 50 sq. in. (300 sq. cm), for each species, with 1/2 of exposed surface finished.

3. Lumber and panel products for shop-applied opaque finish, 8 by 10 inches (200 by 250 mm) for panels and 50 sq. in. (300 sq. cm) for lumber, for each finish system and color, with 1/2 of exposed surface finished.

Retain first paragraph and subparagraphs below and "Forest Certification" Paragraph in "Quality Assurance" Article if wood products are required to be certified for LEED-NC or LEED-CI Credit MR 7, which requires that a minimum of 50 percent of wood-based materials be certified.

D. LEED Submittal:

1. Certificates for Credit MR 7:  Chain-of-custody certificates certifying that products specified to be made from certified wood comply with forest certification requirements.  Include evidence that mill is certified for chain of custody by an FSC-accredited certification body.

a. Include statement indicating costs for each certified wood product.

E. Product Certificates:  For each type of product, signed by product manufacturer.

If retaining paragraph below, also retain paragraphs in "Quality Assurance" Article that require compliance with selected quality standard and certification program.

F. Woodwork Quality Standard Compliance Certificates:  [AWI Quality Certification Program certificates] [WI-certified compliance certificates].

Coordinate paragraph below with qualification requirements in Division 01 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

G. Qualification Data:  For [Installer] [fabricator].

1.4 QUALITY ASSURANCE

Consider retaining one of two options in paragraph below if AWI quality certification or WI compliance certification is required.  AWI's quality certification is only available if fabricator is a certified participant.  WI will inspect work and provide certification for work that passes inspection if fabricator is not a licensed shop.

A. Fabricator Qualifications:  Shop that employs skilled workers who custom-fabricate products similar to those required for this Project and whose products have a record of successful in-service performance.[  Shop is a certified participant in AWI's Quality Certification Program.] [  Shop is a licensee of WI's Certified Compliance Program.]

Delete first paragraph below if not required or available.  If AWI Quality Certification Program labels or certificates are required for installation, installer must be a certified participant.

B. Installer Qualifications:  [Fabricator of products] [Certified participant in AWI's Quality Certification Program] [Licensee of WI's Certified Compliance Program].

C. Quality Standard:  Unless otherwise indicated, comply with AWI's "Architectural Woodwork Quality Standards" for grades of exterior architectural woodwork indicated for construction, finishes, installation, and other requirements.

1. Provide AWI Quality Certification Program [labels] [and] [certificates] indicating that woodwork[, including installation,] complies with requirements of grades specified.

Retain paragraph above or below.  Verify applicability of selected standard for Project's geographic location.  Review selected standard and coordinate its requirements with selected options.  Certification programs of both organizations are available to member and nonmember firms.  AWI's quality certification is only available if fabricator is a certified participant.  WI will inspect work and provide certification for work that passes inspection if fabricator is not a licensed shop.  Contact either organization for a list of certified woodworkers in Project's location.  For projects that include only items that are completely fabricated in a woodworking shop and installed without requiring assembly, consider deleting certification for installation, especially if installation may not be performed by woodworking firm that fabricated the items.

D. Quality Standard:  Unless otherwise indicated, comply with WI's "Manual of Millwork" for grades of exterior architectural woodwork indicated for construction, finishes, installation, and other requirements.

1. Provide WI-certified compliance [labels] [and] [certificates] indicating that woodwork[, including installation,] complies with requirements of grades specified.

Delete paragraph below if no fire-retardant-treated materials or products are required.

E. Fire-Test-Response Characteristics:  Where fire-retardant materials or products are indicated, provide materials and products with specified fire-test-response characteristics as determined by testing identical products per test method indicated by UL, ITS, or another testing and inspecting agency acceptable to authorities having jurisdiction.  Identify with appropriate markings of applicable testing and inspecting agency in the form of separable paper label or, where required by authorities having jurisdiction, imprint on surfaces of materials that will be concealed from view after installation.

Paragraph below may be retained to specify exterior architectural woodwork made from certified wood for LEED-NC or LEED-CI Credit MR 7, which requires that a minimum of 50 percent of wood-based materials be certified.  An alternative method of complying with Credit MR 7 requirement is to retain requirement in Division 01 Section "Sustainable Design Requirements"  that gives Contractor the option and responsibility for determining how Credit MR 7 requirement will be met.

F. Forest Certification:  Provide exterior architectural woodwork produced from wood obtained from forests certified by an FSC-accredited certification body to comply with FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."

Add requirements for mockups and preinstallation conference if desired; copy from Division 06 Section "Interior Architectural Woodwork."

1.5 PROJECT CONDITIONS

Requirements in first paragraph below may not be realistic.  Consider deleting and including requirement in Part 2 for shop priming to protect woodwork until it can be finished.

A. Weather Limitations:  Proceed with installation of exterior woodwork only when existing and forecasted weather conditions permit work to be performed and at least one coat of specified finish to be applied without exposure to rain, snow, or dampness.

B. Field Measurements:  Where woodwork is indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication and indicate measurements on Shop Drawings.  Coordinate fabrication schedule with construction progress to avoid delaying the Work.

1. Locate concealed framing, blocking, and reinforcements that support woodwork by field measurements before being enclosed and indicate measurements on Shop Drawings.

Delete subparagraph below if not allowed.

2. Established Dimensions:  Where field measurements cannot be made without delaying the Work, establish dimensions and proceed with fabricating woodwork without field measurements.  Provide allowance for trimming at site, and coordinate construction to ensure that actual dimensions correspond to established dimensions.

1.6 COORDINATION

A. Coordinate sizes and locations of framing, blocking, reinforcements, and other related units of Work specified in other Sections to ensure that exterior architectural woodwork can be supported and installed as indicated.

PART 2 -  PRODUCTS

2.1 WOODWORK FABRICATORS

Retain this Article if list of preapproved woodworkers is used as quality-control procedure.  See Evaluations.

A. Available Fabricators:  Subject to compliance with requirements, fabricators offering exterior architectural woodwork that may be incorporated into the Work include, but are not limited to, the following:

Retain paragraph above if additional woodworking firms will be considered; retain paragraph below if not.  If retaining above, include procedure for approving other firms in the Instructions to Bidders.

B. Fabricators:  Subject to compliance with requirements, provide exterior architectural woodwork by one of the following:

Insert list of preapproved firms.  Refer to AWI's or WI's member list for names of woodworking firms that could potentially be included.

2.2 MATERIALS

A. General:  Provide materials that comply with requirements of [AWI's] [WI's] quality standard for each type of woodwork and quality grade specified, unless otherwise indicated.

B. Wood Products:  Comply with the following:

Retain subparagraphs below if AWI standard is used or if requirements for wood products selected exceed those in WI standard.  Delete standards for products not used.

1. Hardboard:  AHA A135.4.

2. Softwood Plywood:  DOC PS 1, [Exterior] [Medium Density Overlay].

2.3 WOOD-PRESERVATIVE-TREATED MATERIALS

Delete this Article if not applicable.  See Evaluations for discussion of formulations and requirements of woodworking standards for preservative treatment.

A. Preservative Treatment by Nonpressure Process:  Comply with AWPA N1 using the following preservative for woodwork items indicated to receive water-repellent preservative treatment:

Substitute other formulations for those listed in two subparagraphs below only after verifying availability and compliance with environmental and regulatory controls in effect at Project location.

1. Water-Repellent Preservative:  Formulation made specifically for dip treatment of woodwork items and containing 3-iodo-2-propynyl butyl carbamate (IPBC) complying with AWPA P8 as its active ingredient.

Retain subparagraph above for aboveground decay resistance or subparagraph below where resistance to wood-destroying insects is also required.

2. Water-Repellent Preservative/Insecticide:  Formulation made specifically for dip treatment of woodwork items and containing 3-iodo-2-propynyl butyl carbamate (IPBC) as its active ingredient, combined with an insecticide containing chlorpyrifos as its active ingredient, both complying with AWPA P8.

B. Preservative Treatment by Pressure Process:  AWPA C2 (lumber) and AWPA C9 (plywood) and the following:

Treated lumber that does not contain arsenic and chromium is more expensive than treated lumber that does.  Treatments using arsenic are being phased out for certain uses.  See Evaluations.

1. Preservative Chemicals:  Acceptable to authorities having jurisdiction[ and containing no arsenic or chromium].  Use chemical formulations that do not bleed through or otherwise adversely affect finishes.  Do not use colorants to distinguish treated materials from untreated materials.

2. Kiln-dry lumber and plywood after treatment to a maximum moisture content, respectively, of 19 and 15 percent.  Do not use materials that are warped or do not comply with requirements for untreated materials.

3. Mark each treated item with treatment quality mark of an inspection agency approved by the American Lumber Standards Committee Board of Review.

Delete paragraph below if not all exterior architectural woodwork is preservative treated and Drawings clearly indicate where treatment is required.  Revise to suit Project.

C. Extent of Treatment:  Treat blocking and nailers by pressure process and treat other exterior architectural woodwork either by pressure or nonpressure process.

Delete subparagraph and associated subparagraphs below if not applicable.

1. Items fabricated from the following wood species need not be treated:

a. [Redwood] [All-heart redwood].

b. [Western red cedar] [All-heart western red cedar].

c. Teak.

d. African mahogany.

2.4 FIRE-RETARDANT-TREATED MATERIALS

Delete this Article if fire-retardant-treated materials are not required.  Woodwork cannot be both pressure-preservative treated and fire-retardant treated.

A. General:  Where fire-retardant-treated materials are indicated, provide materials that comply with performance requirements of AWPA C20 (lumber) and AWPA C27 (plywood), exterior type.

1. Fire-Retardant Chemicals:  Use chemical formulations that do not bleed through or otherwise adversely affect finishes.  Do not use colorants to distinguish treated materials from untreated materials.

2. Mill lumber before treatment and implement special procedures during treatment and drying processes that prevent lumber from warping and developing discolorations from drying sticks or other causes, marring, and other defects affecting appearance of treated woodwork.

3. Kiln-dry materials before and after treatment to levels required for untreated materials.

4. Do not use treated materials that do not comply with requirements of referenced woodworking standard or that are warped, discolored, or otherwise defective.

5. Identify fire-retardant-treated materials with appropriate classification marking of UL, U.S. Testing, Timber Products Inspection, or another testing and inspecting agency acceptable to authorities having jurisdiction.

2.5 INSTALLATION MATERIALS

Delete paragraph below if included in a carpentry section.  If retaining, select one of two options or delete both if not required.  Adjust moisture content to suit regional requirements and local availability.

A. Blocking, Shims, and Nailers:  Softwood or hardwood lumber[, pressure-preservative treated] [, fire-retardant treated], kiln dried to less than 15 percent moisture content.

Retain one or more nail types in first paragraph below.  All three are recommended by the California Redwood Association for attaching redwood that is subject to moisture.

B. Nails:  [Aluminum] [hot-dip galvanized] [or] [stainless steel].

C. Screws:  [Aluminum] [bronze] [hot-dip galvanized] [or] [stainless steel].

Delete subparagraph below if not applicable.

1. Provide self-drilling screws for metal framing supports, as recommended by metal-framing manufacturer.

D. Anchors:  Select material, type, size, and finish required for each substrate for secure anchorage.  Provide nonferrous-metal or hot-dip galvanized anchors and inserts, unless otherwise indicated.  Provide toothed-steel or lead expansion sleeves for drilled-in-place anchors.

2.6 FABRICATION, GENERAL

Select one of three ranges of moisture content in first paragraph below.  First range applies to Ontario, Quebec, and most of U.S.  Second applies to damp coastal areas of southern U.S. and to Alberta, Manitoba, Saskatchewan, and the Maritime Provinces.  Third applies to dry southwestern U.S.  See relative humidity and moisture content map in AWI quality standards.

A. Wood Moisture Content:  [9 to 15] [10 to 15] [7 to 12] percent.

B. Fabricate woodwork to dimensions, profiles, and details indicated.  Ease edges to radius indicated for the following:

1. Edges of Solid-Wood (Lumber) Members 3/4 Inch (19 mm) Thick or Less:  1/16 inch (1.6 mm).

2. Edges of Rails and Similar Members More Than 3/4 Inch (19 mm) Thick:  1/8 inch (3 mm).

If applicable, insert subparagraph to describe conditions where eased edges are not required.

Plant assembly of casings (standing trim) is limited to Premium and Custom grades.

C. Complete fabrication, including assembly, finishing, and hardware application, to maximum extent possible before shipment to Project site.  Disassemble components only as necessary for shipment and installation.  Where necessary for fitting at site, provide ample allowance for scribing, trimming, and fitting.

D. Shop cut openings, to maximum extent possible, to receive hardware, electrical work, and similar items.  Locate openings accurately and use templates or roughing-in diagrams to produce accurately sized and shaped openings.  Smooth edges of cutouts and seal with a water-resistant coating suitable for exterior applications.

2.7 EXTERIOR STANDING AND RUNNING TRIM FOR TRANSPARENT FINISH

A. Grade:  [Premium] [Custom] [Economy].

B. Backout or groove backs of flat trim members and kerf backs of other wide, flat members, except for members with ends exposed in finished work.

Retain paragraph below or revise if not applicable to all casings.  Plant assembly of casings applies only to Premium and Custom grades and is not required by AWI standards unless so specified.

C. Assemble casings in plant except where shipping limitations require field assembly.

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

D. Wood Species:  [All-heart redwood] [Western red cedar] [Eastern white pine] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.8 EXTERIOR STANDING AND RUNNING TRIM FOR OPAQUE FINISH

A. Grade:  [Premium] [Custom] [Economy].

B. Backout or groove backs of flat trim members, and kerf backs of other wide, flat members, except for members with ends exposed in finished work.

Retain paragraph below or revise if not applicable to all casings.  Plant assembly of casings applies only to Premium and Custom grades and is not required by AWI standards unless so specified.

C. Assemble casings in plant except where shipping limitations require field assembly.

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

D. Wood Species:  [All-heart redwood] [Western red cedar] [Eastern white pine, sugar pine, or western white pine] [Any closed-grain hardwood] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.9 EXTERIOR FRAMES AND JAMBS FOR TRANSPARENT FINISH

A. Grade:  [Premium] [Custom] [Economy].

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

B. Wood Species:  [Teak] [All-heart redwood] [Western red cedar] [White oak] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.10 EXTERIOR FRAMES AND JAMBS FOR OPAQUE FINISH

A. Grade:  [Premium] [Custom] [Economy].

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

B. Wood Species:  [All-heart redwood] [Western red cedar] [Ponderosa pine] [Eastern white pine, sugar pine, or western white pine] [Any closed-grain hardwood] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.11 EXTERIOR SHUTTERS FOR TRANSPARENT FINISH

A. Grade:  [Premium] [Custom] [Economy].

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

B. Wood Species:  [Teak] [African mahogany] [All-heart redwood] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.12 EXTERIOR SHUTTERS FOR OPAQUE FINISH

A. Grade:  [Premium] [Custom] [Economy].

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

B. Wood Species:  [All-heart redwood] [Ponderosa pine] [Eastern white pine, sugar pine, or western white pine] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.13 EXTERIOR ORNAMENTAL WORK FOR TRANSPARENT FINISH

AWI uses the term "exterior ornamental work" to refer to miscellaneous items of woodwork not included in other categories in its standard.  WI includes these items with the term "millwork" in the category "Exterior - Trim, Frames & Millwork."

A. Exterior ornamental work for transparent finish includes the following:

Edit list below to suit Project.

1. Balustrades.

2. Columns.

3. Cupolas.

4. Pediment heads.

5. Pilasters.

6. <Insert description.>

B. Grade:  [Premium] [Custom] [Economy].

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

C. Wood Species:  [Teak] [African mahogany] [All-heart redwood] [Western red cedar] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.14 EXTERIOR ORNAMENTAL WORK FOR OPAQUE FINISH

AWI uses the term "exterior ornamental work" to refer to miscellaneous items of woodwork not included in other categories in its standard.  WI includes these items with the term "millwork" in the category "Exterior - Trim, Frames & Millwork."

A. Exterior ornamental work for opaque finish includes the following:

Edit list below to suit Project.

1. Balustrades.

2. Columns.

3. Cupolas.

4. Pediment heads.

5. Pilasters.

6. <Insert description.>

B. Grade:  [Premium] [Custom] [Economy].

Options in paragraph below are examples only.  Revise to suit Project.  See Evaluations.

C. Wood Species:  [All-heart redwood] [Western red cedar] [Ponderosa pine] [Eastern white pine, sugar pine, or western white pine] [Any closed-grain hardwood] [Softwood plywood] <Insert species>.

Usually retain subparagraph below if selecting a softwood species above.

1. Do not use plain-sawn lumber with exposed, flat surfaces more than 3 inches (75 mm) wide.

2.15 SHOP PRIMING

Delete this Article if shop priming is not required or if shop finishing is specified.  Shop priming or shop finishing is recommended to seal woodwork.  See Evaluations.

A. Woodwork for Opaque Finish:  Shop prime woodwork for paint finish with one coat of wood primer specified in Division 09 painting Sections.

Retain paragraph above or first paragraph below, or both, as applicable.

B. Woodwork for Transparent Finish:  Shop seal woodwork for transparent finish with stain (if required), other required pretreatments, and first coat of finish as specified in Division 09 painting Sections:

C. Preparations for Finishing:  Comply with referenced quality standard for sanding, filling countersunk fasteners, sealing concealed surfaces, and similar preparations for finishing architectural woodwork, as applicable to each unit of work.

Usually retain subparagraph below; backpriming may prolong life of woodwork by retarding moisture absorption.  Neither AWI nor WI requires backpriming for work not shop finished.  Delete option or entire subparagraph if woodwork is stained rather than painted or varnished.

1. Backpriming:  Apply one coat of sealer or primer, compatible with finish coats, to concealed surfaces of woodwork.[  Apply two coats to surfaces installed in contact with concrete or masonry and to end-grain surfaces.]

2.16 SHOP FINISHING

A. Grade:  Provide finishes of same grades as items to be finished.

Retain paragraph above or delete and select one of three options in paragraph below.  Finish grade can be different from construction grade.

B. Grade:  [Premium] [Custom] [Economy].

Revise first paragraph below if only selected items of exterior architectural woodwork are required to be shop finished.

C. General:  Entire finish of exterior architectural woodwork is specified in this Section.  To greatest extent possible, finish architectural woodwork at fabrication shop.  Defer only final touchup and cleaning until after installation.

D. Preparations for Finishing:  Comply with referenced quality standard for sanding, filling countersunk fasteners, sealing concealed surfaces, and similar preparations for finishing architectural woodwork, as applicable to each unit of work.

Usually retain subparagraph below; backpriming may prolong life of woodwork by retarding moisture absorption.  WI does not require backpriming; AWI only requires it for certain items and grades.

1. Backpriming:  Apply one coat of sealer or primer, compatible with finish coats, to concealed surfaces of woodwork.  Apply two coats to surfaces installed in contact with concrete or masonry and to end-grain surfaces.

E. Opaque Finish:

1. AWI Finish System:  Conversion varnish.

2. AWI Finish System:  Catalyzed polyurethane.

Retain one of two subparagraphs above if AWI standard is referenced or one of three subparagraphs below if WI standard is referenced.

3. WI Finish System 4:  Conversion varnish.

4. WI Finish System 5:  Catalyzed polyurethane.

5. WI Finish System 7a:  Synthetic enamel.

Retain one of three options in subparagraph below.  If retaining first, indicate colors in a separate schedule.

6. Color:  [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range].

Retain one of three options in subparagraph below.  Numerical values for gloss ratings are those published in AWI standard.

7. Sheen:  [Satin 31-45] [Semigloss 46-60] [Gloss 61-100] gloss units measured on 60-degree gloss meter per ASTM D 523.

PART 3 -  EXECUTION

3.1 PREPARATION

A. Before installation, condition woodwork to average prevailing humidity conditions in installation areas.

B. Deliver concrete inserts and similar anchoring devices to be built into substrates well in advance of time substrates are to be built.

C. Before installing architectural woodwork, examine shop-fabricated work for completion and complete work as required, including removal of packing and backpriming.

3.2 INSTALLATION

Revise first paragraph below if installation grade is different from construction grade.

A. Quality Standard:  Install woodwork to comply with same grade specified in Part 2 for type of woodwork involved.

B. Install woodwork true and straight with no distortions.  Shim as required with concealed shims.  Install level and plumb to a tolerance of 1/8 inch in 96 inches (3 mm in 2400 mm).

C. Scribe and cut woodwork to fit adjoining work, and refinish cut surfaces or repair damaged finish at cuts.

Delete paragraph below if no fire-retardant-treated wood.

D. Fire-Retardant-Treated Wood:  Handle, store, and install fire-retardant-treated wood to comply with chemical treatment manufacturer's written instructions, including those for adhesives used to install woodwork.

Delete first paragraph below if no preservative-treated wood.

E. Preservative-Treated Wood:  Where cut or drilled in field, treat cut ends and drilled holes according to AWPA M4.

F. Anchor woodwork to anchors or blocking built in or directly attached to substrates.  Secure to grounds, stripping and blocking with countersunk concealed fasteners and blind nailing.  Use fine finishing nails for exposed nailing, countersunk and filled flush with woodwork.

Delete first paragraph and subparagraph below if not applicable.

G. Standing and Running Trim:  Install with minimum number of joints possible, using full-length pieces (from maximum length of lumber available) to greatest extent possible.  Do not use pieces less than 36 inches (900 mm) long, except where shorter single-length pieces are necessary.  Scarf running joints and stagger in adjacent and related members.

1. Install standing and running trim with no more variation from a straight line than 1/8 inch in 96 inches (3 mm in 2400 mm).

H. Complete finishing work specified in this Section to extent not completed at shop or before installation of woodwork.  Fill nail and screw holes with matching filler where exposed.

Delete paragraph above or below, or both; coordinate with Part 2 "Shop Priming" or "Shop Finishing" Article.

I. Refer to Division 09 Sections for final finishing of installed architectural woodwork.

3.3 ADJUSTING AND CLEANING

A. Repair damaged and defective woodwork, where possible, to eliminate functional and visual defects; replace woodwork where not possible to repair.  Adjust joinery for uniform appearance.

B. Clean woodwork on exposed and semiexposed surfaces.  Touch up shop-applied finishes to restore damaged or soiled areas.

END OF SECTION 064013

EXTERIOR ARCHITECTURAL WOODWORK
064013 - 1

