Copyright 2006 AIA
MASTERSPEC Full Length
05/06

Copyright 2006 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 074600 - SIDING
Revise this Section by inserting or deleting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. Section Includes:

1. [Aluminum] [Fiber-cement] [Vinyl] siding.

2. [Aluminum] [Fiber-cement] [Vinyl] soffit.

B. Related Sections:

Retain Sections in subparagraphs below that contain requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Division 06 Section "Rough Carpentry" for wood furring, grounds, nailers, and blocking.

2. Division 06 Section "Sheathing" for wall sheathing and weather-resistive barriers.

3. Division 06 Section "Exterior Finish Carpentry" for wood and wood-based sidings and for exterior trim.

1.3 SUBMITTALS

First three paragraphs below are defined in Division 01 Section "Submittal Procedures" as "Action Submittals."

A. Product Data:  For each type of product indicated.[  Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.]

Retain subparagraph below for vinyl siding if retaining requirement for certification in "Quality Assurance" Article.

1. For vinyl siding, include VSI's official certification logo printed on product data.

B. Samples for Initial Selection:  For [siding] [and] [soffit] including related accessories.

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain first paragraph below with or without above.  Retain option below if more than one type, color, texture, and pattern are required.

C. Samples for Verification:[  For each type, color, texture, and pattern required.]

Retain one of first two subparagraphs below.

1. 12-inch- (300-mm-) long-by-actual-width Sample of siding.

2. 24-inch- (600-mm-) wide-by-36-inch- (900-mm-) high Sample panel of siding assembled on plywood backing.

Retain one or both subparagraphs below if required.

3. 12-inch- (300-mm-) long-by-actual-width Sample of soffit.

4. 12-inch- (300-mm-) long-by-actual-width Samples of trim and accessories.

Remaining paragraphs are defined in Division 01 Section "Submittal Procedures" as "Informational Submittals."

Retain first paragraph below for vinyl siding and coordinate with qualification requirements in Division 01 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

D. Qualification Data:  For qualified vinyl siding Installer.

Retain first paragraph below for product certificates from manufacturers.

E. Product Certificates:  For each type of [siding] [and] [soffit], from manufacturer.

Retain first paragraph below for fiber-cement siding.

F. Product Test Reports:  Based on evaluation of comprehensive tests performed by a qualified testing agency, for fiber-cement siding.

G. Research/Evaluation Reports:  For each type of siding required, from [the ICC] <Insert applicable model code organization>.

H. Maintenance Data:  For each type of [siding] [and] [soffit] and related accessories to include in maintenance manuals.

I. Warranty:  Sample of special warranty.

1.4 QUALITY ASSURANCE

Retain first paragraph below for fiber-cement siding.  The IBC requires fiber-cement siding to be third-party tested and labeled according to ASTM C 1186.

A. Labeling:  Provide fiber-cement siding that is tested and labeled according to ASTM C 1186 by a qualified testing agency acceptable to authorities having jurisdiction.

Retain first paragraph below if a third-party certification program of vinyl siding installers is required.  Verify availability of qualified personnel in vicinity of Project if applicable.

B. Vinyl Siding Installer Qualifications:  A qualified installer who employs a VSI-Certified Installer on Project.

First paragraph below provides third-party verification that vinyl siding complies with referenced ASTM standard.

C. Vinyl Siding Certification Program:  Provide vinyl siding products that are listed in VSI's list of certified products.

D. Source Limitations:  Obtain [each type, color, texture, and pattern of] [siding] [and] [soffit], including related accessories, from single source from single manufacturer.

E. Mockups:  Build mockups to verify selections made under sample submittals and to demonstrate aesthetic effects and set quality standards for fabrication and installation.

Retain first subparagraph below for large-scale mockup.  Indicate portion of wall represented by mockup on Drawings or draw mockup as separate element.

1. Build mockup of typical wall area as shown on Drawings.

Retain first subparagraph below for limited mockups.

2. Build mockups for [siding] [and] [soffit] including accessories.

a. Size:  [48 inches (1200 mm) long by 60 inches (1800 mm) high] <Insert dimension>.

b. Include outside corner on one end of mockup[ and inside corner on other end].

Retain first subparagraph below if mockups are not only for establishing appearance factors.

3. Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if mockups are installed as part of building rather than erected separately and the intention is to make an exception to the default requirement in Division 01 Section "Quality Requirements" for demolishing and removing mockups when directed unless otherwise indicated.

4. Approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

Retain paragraph below if Work of this Section is extensive or complex enough to justify a preinstallation conference.

F. Preinstallation Conference:  Conduct conference at [Project site] <Insert location>.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Store materials in a dry, well-ventilated, weathertight place.

1.6 COORDINATION

A. Coordinate installation with flashings and other adjoining construction to ensure proper sequencing.

1.7 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

A. Special Warranty:  Standard form in which manufacturer agrees to repair or replace [siding] [and] [soffit] that fail(s) in materials or workmanship within specified warranty period.

1. Failures include, but are not limited to, the following:

a. Structural failures including cracking, deforming[, and fading].

b. Deterioration of metals, metal finishes, and other materials beyond normal weathering.

c. <Insert failure modes>.

Retain ", and fading" option above and first subparagraph below if manufacturers cover fading in their warranty.  If retaining, also retain one of three options below.  Most vinyl siding manufacturers define fading as more than 4 Hunter units.

2. Fading is defined as loss of color, after cleaning with product recommended by manufacturer, of more than [4] [5] [7] Hunter color-difference units as measured according to ASTM D 2244.

Verify available warranties and warranty periods for siding and soffit with manufacturers listed in Part 2 articles.

3. Warranty Period:  [10] [25] [50] <Insert number> years from date of Substantial Completion.

1.8 EXTRA MATERIALS

A. Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Furnish full lengths of [siding] [and] [soffit] including related accessories, in a quantity equal to 2 percent of amount installed.

PART 2 -  PRODUCTS

2.1 ALUMINUM SIDING

A. General:  Formed and coated aluminum siding complying with AAMA 1402.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Alcoa Home Exteriors, Inc.

b. Gentek Building Products, Inc.

c. Kaycan Ltd.

d. Norandex/Reynolds Distribution; an Owens Corning company.

e. Rollex Corporation.

f. <Insert manufacturer's name>.

Retain one or more of first four paragraphs below or revise to suit Project.

B. Horizontal Pattern:  8-inch (203-mm) exposure in [plain, single-] [beaded-edge, single-] [plain, double, 4-inch (102-mm)] <Insert requirement> board style.

C. Horizontal Pattern:  10-inch (254-mm) exposure in [plain,] [Dutch-lap,] <Insert requirement,> double, 5-inch (127-mm) board style.

D. Vertical Pattern:  12-inch (300-mm) exposure in board-and-batten, single-board style.

E. Vertical Pattern:  16-inch (400-mm) exposure in V-grooved, triple, 5-1/3-inch (135-mm) board style.

F. Texture:  [Smooth] [Wood grain] <Insert requirement>.

G. Nominal Thickness:  [0.019 inch (0.5 mm)] [0.024 inch (0.6 mm)] <Insert thickness>.

Retain first paragraph below if required.

H. Insulation:  Manufacturer's standard integral insulation panels.

I. Finish:  Manufacturer's standard [three-coat PVDF] [primer and baked-on acrylic] [primer and baked-on polyester] <Insert requirement>.

Retain one of three options in subparagraph below.  If retaining first option, indicate colors in a separate schedule.

1. Colors:  [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of industry colors].

2.2 FIBER-CEMENT SIDING

A. General:  ASTM C 1186, Type A, Grade II, fiber-cement board, noncombustible when tested according to ASTM E 136; with a flame-spread index of 25 or less when tested according to ASTM E 84.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Cemplank.

b. CertainTeed Corp.

Manufacturer in first subparagraph below makes only shingle-style siding.

c. GAF Materials Corporation.

d. James Hardie.

e. MaxiTile, Inc; a California corporation.

f. Nichiha Fiber Cement.

g. <Insert manufacturer's name>.

Retain one or more of first four subparagraphs below or revise to suit Project.  First subparagraph includes some sizes advertised by manufacturers listed; verify availability before retaining.  Widths are board sizes; exposure will be 1-1/4 to 1-1/2 inches (32 to 38 mm) less.

3. Horizontal Pattern:  Boards [5-1/4 inches (133 mm)] [6-1/4 to 6-1/2 inches (159 to 165 mm)] [7-1/4 to 7-1/2 inches (184 to 190 mm)] [8-1/4 to 8-1/2 inches (210 to 216 mm)] [9-1/4 to 9-1/2 inches (235 to 241 mm)] <Insert size> wide in [plain] [beaded-edge] <Insert requirement> style.

a. Texture:  [Smooth] [Rough sawn] [Wood grain] <Insert requirement>.

4. Vertical Pattern:  48-inch- (1200-mm-) wide sheets with wood-grain texture and grooves [8 inches (203 mm)] [12 inches (300 mm)] <Insert dimension> o.c.

5. Shingle Pattern:  48-inch- (1200-mm-) wide, [straight-edge notched] [staggered-edge notched] <Insert requirement> sheets with wood-grain texture.

6. Panel Texture:  48-inch- (1200-mm-) wide sheets with [smooth] [stucco] [wood-grain] <Insert requirement> texture.

Factory priming is generally offered as an optional feature by manufacturers listed.

7. Factory Priming:  Manufacturer's standard acrylic primer.

2.3 VINYL SIDING

A. General:  Integrally colored vinyl siding complying with ASTM D 3679.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

Manufacturers listed below make and sell VSI-certified products; some also make products that are sold under other brand names and are VSI certified.

a. Alcoa Home Exteriors, Inc.

b. Alside.

c. CertainTeed Corp.

d. Crane Performance Siding.

e. Gentek Building Products, Inc.

f. Heartland Building Products.

g. Kaycan Ltd.

h. Louisiana-Pacific Corporation.

i. Mitten Inc.

j. Owens Corning.

k. Resource Materials Corporation.

l. Rollex Corporation.

m. Royal Building Products.

n. Variform, Inc.

o. <Insert manufacturer's name>.

Retain one or more of first 11 paragraphs below or revise to suit Project.

B. Horizontal Pattern:  6-1/2- or 7-inch (165- or 178-mm) exposure in beaded-edge, single-board style.

C. Horizontal Pattern:  8-inch (203-mm) exposure in plain, [single-] [double, 4-inch (102-mm)] [triple, 2-2/3-inch (68-mm)] <Insert requirement> board style.

D. Horizontal Pattern:  8-inch (203-mm) exposure in Dutch-lap, double, 4-inch (102-mm) board style.

E. Horizontal Pattern:  9-inch (229-mm) exposure in plain, [double, 4-1/2-inch (114-mm)] [triple, 3-inch (76-mm)] <Insert requirement> board style.

F. Horizontal Pattern:  9-inch (229-mm) exposure in Dutch-lap, double, 4-1/2-inch (114-mm) board style.

G. Horizontal Pattern:  10-inch (254-mm) exposure in [plain,] [Dutch-lap,] <Insert requirement,> double, 5-inch (127-mm) board style.

H. Vertical Pattern:  6-inch (152-mm) exposure in V-grooved, single-board style.

I. Vertical Pattern:  8-inch (203-mm) exposure in beaded-edge, double, 4-inch (102-mm) board style.

J. Vertical Pattern:  10-inch (254-mm) exposure in V-grooved, double, 5-inch (127-mm) board style.

K. Vertical Pattern:  12-inch (300-mm) exposure in V-grooved, [double, 6-inch (152-mm)] [triple, 4-inch (102-mm)] <Insert requirement> board style.

L. Shingle Pattern:  48-inch- (1200-mm-) wide, [straight-edge notched] [staggered-edge notched] [half-round edge] [octagon edge] <Insert requirement> sheets with wood-grain texture.

M. Texture:  [Smooth] [Wood grain] <Insert requirement>.

Delete first paragraph below if 0.035-inch (0.9-mm) thickness is acceptable.  Most low-priced vinyl siding is at least 0.040 inch (1.0 mm) thick; premium siding is 0.044 to 0.048 inch (1.1 to 1.2 mm) thick.  See Evaluations.

N. Nominal Thickness:  [0.040 inch (1.0 mm)] [0.044 inch (1.1 mm)] <Insert thickness>.

Retain one of three options in first paragraph below or revise to suit Project.  Profile depth, pattern, and thickness determine stiffness.  Before retaining, verify that profile depth is available in pattern retained.

O. Minimum Profile Depth (Butt Thickness):  [1/2 inch (13 mm)] [5/8 inch (16 mm)] [3/4 inch (19 mm)] <Insert thickness>.

Nailing hem in first paragraph below is a feature of most manufacturers' premium siding that increases wind resistance.

P. Nailing Hem:  Double thickness.

Finish in paragraph below is offered by some manufacturers on their top-of-the-line siding.

Q. Finish:  Wood-grain print with clear protective coating containing not less than 70 percent PVDF.

Retain one of three options in subparagraph below.  If retaining first option, indicate colors in a separate schedule.

1. Colors:  [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of industry colors].

2.4 ALUMINUM SOFFIT

A. General:  Formed and coated aluminum soffit complying with AAMA 1402.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Alcoa Home Exteriors, Inc.

b. Gentek Building Products, Inc.

c. Kaycan Ltd.

d. Norandex/Reynolds Distribution; an Owens Corning company.

e. Rollex Corporation.

f. <Insert manufacturer's name>.

Retain one of first four paragraphs below or revise to suit Project.

B. Pattern:  6-inch (152-mm) exposure in V-grooved, single-board style.

C. Pattern:  10-inch (254-mm) exposure in V-grooved, double, 5-inch (127-mm) board style.

D. Pattern:  12-inch (300-mm) exposure in V-grooved, double, 6-inch (152-mm) board style.

E. Pattern:  16-inch (400-mm) exposure in V-grooved, [triple, 5-1/3-inch (135-mm)] [quadruple, 4-inch (102-mm)] <Insert requirement> board style.

F. Texture:  [Smooth] [Wood grain] <Insert requirement>.

G. Ventilation:  Provide [perforated] [unperforated] soffit[ unless otherwise indicated].

H. Nominal Thickness:  [0.019 inch (0.5 mm)] [0.024 inch (0.6 mm)] <Insert thickness>.

I. Finish:  Manufacturer's standard [three-coat PVDF] [primer and baked-on acrylic] [primer and baked-on polyester] <Insert requirement>.

Retain one of four options in subparagraph below.  If retaining first option, indicate colors in a separate schedule.

1. Colors:  [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of industry colors] [Match adjacent siding].

2.5 FIBER-CEMENT SOFFIT

A. General:  ASTM C 1186, Type A, Grade II, fiber-cement board, noncombustible when tested according to ASTM E 136; with a flame-spread index of 25 or less when tested according to ASTM E 84.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Cemplank.

b. CertainTeed Corp.

c. GAF Materials Corporation.

d. James Hardie.

e. <Insert manufacturer's name>.

B. Pattern:  [12-inch- (300-mm-)] [16-inch- (400-mm-)] [24-inch- (600-mm-)] <Insert dimension> wide sheets with [smooth] [wood-grain] <Insert requirement> texture.

Perforated panels are offered by James Hardie.

C. Ventilation:  Provide [perforated] [unperforated] soffit[ unless otherwise indicated].

Factory priming is generally offered as an optional feature by manufacturers listed.

D. Factory Priming:  Manufacturer's standard acrylic primer.

2.6 VINYL SOFFIT

A. General:  Integrally colored vinyl soffit complying with ASTM D 4477.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Alcoa Home Exteriors, Inc.

b. Alside.

c. CertainTeed Corp.

d. Crane Performance Siding.

e. Gentek Building Products, Inc.

f. Heartland Building Products.

g. Kaycan Ltd.

h. Louisiana-Pacific Corporation.

i. Mitten Inc.

j. Owens Corning.

k. Resource Materials Corporation.

l. Rollex Corporation.

m. Royal Building Products.

n. Variform, Inc.

o. <Insert manufacturer's name>.

Retain one of first four paragraphs below or revise to suit Project.

B. Pattern:  6-inch (152-mm) exposure in [V-grooved, single-] [beaded-edge, triple, 2-inch (51-mm)] <Insert requirement> board style.

C. Pattern:  8-inch (203-mm) exposure in V-grooved, double, 4-inch (102-mm) board style.

D. Pattern:  10-inch (254-mm) exposure in V-grooved, double, 5-inch (127-mm) board style.

E. Pattern:  12-inch (300-mm) exposure in V-grooved, [double, 6-inch (152-mm)] [triple, 4-inch (102-mm)] <Insert requirement> board style.

F. Texture:  [Smooth] [Wood grain] <Insert requirement>.

G. Ventilation:  Provide [perforated] [unperforated] soffit[ unless otherwise indicated].

Delete first paragraph below if 0.030-inch (0.8-mm) thickness is acceptable.  Most low-priced vinyl soffit is at least 0.040 inch (1.0 mm) thick; premium soffit is usually 0.044 inch (1.1 mm) thick.  See Evaluations.

H. Nominal Thickness:  [0.035 inch (0.9 mm)] [0.040 inch (1.0 mm)] [0.044 inch (1.1 mm)] <Insert thickness>.

Retain one of three options in first paragraph below or revise to suit Project.  Profile depth, pattern, and thickness determine stiffness.  Before retaining, verify that profile depth is available in pattern retained.

I. Minimum Profile Depth:  [1/2 inch (13 mm)] [5/8 inch (16 mm)] [3/4 inch (19 mm)] <Insert dimension>.

Retain one of four options in paragraph below.  If retaining first option, indicate colors in a separate schedule.

J. Colors:  [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of industry colors] [Match adjacent siding].

2.7 ACCESSORIES

Retain first paragraph below if using aluminum or vinyl siding.

A. Siding Accessories, General:  Provide starter strips, edge trim, outside and inside corner caps, and other items as recommended by siding manufacturer for building configuration.

1. Provide accessories [made from same material as] [matching color and texture of] adjacent siding unless otherwise indicated.

B. Aluminum Accessories:  Where aluminum accessories are indicated, provide accessories complying with AAMA 1402.

1. Texture:  [Smooth] [Wood grain] <Insert requirement>.

2. Nominal Thickness:  [0.019 inch (0.5 mm)] [0.024 inch (0.6 mm)] <Insert thickness>.

3. Finish:  Manufacturer's standard [three-coat PVDF] [primer and baked-on acrylic] [primer and baked-on polyester] <Insert requirement>.

C. Vinyl Accessories:  Integrally colored vinyl accessories complying with ASTM D 3679 except for wind-load resistance.

1. Texture:  [Smooth] [Wood grain] <Insert requirement>.

Retain first paragraph below if decorative accessories are required for Project.  Not all materials or types are available from all manufacturers.  Verify availability with manufacturers.

D. Decorative Accessories:  Provide the following [aluminum] [fiber-cement] [vinyl] decorative accessories as indicated:

Retain decorative accessories in subparagraphs below and revise to suit Project.  Show locations and details on Drawings.

1. Corner posts[ with fluted faces].

2. Door and window casings [with fluted faces] [and corner rosettes].

3. Entrance and window head pediments.

4. Pilasters[ with fluted faces].

5. Shutters with [paneled] [louvered] faces.

6. Louvers.

7. Lattice.

8. Fasciae.

9. Moldings and trim.

10. <Insert accessories>.

Retain one of four options in first paragraph below.  If retaining first option, indicate colors in a separate schedule.

E. Colors for Decorative Accessories:  [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of industry colors] [Match adjacent siding].

F. Flashing:  Provide [aluminum] [stainless-steel] <Insert metal> flashing complying with Division 07 Section "Sheet Metal Flashing and Trim" at window and door heads and where indicated.

1. Finish for Aluminum Flashing:  [Same as aluminum siding] [Siliconized polyester coating, same color as siding] [High-performance organic finish, same color as siding] [Factory-prime coating] <Insert finish>.

G. Fasteners:

Retain first subparagraph below if fastening siding and soffits to wood framing.

1. For fastening to wood, use [siding nails] [ribbed bugle-head screws] of sufficient length to penetrate a minimum of 1 inch (25 mm) into substrate.

Retain first subparagraph below if fastening siding and soffits to metal framing.

2. For fastening to metal, use ribbed bugle-head screws of sufficient length to penetrate a minimum of 1/4 inch (6 mm), or three screw-threads, into substrate.

Retain first subparagraph below if using aluminum siding or soffit.

3. For fastening aluminum, use aluminum fasteners.  Where fasteners will be exposed to view, use prefinished aluminum fasteners in color to match item being fastened.

Retain first subparagraph below if using fiber-cement siding or soffit.

4. For fastening fiber cement, use [hot-dip galvanized] [stainless-steel] fasteners.

Retain subparagraph below if using vinyl siding or soffit.

5. For fastening vinyl, use [aluminum] [hot-dip galvanized] [stainless-steel] fasteners.  Where fasteners will be exposed to view, use prefinished aluminum fasteners in color to match item being fastened.

H. Insect Screening for Soffit Vents:  [Aluminum, 18-by-16 (1.4-by-1.6-mm) mesh] [PVC-coated, glass-fiber fabric, 18-by-14 or 18-by-16 (1.4-by-1.8- or 1.4-by-1.6-mm) mesh] [Stainless steel, 18-by-18 (1.4-by-1.4-mm) mesh] <Insert requirement>.

I. Continuous Soffit Vents:  Aluminum, hat-channel shape, with [stamped louvers] [perforations]; 2 inches (51 mm) wide and not less than 96 inches (2438 mm) long.

1. Net-Free Area:  [4 sq. in./linear ft. (280 sq. cm/m)] [6 sq. in./linear ft. (420 sq. cm/m)] [8 sq. in./linear ft. (560 sq. cm/m)] <Insert dimension>.

2. Finish:  [Mill finish] [White paint] [Brown paint] <Insert requirement>.

J. Round Soffit Vents:  Stamped aluminum louvered vents, [2 inches (51 mm)] [2-1/2 inches (64 mm)] [3 inches (76 mm)] [4 inches (102 mm)] <Insert dimension> in diameter, made to be inserted into round holes cut into soffit.

1. Finish:  [Mill finish] [White paint] [Brown paint] <Insert requirement>.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine substrates for compliance with requirements for installation tolerances and other conditions affecting performance of [siding] [and] [soffit] and related accessories.

B. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. Clean substrates of projections and substances detrimental to application.

3.3 INSTALLATION

A. General:  Comply with [siding] [and] [soffit] manufacturer's written installation instructions applicable to products and applications indicated unless more stringent requirements apply.

1. Do not install damaged components.

Retain subparagraph below if using aluminum or vinyl siding.

2. Center nails in elongated nailing slots without binding siding to allow for thermal movement.

B. Install aluminum [siding] [and] [soffit] and related accessories according to AAMA 1402.

1. Install fasteners no more than [24 inches (600 mm)] <Insert dimension> o.c.

C. Install fiber-cement [siding] [and] [soffit] and related accessories.

1. Install fasteners no more than [24 inches (600 mm)] <Insert dimension> o.c.

D. Install vinyl [siding] [and] [soffit] and related accessories according to ASTM D 4756.

1. Install fasteners for horizontal vinyl siding no more than [16 inches (400 mm)] <Insert dimension> o.c.

2. Install fasteners for vertical vinyl siding no more than [12 inches (300 mm)] <Insert dimension> o.c.

E. Install joint sealants as specified in Division 07 Section "Joint Sealants" and to produce a weathertight installation.

Retain paragraph below if using aluminum siding, related accessories, or trim.

F. Where aluminum siding will contact dissimilar metals, protect against galvanic action by painting contact surfaces with primer or by applying sealant or tape or installing nonconductive spacers as recommended by manufacturer for this purpose.

3.4 ADJUSTING AND CLEANING

A. Remove damaged, improperly installed, or otherwise defective materials and replace with new materials complying with specified requirements.

B. Clean finished surfaces according to manufacturer's written instructions and maintain in a clean condition during construction.

END OF SECTION 074600

	SIDING
	074600 - 1



