Copyright 2011 AIA
MasterSpec Professional
11/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 083473.16 - WOOD SOUND CONTROL DOOR ASSEMBLIES
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes wood sound control door assemblies.

1.2 PREINSTALLATION MEETINGS

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

1.3 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

First option in "Certificates for (Credit MR 6) (Credit MR 7)" Subparagraph below applies to LEED-CS; second applies to LEED-NC, LEED-CI, and LEED for Schools.

1. Certificates for [Credit MR 6] [Credit MR 7]: Chain-of-custody certificates indicating that flush wood doors comply with forest certification and chain-of-custody requirements. Include statement indicating cost for each certified wood product.

"Product Data for Credit IEQ 4.4" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS.

2. Product Data for Credit IEQ 4.4: For adhesives and composite wood products, documentation indicating that product contains no urea formaldehyde.

"Laboratory Test Reports for Credit IEQ 4.1" Subparagraph below applies to LEED for Schools.

3. Laboratory Test Reports for Credit IEQ 4.1: For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Public Health's (formerly, the California Department of Health Services') "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

"Laboratory Test Reports for Credit IEQ 4.2" Subparagraph below applies to LEED for Schools.

4. Laboratory Test Reports for Credit IEQ 4.2: For paints and coatings, documentation indicating that products comply with the testing and product requirements of the California Department of Public Health's (formerly, the California Department of Health Services') "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

"Laboratory Test Reports for Credit IEQ 4.4" Subparagraph below applies to LEED for Schools.

5. Laboratory Test Reports for Credit IEQ 4.4: For composite wood products, documentation indicating that products comply with the testing and product requirements of the California Department of Public Health's (formerly, the California Department of Health Services') "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

C. Shop Drawings: For sound control door assemblies. Include elevations, details, seals, anchorages, and accessories.

D. Samples: For units with factory-applied finishes.

E. Schedule: Provide a schedule of sound control door assemblies prepared using same reference numbers for details and openings as those on Drawings.

1.4 INFORMATIONAL SUBMITTALS

A. Product certificates.

B. Product test reports.

1.5 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.6 QUALITY ASSURANCE

Retain "Manufacturer Qualifications" and "Vendor Qualifications" paragraphs below if required for LEED.

A. Manufacturer Qualifications: A qualified manufacturer that [is certified for chain of custody by an FSC-accredited certification body] [and] [is a certified participant in AWI's Quality Certification Program] [is a licensee of WI's Certified Compliance Program].

Chain-of-custody certification is not required for subcontractors (entities that install products) but is required for vendors unless they are simply acting as an agent of manufacturer and the sale is actually between manufacturer and Contractor or subcontractor.

B. Vendor Qualifications: A vendor that is certified for chain of custody by an FSC-accredited certification body.

1.7 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of sound control door assemblies that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for units and components.

1. Warranty Period: [Two] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PERFORMANCE REQUIREMENTS

See the Sound Control Door Assembly Table at the end of the Evaluations for a list of manufacturers and STC ratings. Higher ratings may require doors that are thicker than 1-3/4 inches (44 mm). Verify with manufacturer.

A. STC Rating: [As indicated on Drawings] [As indicated in the Door Schedule] <Insert STC rating> as calculated by ASTM E 413 when tested in an operable condition according to ASTM E 90.

2.2 WOOD SOUND CONTROL DOORS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. AMBICO Limited.

2. Eggers Industries.

3. Krieger Specialty Products Company.

4. Marshfield DoorSystems, Inc.

5. Overly Door Company.

6. Security Acoustics; a division of Security Metal Products Corp.

7. Vancouver Door Company.

8. <Insert manufacturer's name>.

C. Certified Wood: Wood doors shall be certified as "FSC Pure"[ or "FSC Mixed Credit"] according to FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship," and to FSC STD-40-004, "FSC Standard for Chain of Custody Certification."

Retain first "Low-Emitting Materials" Paragraph below if required for LEED-NC, LEED-CS, or LEED-CI Credit IEQ 4.4. Before retaining, verify availability with manufacturers.

D. Low-Emitting Materials: Fabricate doors with adhesives and composite wood products that do not contain urea formaldehyde.

Retain "Low-Emitting Materials" Paragraph below if required for LEED for Schools Credit IEQ 4.1 or Credit IEQ 4.4.

E. Low-Emitting Materials: Fabricate doors with [adhesives] [and] [composite wood products] that comply with the testing and product requirements of the California Department of Public Health's (formerly, the California Department of Health Services') "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

F. Doors: Flush-design sound control doors, [1-3/4 inches (44 mm) thick] [2 inches (51 mm) thick] [2-1/4 inches (57 mm) thick] [thickness as required to provide STC rating] <Insert dimension>; with manufacturer's standard sound-retardant core as required to provide STC rating indicated. Fabricate according to WDMA 1.S.1-A.

G. Glazing: [Manufacturers' standard factory-installed glazing.] [Comply with requirements in Section 088000 "Glazing."]

If no other flush wood doors are required for Project, revise "Materials" and "Finishes" paragraphs below by inserting requirements for materials and finishes.

H. Materials: Comply with [Section 081416 "Flush Wood Doors"] [Section 081433 "Stile and Rail Wood Doors"] for grade, faces, veneer matching, fabrication, finishing, and other requirements unless otherwise indicated.

I. Finishes:

1. Factory finish sound control wood doors to match doors specified in [Section 081416 "Flush Wood Doors."] [Section 081433 "Stile and Rail Wood Doors."]

Retain paragraph below if required for LEED for Schools Credit IEQ 4.

J. Use only paints and coatings that comply with the testing and product requirements of the California Department of Public Health's (formerly, the California Department of Health Services') "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

2.3 SOUND CONTROL FRAMES

A. Frames: Fabricate sound control door frames with corners mitered, reinforced, and continuously welded the full depth and width of frame. Fabricate according to NAAMM-HMMA 865.

1. Weld frames according to NAAMM-HMMA 820.

2. Interior Frames: Fabricate from cold-rolled steel sheet unless otherwise indicated, 0.075-inch (1.90-mm) nominal thickness or thicker as required to provide STC rating indicated.

3. Hardware Reinforcement: Fabricate according to NAAMM-HMMA 865 of same material as face sheets.

B. Materials:

1. Cold-Rolled Steel Sheet: ASTM A 1008/A 1008M, Commercial Steel (CS), Type B.

2. Hot-Rolled Steel Sheet: ASTM A 1011/A 1011M, Commercial Steel (CS), Type B.

3. Mineral-Fiber Insulation: Insulation composed of rock-wool fibers, slag-wool fibers, or glass fibers.

C. Finishes:

Retain "Prime Finish" or "Factory-Applied Paint Finish" Subparagraph below.

1. Prime Finish: Apply manufacturer's standard primer immediately after cleaning and pretreating.

2. Factory-Applied Paint Finish: Manufacturer's standard primer and finish coats, complying with SDI A250.3 for performance and acceptance criteria.

a. Color and Gloss: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

2.4 HARDWARE

A. Sound Control Door Hardware: Manufacturer's standard sound control system, including head and jamb seals, door bottoms, [cam-lift hinges, ] and thresholds, as required by testing to achieve STC[ and fire] rating indicated.

1. Compression Seals: One-piece units consisting of closed-cell sponge neoprene or silicone seal held in place by metal retainer, with retainer cover of same material as door frame; attached to door frame with concealed screws.

2. Magnetic Seals: One-piece units consisting of closed-cell sponge neoprene seal and resiliently mounted magnet held in place by metal retainer, with retainer cover of same material as door frame; attached to door frame with concealed screws.

Retain "Automatic Door Bottoms" or "Door Bottoms" Subparagraph below.

3. Automatic Door Bottoms: Neoprene or silicone gasket, held in place by metal housing, that automatically drops to form seal when door is closed; mounted to bottom edge of door with screws.

4. Door Bottoms: Neoprene or silicone gasket held in place by metal housing; mortised into bottom edge of door.

5. Cam-Lift Hinges: Full-mortise template type that raises door 1/2 inch (13 mm) when door is fully open; with hardened pin; fabricated from stainless steel.

6. Thresholds: Flat, smooth, unfluted type as recommended by manufacturer; fabricated from [aluminum] [stainless steel] [solid wood matching wood door faces].

B. Other Hardware: Comply with requirements in [Section 087100 "Door Hardware."] [Section 087111 "Door Hardware (Descriptive Specification)."]

2.5 FABRICATION

A. Wood Sound Control Door Fabrication: Factory fit doors to suit frame-opening sizes indicated, with uniform clearances and bevels according to WDMA I.S.1-A unless otherwise indicated. Comply with final door hardware schedules and hardware templates.

Delete "Glazed Lites" Subparagraph below if lites are field installed or if no lites.

1. Glazed Lites: Factory install glazed lites according to requirements of tested assembly to achieve STC rating indicated.

2. Locate door hardware as indicated, or if not indicated, according to DHI WDHS.3, "Recommended Locations for Architectural Hardware for Wood Flush Doors."

a. Coordinate measurements of hardware mortises in steel frames to verify dimensions and alignment before factory machining.

B. Sound Control Frame Fabrication:

1. Weld flush face joints continuously; grind, fill, dress, and make smooth, flush, and invisible. Where frames are fabricated in sections due to shipping or handling limitations, provide alignment plates or angles at each joint, fabricated from same thickness metal as frames.

2. Provide countersunk, flat- or oval-head exposed screws and bolts for exposed fasteners unless otherwise indicated.

3. Anchors: Provide number and spacing of anchors as indicated in NAAMM-HMMA 865.

a. Postinstalled Expansion Type: Locate anchors not more than 6 inches (152 mm) from top and bottom of frame. Space anchors not more than 26 inches (660 mm) o.c.

4. Hardware Preparation: Factory prepare sound control frames to receive templated mortised hardware; include cutouts, reinforcement, mortising, drilling, and tapping.

a. Reinforce frames to receive nontemplated mortised and surface-mounted door hardware.

5. Tolerances: Fabricate frames to tolerances indicated in NAAMM-HMMA 865.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Frames: Install sound control door frames in sizes and profiles indicated.

1. Set frames accurately in position; plumbed, aligned, and braced securely until permanent anchors are set. After wall construction is complete, remove temporary braces, leaving surfaces smooth and undamaged.

a. Where frames are fabricated in sections due to shipping or handling limitations, field splice at approved locations by welding face joint continuously; grind, fill, and dress; make splice smooth, flush, and invisible on exposed faces.

b. Install sound control frames with removable glazing stops located on secure side of opening.

c. Remove temporary braces only after frames or bucks have been properly set and secured.

d. Check squareness, twist, and plumbness of frames as walls are constructed. Shim as necessary to comply with installation tolerances.

e. Apply corrosion-resistant coating to backs of frames to be filled with mortar, grout, and plaster containing antifreezing agents.

2. Floor Anchors: Provide floor anchors for each jamb and mullion that extends to floor, and secure with postinstalled expansion anchors.

3. Metal-Stud Partitions: Fully fill frames with mineral-fiber insulation.

4. In-Place Concrete or Masonry Construction: Secure frames in place with postinstalled expansion anchors. Countersink anchors, and fill and make smooth, flush, and invisible on exposed faces.

5. In-Place Gypsum Board Partitions: Secure frames in place with postinstalled expansion anchors through floor anchors at each jamb. Countersink anchors, and fill and make smooth, flush, and invisible on exposed faces.

6. Ceiling Struts: Extend struts vertically from top of frame at each jamb to supporting construction above unless frame is anchored to masonry or to other structural support at each jamb. Bend top of struts to provide flush contact for securing to supporting construction above. Provide adjustable wedged or bolted anchorage to frame jamb members.

7. Installation Tolerances: Adjust sound control door frames for squareness, alignment, twist, and plumbness to the following tolerances as indicated in NAAMM-HMMA 865.

B. Doors: Fit sound control doors accurately in frames, within clearances as indicated in NAAMM-HMMA 865.

C. Sound Control Seals: Where seals have been factory prefit and preinstalled and subsequently removed for shipping, reinstall seals and adjust according to manufacturer's written instructions.

D. Cam-Lift Hinges: Install hinges according to manufacturer's written instructions.

E. Thresholds: Set thresholds in full bed of sealant complying with requirements in Section 079200 "Joint Sealants."

F. Glazing: Comply with installation requirements in Section 088000 "Glazing" and with sound control door assembly manufacturer's written instructions.

END OF SECTION 083473.16
	WOOD SOUND CONTROL DOOR ASSEMBLIES
	083473.16 - 1



