Copyright 2010 AIA
MasterSpec Professional
02/10

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 092713 - GLASS-FIBER-REINFORCED PLASTER FABRICATIONS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes factory-molded, glass-fiber-reinforced plaster fabrications for interior applications.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

"Product Data for Credit EQ 4.1" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for adhesives.

1. Product Data for Credit EQ 4.1: For adhesives, documentation including printed statement of VOC content.

"Laboratory Test Reports for Credit EQ 4" Subparagraph below applies to LEED for Schools.

2. Laboratory Test Reports for Credit EQ 4: For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

C. Shop Drawings: Include plans, elevations, sections, details, and attachments to other work.

"Samples" Paragraph below assumes manufacturer's standard-size Samples are acceptable. Revise to suit Project.

D. Samples: For each exposed product and for each color and texture specified.

1.3 DELIVERY, STORAGE, AND HANDLING

A. Comply with ASTM C 1467/C 1467M.

1.4 FIELD CONDITIONS

A. Environmental Conditions: Comply with ASTM C 1467/C 1467M.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 GLASS-FIBER-REINFORCED PLASTER FABRICATIONS

A. Fabrications: Molded, glass-fiber-reinforced plaster units complying with ASTM C 1381/C 1381M.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Architectural Reproductions Inc.

b. Casting Designs, Inc.

c. DEC Architectural Composites; Division of DEC Associates, Inc.

d. Felber Ornamental Plastering Corporation.

e. First Class Building Products, Inc.

f. Formglas Inc.

g. Melton Classics, Inc.

h. Plastrglas, Incorporated.

i. Stromberg Architectural Products, Inc.

j. <Insert manufacturer's name>.

B. Embedments: [Cold-rolled steel channels with ASTM 653/A 653M, G60 (Z180) hot-dip galvanized coating] [As standard with glass-fiber-reinforced plaster fabrication manufacturer and as required for reinforcement and for anchorage to substrates and framing] <Insert requirements>.

Not all manufacturers recommend gloss paint finish or offer more than a smooth paint finish.

C. Finish: [Smooth for paint finish] [Smooth for gloss paint finish] <Insert finish>.

2.2 AUXILIARY MATERIALS

A. Adhesives: As recommended in glass-fiber-reinforced plaster fabrication manufacturer's written instructions[.] [ and as follows:]

Retain first subparagraph below if required for LEED-NC, LEED-CI, LEED-CS Credit EQ 4.1.

1. Adhesive shall have VOC content of [50] <Insert value> g/L or less when calculated according to 40 CFR 59, Subpart D (EPA Method 24).

Retain subparagraph below if required for LEED for Schools Credit EQ 4.

2. Adhesive shall comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

B. Steel Drill Screws: Of sufficient length and size to securely fasten glass-fiber-reinforced plaster fabrications to framing members, and as follows:

Retain only those applications below that are required for Project.

1. Screws complying with ASTM C 1002 for fastening glass-fiber-reinforced plaster fabrications to steel members less than 0.033 inch (0.84 mm) thick.

2. Screws complying with ASTM C 1002 for fastening glass-fiber-reinforced plaster fabrications to wood members.

3. Screws complying with ASTM C 954 for fastening glass-fiber-reinforced plaster fabrications to steel members from 0.033 to 0.112 inch (0.84 to 2.84 mm) thick.

C. Joint-Treatment Materials: ASTM C 475/C 475M.

Retain "Control Joints" Paragraph below if control joints are required. If retaining, revise if another type of control joint is required. Indicate locations of control joints on Drawings.

D. Control Joints: ASTM C 1047, one-piece control joint with V-shaped slot and removable strip covering the slot opening.

1. Material: [Steel sheet zinc-coated by hot-dip process] [Rolled zinc].

PART 3 -  EXECUTION

3.1 GLASS-FIBER-REINFORCED PLASTER INSTALLATION

A. Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

B. Proceed with installation only after unsatisfactory conditions have been corrected.

C. Comply with ASTM C 1467/C 1467M.

D. Install glass-fiber-reinforced plaster fabrications level, plumb, true, and aligned with adjacent materials. Use concealed shims where required for alignment.

E. Attach glass-fiber-reinforced plaster fabrications to framing and substrates with steel drill screws unless otherwise indicated. Do not use pneumatic staple guns. Countersink screw heads below adjoining finished surface.

1. Predrill fastener holes in units. Clean fastener holes to remove dirt and oil.

2. Locate fasteners not less than 5/16 inch (7.9 mm) from edges or ends of units.

F. Where glass-fiber-reinforced plaster fabrications are joined to form composite units, join fabrications with adhesive. Band or brace units together until adhesive cures.

G. Install control joints between glass-fiber-reinforced plaster fabrications where indicated.

H. Use joint-treatment materials to finish glass-fiber-reinforced plaster fabrications to produce surfaces ready to receive primers and paint finishes specified in Section 099113 "Exterior Painting" and Section 099123 "Interior Painting."

1. Finish joints between units, other than control joints, and countersunk fastener heads to comply with ASTM C 840 for [Level 4] [Level 5] and to match surface texture of units.

2. Repair hollows, voids, scratches, and other surface imperfections on units.

END OF SECTION 092713

	GLASS-FIBER-REINFORCED PLASTER FABRICATIONS
	092713 - 1



