Copyright 2011 AIA
MasterSpec Professional
02/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 095113 - ACOUSTICAL PANEL CEILINGS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes acoustical panels and exposed suspension systems for ceilings.

1.2 PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in SECTION 013100 "Project Management and Coordination."

1.3 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

1. Product Data for Credit MR 4: For products having recycled content, documentation indicating percentages by weight of postconsumer and preconsumer recycled content. Include statement indicating costs for each product having recycled content.

"Laboratory Test Reports for Credit EQ 4" Subparagraph below applies to LEED for Schools.

2. Laboratory Test Reports for Credit EQ 4: For ceiling systems, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

"Samples" Paragraph below assumes manufacturer's standard-size Samples are acceptable. Revise to suit Project.

C. Samples: For each exposed product and for each color and texture specified.

1.4 INFORMATIONAL SUBMITTALS

A. Product test reports.

B. Evaluation reports.

C. Field quality-control reports.

1.5 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.6 QUALITY ASSURANCE

A. Testing Agency Qualifications: Qualified according to NVLAP.

B. Mockups: Build mockups to verify selections made under sample submittals and to demonstrate aesthetic effects and set quality standards for materials and execution.

Indicate portion of ceiling represented by mockup on Drawings or draw mockup as separate element.

1. Build mockup of typical ceiling area as shown on Drawings.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

2. Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PERFORMANCE REQUIREMENTS

Retain "Seismic Performance" Paragraph below for projects requiring seismic design. Delete paragraph if performance requirements are indicated on Drawings. Retain option if International Building Code seismic design requirements prevail. Coordinate requirements with structural engineer.

A. Seismic Performance: Acoustical ceiling shall withstand the effects of earthquake motions determined according to [ASCE/SEI 7] <Insert requirement>.

B. Surface-Burning Characteristics: Comply with ASTM E 84; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

Retain one option in "Flame-Spread Index" Subparagraph below.

1. Flame-Spread Index: Comply with ASTM E 1264 for [Class A] [Class B] [Class C] materials.

2. Smoke-Developed Index: [50] [450] <Insert value> or less.

2.2 ACOUSTICAL PANEL CEILINGS, GENERAL

Retain "Low-Emitting Materials" Paragraph below if required for LEED for Schools Credit EQ 4.

A. Low-Emitting Materials: Acoustical panel ceilings shall comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

Retain "Recycled Content" Paragraph below to specify recycled content if applying for LEED Credit MR 4.

B. Recycled Content: Postconsumer recycled content plus one-half of preconsumer recycled content not less than <Insert number> percent.

C. Glass-Fiber-Based Panels: Made with binder containing no urea formaldehyde.

D. Acoustical Panel Standard: Comply with ASTM E 1264.

E. Metal Suspension System Standard: Comply with ASTM C 635.

F. Attachment Devices: Size for five times the design load indicated in ASTM C 635, Table 1, "Direct Hung," unless otherwise indicated. Comply with seismic design requirements.

2.3 ACOUSTICAL PANELS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert number to complete drawing designation. Use same designation for acoustical panels in this article and for suspension system in "Metal Suspension System" Article; together, they make up the ceiling. Use these designations on Drawings to identify each ceiling assembly.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. Armstrong World Industries, Inc.

2. CertainTeed Corp.

3. Chicago Metallic Corporation.

4. Tectum Inc.

5. USG Interiors, Inc.; Subsidiary of USG Corporation.

6. <Insert manufacturer's name>.

Do not assume that every combination of fire-resistance rating, classification, pattern, color, light reflectance, acoustical rating, edge detail, thickness, and size listed under each product description is available. Before retaining salient characteristics, verify availability with manufacturers.

C. Classification: <Insert type, form, and pattern>.

D. Color: [White] [As selected from manufacturer's full range] [Match Architect's sample] [As indicated by manufacturer's designation] [As indicated on Drawings] [As indicated in a schedule] <Insert color>.

E. LR: <Insert LR>.

F. NRC: <Insert NRC>, Type E-400 mounting according to ASTM E 795.

G. CAC: <Insert CAC>.

H. AC: <Insert AC>.

I. Edge/Joint Detail: [Square] [Reveal sized to fit flange of exposed suspension-system members] [Flush reveal sized to fit flange of exposed suspension-system members] [Beveled, kerfed and rabbeted long edges and square, butt-on short edges] <Insert manufacturer's special proprietary edge detail>.

J. Thickness: [1/8 inch (3 mm)] [7/16 inch (12 mm)] [9/16 inch (15 mm)] [5/8 inch (15 mm)] [3/4 inch (19 mm)] [7/8 inch (22 mm)] [1 inch (25 mm)] [1-1/2 inches (38 mm)] [2 inches (51 mm)] [3 inches (76 mm)] [As indicated on Drawings] [As indicated in a schedule] <Insert dimension>.

K. Modular Size: [24 by 24 inches (610 by 610 mm)] [24 by 48 inches (610 by 1220 mm)] [600 by 600 mm] [600 by 1200 mm] [As indicated on Drawings] [As indicated in a schedule] <Insert size>.

2.4 METAL SUSPENSION SYSTEM <Insert drawing designation>

Copy this article and re-edit for each product.

Insert number to complete drawing designation. Use same designation for suspension system in this article and for acoustical panels in "Acoustical Panels" Article; together, they make up the ceiling. Use these designations on Drawings to identify each ceiling assembly.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. Armstrong World Industries, Inc.

2. CertainTeed Corp.

3. Chicago Metallic Corporation.

4. USG Interiors, Inc.; Subsidiary of USG Corporation.

5. <Insert manufacturer's name>.

"Narrow-Face, Capped, Double-Web, Steel Suspension System" Paragraph below is an example of a suspension system for acoustical panel ceilings; retain or revise to suit Project.

C. Narrow-Face, Capped, Double-Web, Steel Suspension System: Main and cross runners roll formed from cold-rolled steel sheet; prepainted, electrolytically zinc coated, or hot-dip galvanized according to ASTM A 653/A 653M, not less than G30 (Z90) coating designation; with prefinished 9/16-inch- (15-mm-) wide metal caps on flanges.

1. Structural Classification: [Intermediate] [Heavy]-duty system.

2. End Condition of Cross Runners: [Override (stepped)] [or] [butt-edge] type.

3. Face Design: [Flat, flush] [Flanges formed with an integral center reveal].

4. Cap Material: [Steel] [or] [aluminum] cold-rolled sheet.

Coordinate finish in "Cap Finish" Subparagraph below with metal type selected.

5. Cap Finish: [Painted white] [Painted in color as selected from manufacturer's full range] [Painted to match color indicated by manufacturer's designation] [Painted to match color of acoustical unit] [Plated with metallic finish as selected from manufacturer's full range] [Plated with metallic finish indicated by manufacturer's designation] [Natural finish for aluminum].

D. Roll-Formed, Sheet-Metal Edge Moldings and Trim: Type and profile indicated or, if not indicated, manufacturer's standard moldings for edges and penetrations that comply with seismic design requirements; formed from sheet metal of same material, finish, and color as that used for exposed flanges of suspension-system runners.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Install acoustical panel ceilings to comply with ASTM C 636/C 636M and seismic design requirements indicated, according to manufacturer's written instructions and CISCA's "Ceiling Systems Handbook."

B. Measure each ceiling area and establish layout of acoustical panels to balance border widths at opposite edges of each ceiling. Avoid using less-than-half-width panels at borders, and comply with layout shown on reflected ceiling plans.

1. Arrange directionally patterned acoustical panels as indicated on reflected ceiling plans.

END OF SECTION 095113

	ACOUSTICAL PANEL CEILINGS
	095113 - 1



