Copyright 2011 AIA
MasterSpec Professional
05/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 096513 - RESILIENT BASE AND ACCESSORIES
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Resilient base.

2. Resilient stair accessories.

3. Resilient molding accessories.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

"Product Data for Credit IEQ 4.1" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for adhesives.

1. Product Data for Credit IEQ 4.1: For adhesives, documentation including printed statement of VOC content.

"Laboratory Test Reports for Credit IEQ 4.1" Subparagraph below applies to LEED for Schools.

2. Laboratory Test Reports for Credit IEQ 4.1: For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

First "Product Data for Credit IEQ 4.3" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for adhesives.

3. Product Data for Credit IEQ 4.3: For adhesives, documentation including printed statement of VOC content.

"Product Data for Credit IEQ 4.3" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for resilient stair accessories.

4. Product Data for Credit IEQ 4.3: For resilient stair accessories, documentation from an independent testing agency indicating compliance with the FloorScore standard.

"Laboratory Test Reports for Credit IEQ 4.3" Subparagraph below applies to LEED for Schools.

5. Laboratory Test Reports for Credit IEQ 4.3: For resilient stair accessories, documentation indicating that products comply with the testing and product requirements of the California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

C. Samples: For each exposed product and for each color and texture specified, not less than 12 inches (300 mm) long.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PERFORMANCE REQUIREMENTS

Retain "FloorScore Compliance" Paragraph below if required for LEED-NC, LEED-CS, or LEED-CI Credit IEQ 4.3; before retaining, verify availability of materials that comply.

A. FloorScore Compliance: Resilient [base] [and] [stair accessories] shall comply with requirements of FloorScore certification.

Retain "Low-Emitting Materials" Paragraph below if required for LEED for Schools Credit IEQ 4.1 or Credit IEQ 4.3.

B. Low-Emitting Materials: Flooring system shall comply with the testing and product requirements of the California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

2.2 THERMOSET-RUBBER BASE <Insert drawing designation>

Copy this article and re-edit for each thermoset-rubber base product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. Burke Mercer Flooring Products, Division of Burke Industries Inc.

2. Flexco.

3. Roppe Corporation, USA.

4. <Insert manufacturer's name>.

B. Product Standard: ASTM F 1861, Type TS (rubber, vulcanized thermoset), Group I (solid, homogeneous).

1. Style and Location:

a. Style A, Straight: [Provide in areas with carpet] <Insert requirements>.

b. Style B, Cove: [Provide in areas with resilient flooring] <Insert requirements>.

c. Style C, Butt to: [Provide in areas indicated] <Insert requirements>.

C. Thickness: 0.125 inch (3.2 mm).

Coordinate height retained in "Height" Paragraph with length retained in "Lengths" Paragraph below; some manufacturers do not offer coils for every height.

D. Height: [2-1/2 inches (64 mm)] [4 inches (102 mm)] [6 inches (152 mm)] [As indicated on Drawings].

E. Lengths: [Cut lengths 48 inches (1219 mm) long] [Coils in manufacturer's standard length] [Cut lengths 48 inches (1219 mm) long or coils in manufacturer's standard length].

Coordinate "Outside Corners" and "Inside Corners" paragraphs below with "Resilient Base Installation" Article.

F. Outside Corners: [Job formed] [Preformed] [Job formed or preformed].

G. Inside Corners: [Job formed] [Preformed] [Job formed or preformed].

Retain "Colors" Paragraph below if colors are not indicated in a separate schedule.

H. Colors: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors>.

2.3 THERMOPLASTIC-RUBBER BASE <Insert drawing designation>

Copy this article and re-edit for each thermoplastic-rubber base product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. AB; American Biltrite.

2. Allstate Rubber Corp.

3. Armstrong World Industries, Inc.

4. Burke Mercer Flooring Products, Division of Burke Industries Inc.

5. Flexco.

6. Johnsonite; A Tarkett Company.

7. Mondo Rubber International, Inc.

8. Nora Systems, Inc.

9. Roppe Corporation, USA.

10. VPI, LLC, Floor Products Division.

11. <Insert manufacturer's name>.

B. Product Standard: ASTM F 1861, Type TP (rubber, thermoplastic).

1. Group: [I (solid, homogeneous)] [or] [II (layered)].

2. Style and Location:

a. Style A, Straight: [Provide in areas with carpet] <Insert requirements>.

b. Style B, Cove: [Provide in areas with resilient flooring] <Insert requirements>.

c. Style C, Butt to: [Provide in areas indicated] <Insert requirements>.

d. Style D, Sculptured: [Provide in areas indicated] <Insert requirements>.

1) Profile: [As indicated] <Insert requirement>.

Thermoplastic-rubber base is generally 0.125 inch (3.2 mm) thick; however, sculptured base thicknesses might vary.

C. Thickness: [0.125 inch (3.2 mm)] <Insert dimension>.

Coordinate height retained in "Height" Paragraph with length retained in "Lengths" Paragraph below; some manufacturers do not offer coils for every height.

D. Height: [2-1/2 inches (64 mm)] [4 inches (102 mm)] [6 inches (152 mm)] [As indicated on Drawings].

E. Lengths: [Cut lengths 48 inches (1219 mm) long] [Coils in manufacturer's standard length] [Cut lengths 48 inches (1219 mm) long or coils in manufacturer's standard length].

Coordinate "Outside Corners" and "Inside Corners" paragraphs below with "Resilient Base Installation" Article.

F. Outside Corners: [Job formed] [Preformed] [Job formed or preformed].

G. Inside Corners: [Job formed] [Preformed] [Job formed or preformed].

Retain "Colors" Paragraph below if colors are not indicated in a separate schedule.

H. Colors: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors>.

2.4 VINYL BASE <Insert drawing designation>

Copy this article and re-edit for each vinyl base product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. Armstrong World Industries, Inc.

2. Burke Mercer Flooring Products, Division of Burke Industries Inc.

3. Flexco.

4. Johnsonite; A Tarkett Company.

5. Roppe Corporation, USA.

6. VPI, LLC, Floor Products Division.

7. <Insert manufacturer's name>.

B. Product Standard: ASTM F 1861, Type TV (vinyl, thermoplastic).

1. Group: [I (solid, homogeneous)] [or] [II (layered)].

2. Style and Location:

a. Style A, Straight: [Provide in areas with carpet] <Insert requirements>.

b. Style B, Cove: [Provide in areas with resilient flooring] <Insert requirements>.

C. Minimum Thickness: [0.125 inch (3.2 mm)] [0.080 inch (2.0 mm)] <Insert dimension>.

Coordinate height retained in "Height" Paragraph with length retained in "Lengths" Paragraph below; some manufacturers do not offer coils for every height.

D. Height: [2-1/2 inches (64 mm)] [4 inches (102 mm)] [6 inches (152 mm)] [As indicated on Drawings].

E. Lengths: [Cut lengths 48 inches (1219 mm) long] [Coils in manufacturer's standard length] [Cut lengths 48 inches (1219 mm) long or coils in manufacturer's standard length].

Coordinate "Outside Corners" and "Inside Corners" paragraphs below with "Resilient Base Installation" Article.

F. Outside Corners: [Job formed] [Preformed] [Job formed or preformed].

G. Inside Corners: [Job formed] [Preformed] [Job formed or preformed].

Retain "Colors and Patterns" Paragraph below if colors and patterns are not indicated in a separate schedule.

H. Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

2.5 RUBBER STAIR ACCESSORIES <Insert drawing designation>

Copy this article and re-edit for each set of rubber stair-accessory product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Fire-Test-Response Characteristics" Paragraph below if required by authorities having jurisdiction. In most building codes, resilient flooring products are exempt from fire-test-response requirements. See "Fire-Test-Response Characteristics" Article in the Evaluations for additional information.

A. Fire-Test-Response Characteristics: As determined by testing identical products according to ASTM E 648 or NFPA 253 by a qualified testing agency.

1. Critical Radiant Flux Classification: Class I, not less than 0.45 W/sq. cm.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

B. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. AB; American Biltrite.

2. Allstate Rubber Corp.

3. Armstrong World Industries, Inc.

4. Burke Mercer Flooring Products, Division of Burke Industries Inc.

5. Flexco.

6. Johnsonite; A Tarkett Company.

7. Mondo Rubber International, Inc.

8. Musson Rubber Company.

9. Nora Systems, Inc.

10. PRF USA, Inc.

11. R.C.A. Rubber Company (The).

12. Roppe Corporation, USA.

13. VPI, LLC, Floor Products Division.

14. <Insert manufacturer's name>.

C. Stair Treads: ASTM F 2169.

The literature of most stair-tread manufacturers does not indicate whether rubber products are thermoset or thermoplastic according to ASTM F 2169.

1. Type: [TS (rubber, vulcanized thermoset)] [or] [TP (rubber, thermoplastic)].

2. Class: [1 (smooth, flat)] [2 (pattern; embossed, grooved, or ribbed)].

Insert specific requirements in "Group" Subparagraph below to suit Project; e.g., number of abrasive strips per tread and contrasting strip locations.

3. Group: [1 (embedded abrasive strips)] [2 (with contrasting color for the visually impaired)].

4. Nosing Style: [Square, adjustable to cover angles between 60 and 90 degrees] [Square] [Round].

5. Nosing Height: [1-1/2 inches (38 mm)] [2 inches (51 mm)] [2-3/16 inches (56 mm)] <Insert dimension>.

6. Thickness: [1/4 inch (6 mm) and tapered to back edge] <Insert thickness>.

7. Size: Lengths and depths to fit each stair tread in [one piece] [one piece or, for treads exceeding maximum lengths manufactured, in equal-length units].

8. Integral Risers: Smooth, flat; in height that fully covers substrate.

Retain "Integral Risers" Subparagraph above or "Separate Risers" Paragraph below.

D. Separate Risers: Smooth, flat; in height that fully covers substrate; produced by same manufacturer as treads and recommended by manufacturer for installation with treads.

1. Style: [Coved toe, 7 inches (178 mm) high by length matching treads] [Toeless, by length matching treads].

2. Thickness: [0.125 inch (3.2 mm)] [Manufacturer's standard] <Insert thickness>.

E. Stringers: Height and length after cutting to fit risers and treads and to cover stair stringers; produced by same manufacturer as treads and recommended by manufacturer for installation with treads.

1. Thickness: [0.125 inch (3.2 mm)] [0.080 inch (2.0 mm)] [Manufacturer's standard] <Insert thickness>.

Retain "Landing Tile" Paragraph below if required. Not all manufacturers provide landing tiles to match treads.

F. Landing Tile: [Matching treads; produced by same manufacturer as treads and recommended by manufacturer for installation with treads] <Insert requirements>.

G. Locations: [Provide rubber stair accessories in areas indicated] <Insert requirements>.

Retain "Colors and Patterns" Paragraph below if colors and patterns are not indicated in a separate schedule.

H. Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

2.6 VINYL STAIR ACCESSORIES <Insert drawing designation>

Copy this article and re-edit for each set of vinyl stair-accessory product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Fire-Test-Response Characteristics" Paragraph below if required by authorities having jurisdiction. In most building codes, resilient flooring products are exempt from fire-test-response requirements. See "Fire-Test-Response Characteristics" Article in the Evaluations for additional information.

A. Fire-Test-Response Characteristics: As determined by testing identical products according to ASTM E 648 or NFPA 253 by a qualified testing agency.

1. Critical Radiant Flux Classification: Class I, not less than 0.45 W/sq. cm.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

B. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. Burke Mercer Flooring Products, Division of Burke Industries Inc.

2. Flexco.

3. Johnsonite; A Tarkett Company.

4. Musson Rubber Company.

5. R.C.A. Rubber Company (The).

6. Roppe Corporation, USA.

7. VPI, LLC, Floor Products Division.

8. <Insert manufacturer's name>.

C. Stair Treads: ASTM F 2169, Type TV (vinyl, thermoplastic).

1. Class: [1 (smooth, flat)] [2 (pattern; embossed, grooved, or ribbed)].

Insert specific requirements in "Group" Subparagraph below to suit Project; e.g., number of abrasive strips per tread and contrasting strip locations.

2. Group: [1 (embedded abrasive strips)] [2 (with contrasting color for the visually impaired)].

3. Nosing Style: [Square, adjustable to cover angles between 60 and 90 degrees] [Square] [Round].

4. Nosing Height: [1-1/2 inches (38 mm)] [2 inches (51 mm)] [2-3/16 inches (56 mm)] <Insert dimension>.

5. Thickness: [1/4 inch (6 mm) and tapered to back edge] <Insert thickness>.

6. Size: Lengths and depths to fit each stair tread in [one piece] [one piece or, for treads exceeding maximum lengths manufactured, in equal-length units].

7. Integral Risers: Smooth, flat; in height that fully covers substrate.

Retain "Integral Risers" Subparagraph above or "Separate Risers" Paragraph below.

D. Separate Risers: Smooth, flat; in height that fully covers substrate; produced by same manufacturer as treads and recommended by manufacturer for installation with treads.

1. Style: [Coved toe, 7 inches (178 mm) high by length matching treads] [Toeless, by length matching treads].

2. Thickness: [0.125 inch (3.2 mm)] [0.080 inch (2.0 mm)] [Manufacturer's standard] <Insert thickness>.

E. Stringers: Height and length after cutting to fit risers and treads and to cover stair stringers; produced by same manufacturer as treads and recommended by manufacturer for installation with treads.

1. Thickness: [0.125 inch (3.2 mm)] [0.080 inch (2.0 mm)] [Manufacturer's standard] <Insert thickness>.

Retain "Landing Tile" Paragraph below if required. Not all manufacturers provide landing tiles to match treads.

F. Landing Tile: [Matching treads; produced by same manufacturer as treads and recommended by manufacturer for installation with treads] <Insert requirements>.

G. Locations: [Provide vinyl stair accessories in areas indicated] <Insert requirements>.

Retain "Colors and Patterns" Paragraph below if colors and patterns are not indicated in a separate schedule.

H. Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

2.7 RUBBER MOLDING ACCESSORY <Insert drawing designation>

Copy this article and re-edit for each rubber molding accessory product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. Roppe Corporation, USA.

2. VPI, LLC, Floor Products Division.

3. <Insert manufacturer's name>.

B. Description: Rubber [stair-tread nosing] [cap for cove carpet] [cap for cove resilient flooring] [carpet bar for tackless installations] [carpet edge for glue-down applications] [nosing for carpet] [nosing for resilient flooring] [reducer strip for resilient flooring] [joiner for tile and carpet] [transition strips] <Insert description>.

C. Profile and Dimensions: [As indicated] <Insert profile and dimensions>.

D. Locations: [Provide rubber molding accessories in areas indicated] <Insert requirements>.

Retain "Colors and Patterns" Paragraph below if colors and patterns are not indicated in a separate schedule.

E. Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

2.8 VINYL MOLDING ACCESSORY <Insert drawing designation>

Copy this article and re-edit for each vinyl molding accessory product if more than one is required.

Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

1. Armstrong World Industries, Inc.

2. Burke Mercer Flooring Products, Division of Burke Industries Inc.

3. Flexco.

4. Johnsonite; A Tarkett Company.

5. Musson Rubber Company.

6. Roppe Corporation, USA.

7. <Insert manufacturer's name>.

B. Description: Vinyl [stair-tread nosing] [cap for cove carpet] [cap for cove resilient flooring] [carpet bar for tackless installations] [carpet edge for glue-down applications] [nosing for carpet] [nosing for resilient flooring] [reducer strip for resilient flooring] [joiner for tile and carpet] [transition strips] <Insert description>.

C. Profile and Dimensions: [As indicated] <Insert profile and dimensions>.

D. Locations: [Provide vinyl molding accessories in areas indicated] <Insert requirements>.

Retain "Colors and Patterns" Paragraph below if colors and patterns are not indicated in a separate schedule.

E. Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

2.9 INSTALLATION MATERIALS

A. Trowelable Leveling and Patching Compounds: Latex-modified, portland cement based or blended hydraulic-cement-based formulation provided or approved by resilient-product manufacturer for applications indicated.

B. Adhesives: Water-resistant type recommended by resilient-product manufacturer for resilient products and substrate conditions indicated.

Retain first subparagraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.1.

1. Adhesives shall have a VOC content of 50 g/L or less[ except that adhesive for rubber stair treads shall have a VOC content of 60 g/L or less].

Retain subparagraph below if required for LEED for Schools Credit IEQ 4.1 or Credit IEQ 4.3.

2. Adhesives shall comply with the testing and product requirements of the California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

Retain "Stair-Tread Nose Filler" Paragraph below if resilient stair treads are specified in this Section.

C. Stair-Tread Nose Filler: Two-part epoxy compound recommended by resilient stair-tread manufacturer to fill nosing substrates that do not conform to tread contours.

Retain "Floor Polish" Paragraph below for resilient stair accessories after verifying manufacturers' written instructions for floor-polish products.

D. Floor Polish: Provide protective, liquid floor-polish products recommended by resilient stair-tread manufacturer.

PART 3 -  EXECUTION

3.1 PREPARATION

Extensive surface preparation is required over substrates from which existing products have been removed. Requirements vary among manufacturers. Insert requirements to suit Project.

A. Prepare substrates according to manufacturer's written instructions to ensure adhesion of resilient products.

B. Concrete Substrates for Resilient Stair Accessories: Prepare horizontal surfaces according to ASTM F 710.

1. Verify that substrates are dry and free of curing compounds, sealers, and hardeners.

2. Remove substrate coatings and other substances that are incompatible with adhesives and that contain soap, wax, oil, or silicone, using mechanical methods recommended by manufacturer. Do not use solvents.

3. Alkalinity and Adhesion Testing: Perform tests recommended by manufacturer. Proceed with installation only after substrate alkalinity falls within range on pH scale recommended by manufacturer in writing, but not less than 5 or more than [9] [10] <Insert number> pH.

4. Moisture Testing: Proceed with installation only after substrates pass testing according to manufacturer's written recommendations, but not less stringent than the following:

a. Perform anhydrous calcium chloride test according to ASTM F 1869. Proceed with installation only after substrates have maximum moisture-vapor-emission rate of [3 lb of water/1000 sq. ft. (1.36 kg of water/92.9 sq. m)] <Insert rate> in 24 hours.

b. Perform relative humidity test using in situ probes according to ASTM F 2170. Proceed with installation only after substrates have maximum [75] <Insert number> percent relative humidity level.

C. Fill cracks, holes, and depressions in substrates with trowelable leveling and patching compound; remove bumps and ridges to produce a uniform and smooth substrate.

D. Do not install resilient products until they are the same temperature as the space where they are to be installed.

E. Immediately before installation, sweep and vacuum clean substrates to be covered by resilient products.

3.2 RESILIENT BASE INSTALLATION

A. Comply with manufacturer's written instructions for installing resilient base.

B. Apply resilient base to walls, columns, pilasters, casework and cabinets in toe spaces, and other permanent fixtures in rooms and areas where base is required.

C. Install resilient base in lengths as long as practical without gaps at seams and with tops of adjacent pieces aligned.

D. Tightly adhere resilient base to substrate throughout length of each piece, with base in continuous contact with horizontal and vertical substrates.

E. Do not stretch resilient base during installation.

Retain first paragraph below if required or revise to suit Project.

F. On masonry surfaces or other similar irregular substrates, fill voids along top edge of resilient base with manufacturer's recommended adhesive filler material.

Retain "Preformed Corners" Paragraph below for resilient base with preformed corners.

G. Preformed Corners: Install preformed corners before installing straight pieces.

Retain "Job-Formed Corners" Paragraph below for resilient base with job-formed corners.

H. Job-Formed Corners:

1. Outside Corners: Use straight pieces of maximum lengths possible and form with returns not less than [3 inches (76 mm)] <Insert dimension> in length.

a. Form without producing discoloration (whitening) at bends.

2. Inside Corners: Use straight pieces of maximum lengths possible and form with returns not less than [3 inches (76 mm)] <Insert dimension> in length.

a. [Miter] [Cope] [Miter or cope] corners to minimize open joints.

3.3 RESILIENT ACCESSORY INSTALLATION

A. Comply with manufacturer's written instructions for installing resilient accessories.

B. Resilient Stair Accessories:

1. Use stair-tread-nose filler to fill nosing substrates that do not conform to tread contours.

2. Tightly adhere to substrates throughout length of each piece.

Retain subparagraph below if applicable.

3. For treads installed as separate, equal-length units, install to produce a flush joint between units.

C. Resilient Molding Accessories: Butt to adjacent materials and tightly adhere to substrates throughout length of each piece. Install reducer strips at edges of floor covering that would otherwise be exposed.

3.4 CLEANING AND PROTECTION

A. Comply with manufacturer's written instructions for cleaning and protecting resilient products.

Before retaining "Floor Polish" Paragraph below, verify resilient stair-tread manufacturer's written instructions for floor polish. Floor polish is used with most vinyl products.

B. Floor Polish: Remove soil, visible adhesive, and surface blemishes from resilient stair treads before applying liquid floor polish.

Revise subparagraph below to suit products.

1. Apply [one] [two] [three] <Insert requirement> coat(s).

C. Cover resilient products subject to wear and foot traffic until Substantial Completion.

END OF SECTION 096513

	RESILIENT BASE AND ACCESSORIES
	096513 - 1



