Copyright 2011 AIA
MasterSpec Professional
02/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 097519 - STONE TRIM
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes interior stone trim.

1.2 ACTION SUBMITTALS

A. Product Data: For each[ variety of stone,] stone accessory, and manufactured product.

B. LEED Submittals:

First "Product Certificates for Credit MR 5" Subparagraph below applies to LEED-NC, LEED-CS, and LEED for Schools.

1. Product Certificates for Credit MR 5: For products and materials required to comply with requirements for regional materials, certificates indicating location of material manufacturer and point of extraction, harvest, or recovery for each raw material. Include statement indicating distance to Project, cost for each regional material, and fraction by weight that is considered regional.

"Product Certificates for Credit MR 5" Subparagraph below applies to LEED-CI. Retain option for Credit MR 5, Option 2.

2. Product Certificates for Credit MR 5: For products and materials required to comply with requirements for regionally manufactured[ and regionally extracted and manufactured] materials. Include statement indicating cost for each regionally manufactured material.

First subparagraph below applies to LEED-CI Credit MR 5, Option 1 and Option 2.

a. Include statement indicating location of manufacturer and distance to Project for each regionally manufactured material.

First subparagraph below applies to LEED-CI Credit MR 5, Option 2.

b. Include statement indicating location of manufacturer and point of extraction, harvest, or recovery for each raw material used in regionally extracted and manufactured materials. Indicate distance to Project and fraction by weight of each regionally manufactured material that is regionally extracted.

"Product Data for Credit IEQ 4.1" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for adhesives.

3. Product Data for Credit IEQ 4.1: For adhesives, documentation including printed statement of VOC content.

"Laboratory Test Reports for Credit IEQ 4.1" Subparagraph below applies to LEED for Schools.

4. Laboratory Test Reports for Credit IEQ 4.1: For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

C. Shop Drawings: Show fabrication and installation details for stone trim, including dimensions and profiles of stone units.

Generally retain "Samples" Paragraph below, especially for more variable varieties where Samples may serve to define acceptable range of colors, patterns, etc.

D. Samples:

SECTION 013300 "Submittal Procedures" requires that Samples for verification "show the full range of color and texture variations expected"; however, this requirement may be inadequate for stone varieties that exhibit variations in color and veining. First subparagraph below may provide better control.

1. For each stone type indicated. Include [two] [three] [four] [five] <Insert number> or more Samples in each set and show the full range of variations in appearance characteristics in completed Work.

2. For each color of [grout] [pointing mortar] required.

1.3 QUALITY ASSURANCE

A. Mockups: Build mockups to demonstrate aesthetic effects and to set quality standards for fabrication and installation.

Retain first subparagraph below for large-scale mockup or include in Section 014000 "Quality Requirements." Indicate portion of wall represented by mockup on Drawings or draw mockup as separate element.

1. Build mockup of typical wall area as shown on Drawings.

Retain first subparagraph below for limited mockups.

2. Build mockups for typical interior stone trim as shown on Drawings.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

3. Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 STONE, GENERAL

Delete third option in "Source Limitations for Stone" Paragraph below if not using other stone Sections or if other Sections do not require same stone varieties.

A. Source Limitations for Stone: Obtain[ each variety of] stone, [regardless of finish, ]from a single quarry[, whether specified in this Section or in another Section of the Specifications].

Retain "Varieties and Sources" Paragraph below and delete stone type articles below if specifying stone varieties and sources in Section 044200 "Exterior Stone Cladding." Specifying all stone types in one Section helps ensure uniformity of stone supplied under different Sections.

B. Varieties and Sources: Subject to compliance with requirements, provide stone of varieties and from sources complying with Section 044200 "Exterior Stone Cladding."

Retain first "Regional Materials" Paragraph below for LEED-NC, LEED-CS, or LEED for Schools Credit MR 5 and LEED-CI Credit MR 5, Option 2; before retaining, verify availability of materials that comply.

C. Regional Materials: Stone shall be extracted and fabricated within 500 miles (800 km) of Project site.

Retain "Regional Materials" Paragraph below for LEED-CI Credit MR 5, Option 1; before retaining, verify availability of materials that comply.

D. Regional Materials: Stone shall be fabricated within 500 miles (800 km) of Project site.

If retaining articles describing stone types here and only one type from each category is required, retain the generic name and use on Drawings instead of a stone-type designation; otherwise, copy and re-edit article retained for each type required, inserting a different stone-type designation each time.

2.2 GRANITE <Insert drawing designation>

Some varieties of granite that are suitable for interior use do not comply with ASTM C 615; verify that granite selected will comply before retaining "Material Standard" Paragraph below.

A. Material Standard: Comply with ASTM C 615.

B. Varieties and Sources: Subject to compliance with requirements, [provide the following] [provide one of the following] [available stone varieties that may be incorporated into the Work include, but are not limited to, the following]:

1. <Insert, in separate subparagraphs, names of varieties and producers, distributors, or importers>.

C. Finish: [Polished] [Honed] [Thermal] [As indicated] [Match Architect's sample] <Insert finish>.

Retain paragraph below for added quality control if required.

D. Match Architect's samples.

2.3 LIMESTONE <Insert drawing designation>

A. Material Standard: Comply with ASTM C 568.

Usually retain one of three options in "Classification" Subparagraph below. If naming varieties and sources, specifying a classification may be unnecessary but would provide additional quality control and a salient characteristic to use in determining if another variety or source is equivalent. First option generally applies to very porous limestone, such as shell limestone; second, to oolitic limestone; and third, to dolomitic limestone.

1. Classification: [I Low] [II Medium] [III High] Density.

B. Varieties and Sources: Subject to compliance with requirements, [provide the following] [provide one of the following] [available stone varieties that may be incorporated into the Work include, but are not limited to, the following]:

1. <Insert, in separate subparagraphs, names of varieties and producers, distributors, or importers>.

C. Finish: [Smooth] [Sand rubbed] [As indicated] [Match Architect's sample] <Insert finish>.

Retain paragraph below for added quality control if required.

D. Match Architect's samples.

2.4 MARBLE <Insert drawing designation>

Many marble varieties that are suitable for interior use do not comply with ASTM C 503; verify that marble selected will comply before retaining "Material Standard" Paragraph below.

A. Material Standard: Comply with ASTM C 503.

B. Varieties and Sources: Subject to compliance with requirements, [provide the following] [provide one of the following] [available stone varieties that may be incorporated into the Work include, but are not limited to, the following]:

1. <Insert, in separate subparagraphs, names of varieties and producers, distributors, or importers>.

C. Finish: [Polished] [Honed] [As indicated] [Match Architect's sample] <Insert finish>.

Retain paragraph below for added quality control if required.

D. Match Architect's samples.

2.5 SERPENTINE <Insert drawing designation>

If retaining this article, retain one of two options in "Material Standard" Paragraph below. Classification I Exterior requires lower absorption and higher strength; verify that serpentine selected complies with this classification before retaining.

A. Material Standard: Comply with ASTM C 1526, [Classification I Exterior] [Classification II Interior].

B. Varieties and Sources: Subject to compliance with requirements, [provide the following] [provide one of the following] [available stone varieties that may be incorporated into the Work include, but are not limited to, the following]:

1. <Insert, in separate subparagraphs, names of varieties and producers, distributors, or importers>.

C. Finish: [Polished] [Honed] [As indicated] [Match Architect's sample] <Insert finish>.

Retain paragraph below for added quality control if required.

D. Match Architect's samples.

2.6 TRAVERTINE <Insert drawing designation>

If retaining this article, retain one of two options in "Material Standard" Paragraph below. Classification I Exterior requires higher strength; verify that travertine selected complies with this classification before retaining.

A. Material Standard: Comply with ASTM C 1527, [Classification I Exterior] [Classification II Interior].

B. Varieties and Sources: Subject to compliance with requirements, [provide the following] [provide one of the following] [available stone varieties that may be incorporated into the Work include, but are not limited to, the following]:

1. <Insert, in separate subparagraphs, names of varieties and producers, distributors, or importers>.

Usually retain vein cut in "Cut" Paragraph below. Fleuri-cut travertine is often called "cross cut travertine." See Evaluations.

C. Cut: [Vein] [Fleuri].

Retain "Orientation of Veining" Subparagraph below if retaining vein cut.

1. Orientation of Veining: [Horizontal] [Vertical] [As indicated].

Retain "Filling" Paragraph below if filling is required.

D. Filling: Fill pores on faces of stone with cementitious filler of color [selected by Architect] [matching Architect's sample].

E. Finish: [Polished] [Honed] [As indicated] [Match Architect's sample] <Insert finish>.

Retain paragraph below for added quality control if required.

F. Match Architect's samples.

2.7 SETTING MATERIALS

Retain "Molding Plaster" Paragraph below for standard setting of stone in locations where moisture is not a concern. For wet areas, use mortar for setting spots.

A. Molding Plaster: ASTM C 59/C 59M.

Retain "Portland Cement," "Hydrated Lime," and "Aggregate" paragraphs below if using mortar for setting spots.

B. Portland Cement: ASTM C 150, Type I or Type II.

Retain "Low-Alkali Cement" Subparagraph below if recommended by stone source to limit staining.

1. Low-Alkali Cement: Not more than 0.60 percent total alkali when tested according to ASTM C 114.

C. Hydrated Lime: ASTM C 207, Type S.

D. Aggregate: ASTM C 144.

E. Water: Potable.

F. Adhesives, General: Use only adhesives formulated for stone and ceramic tile and recommended by their manufacturer for the application indicated.

Water-cleanable epoxy adhesive can be used for adhering stone trim to gypsum board partitions or to stone paneling. Water-cleanable epoxy adhesive is the preferred setting material for certain stone types that are prone to warping due to water absorption from various mortar types and from water-emulsion organic adhesives. (Although below is water cleanable, it contains very little water.) Retain second option in "Water-Cleanable Epoxy Adhesive" Paragraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.1. VOC content limit is that for ceramic tile adhesives. Retain third option if required for LEED for Schools Credit IEQ 4.

G. Water-Cleanable Epoxy Adhesive: ANSI A118.3[.][, with a VOC content of 65 g/L or less.][, that complies with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."]

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Boiardi Products; a QEP company.
b. Bonstone Materials Corporation.
c. Bostik, Inc.
d. C-Cure.
e. Custom Building Products.
f. Jamo Inc.
g. Laticrete International, Inc.
h. MAPEI Corporation.
i. Mer-Krete Systems; ParexLahabra, Inc.
j. Prospec; Bonsal American; a division of Oldcastle Architectural Products Group.
k. Summitville Tiles, Inc.
l. TEC, Specialty Construction Brands, Inc.; an H. B. Fuller company.
m. <Insert manufacturer's name>.

2.8 GROUT

Retain this article if using grouts. Grouts may be used for stone joints less than 3/16 inch (5 mm) wide; point wider joints with mortar.

A. Grout Colors: [Match stone] [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range].

Retain grout types from "Sand-Portland Cement Grout, "Standard Cement Grout," "Polymer-Modified Tile Grout," and "Water-Cleanable Epoxy Grout" paragraphs below. Coordinate with installation requirements. Insert colors for each grout type or indicate in a schedule.

Grout in "Sand-Portland Cement Grout" Paragraph below is field-mixed portland cement and fine-graded sand. Avoid sand-portland cement grout for stone with honed or polished finish; sand can scratch the stone when grout is spread and wiped.

B. Sand-Portland Cement Grout: ANSI A108.10, composed of white or gray cement and white or colored aggregate to produce required color.

C. Standard Cement Grout: ANSI A118.6, packaged.

D. Polymer-Modified Tile Grout: ANSI A118.7, packaged.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Bostik, Inc.
b. C-Cure.
c. Custom Building Products.
d. DAP Inc.
e. Jamo Inc.
f. Laticrete International, Inc.
g. MAPEI Corporation.
h. Mer-Krete Systems; ParexLahabra, Inc.
i. Prospec; Bonsal American; a division of Oldcastle Architectural Products Group.
j. Southern Grouts & Mortars, Inc.
k. Summitville Tiles, Inc.
l. TEC, Specialty Construction Brands, Inc.; an H. B. Fuller company.
m. <Insert manufacturer's name>.

Water-cleanable epoxy grout is more resistant to staining than portland cement-based grouts. Water-cleanable epoxy grout is also the preferred grout for certain stone types that are prone to warping due to water absorption from setting materials and grouts (water-cleanable epoxy grout contains very little water). See Evaluations.

E. Water-Cleanable Epoxy Grout: ANSI A118.3, packaged, chemical-resistant, water-cleanable, tile-setting and -grouting epoxy.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Boiardi Products; a QEP company.
b. Bostik, Inc.
c. C-Cure.
d. Custom Building Products.
e. Jamo Inc.
f. Laticrete International, Inc.
g. MAPEI Corporation.
h. Mer-Krete Systems; ParexLahabra, Inc.
i. Prospec; Bonsal American; a division of Oldcastle Architectural Products Group.
j. Summitville Tiles, Inc.
k. TEC, Specialty Construction Brands, Inc.; an H. B. Fuller company.
l. <Insert manufacturer's name>.

2.9 POINTING MORTAR MATERIALS

Use pointing mortar instead of grout for joints 3/16 inch (5 mm) and wider.

Retain "Portland Cement" Paragraph below for portland cement-lime mixes or for mortar mixed at Project site.

A. Portland Cement: ASTM C 150, Type I or Type II. Provide natural color or white cement as required to produce mortar color indicated.

Retain "Low-Alkali Cement" Subparagraph below if recommended by stone source to limit staining.

1. Low-Alkali Cement: Not more than 0.60 percent total alkali when tested according to ASTM C 114.

B. Hydrated Lime: ASTM C 207, Type S.

Retain "Mortar Pigments" Paragraph below for colored portland cement-lime mix or for pigments added to mortar mixed at Project site.

C. Mortar Pigments: Natural and synthetic iron oxides and chromium oxides, compounded for use in mortar mixes and complying with ASTM C 979. Pigments shall have a record of satisfactory performance in mortar.

Retain "Products" Subparagraph and list of manufacturers and products below to require specific products or a comparable product from other manufacturers.

1. Products: Subject to compliance with requirements, [provide the following] [provide one of the following] [available products that may be incorporated into the Work include, but are not limited to, the following]:

a. Davis Colors; True Tone Mortar Colors.

b. Lanxess Corporation; Bayferrox Iron Oxide Pigments.

c. Solomon Colors; SGS Mortar Colors.

d. <Insert manufacturer's name; product name or designation>.

D. Aggregate: ASTM C 144, except with 100 percent passing No. 16 (1.18-mm) sieve.

1. White Aggregates: Natural white sand or ground white stone.

2. Colored Aggregates: Natural-colored sand or ground marble, granite, or other durable stone; of color necessary to produce required mortar color.

E. Water: Potable.

2.10 STONE ANCHORS AND ATTACHMENTS

A. Fabricate anchors from stainless steel, ASTM A 240/A 240M or ASTM A 666, Type 304.

1. Fasteners for Stainless-Steel Anchors: Annealed stainless-steel bolts, nuts, and washers; ASTM F 593 (ASTM F 738M) for bolts and ASTM F 594 (ASTM F 836M) for nuts, Alloy Group 1 (A1).

B. Fabricate dowels from stainless steel, ASTM A 276, Type 304.

No ASTM standard exists for wire used as a tie or anchor, but building codes do define wire gage or diameter and noncorrosive requirements. See details in MIA's "Dimension Stone - Design Manual VII." Retain third option in "Wire Tiebacks" Paragraph below if staining might be a problem.

C. Wire Tiebacks: [No. 9 AWG copper or copper-alloy] [or] [0.120-inch- (3.0-mm-) diameter, stainless-steel] wire.

2.11 STONE ACCESSORIES

A. Temporary Setting Shims: Rigid plastic shims, nonstaining to stone, sized to suit joint thickness.

B. Cleaner: Stone cleaner specifically formulated for stone types, finishes, and applications indicated, as recommended by stone producer. Do not use cleaning compounds containing acids, caustics, harsh fillers, or abrasives.

Before retaining "Stone Sealer" Paragraph below, verify that it is required and recommended for stone type used.

C. Stone Sealer: Colorless, stain-resistant sealer that does not affect color or physical properties of stone surfaces, as recommended by stone producer for application indicated.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Bostik, Inc.
b. Custom Building Products.
c. Hillyard, Inc.
d. HMK Stone Care; ACI International.
e. Miracle Sealants Company.
f. Stone Care International.
g. Summitville Tiles, Inc.
h. <Insert manufacturer's name>.

2.12 STONE FABRICATION

A. Select stone for intended use to prevent fabricated units from containing cracks, seams, and starts that could impair structural integrity or function.

B. Cut stone to produce pieces of thickness, size, and shape indicated and to comply with fabrication and construction tolerances recommended by applicable stone association.

1. Where items are installed with adhesive or where stone edges are visible in the finished work, make items uniform in thickness and of identical thickness for each type of item; gage back of stone if necessary.

2. Dress joints straight and at right angle to face unless otherwise indicated.

C. Fabricate molded work to produce stone shapes with a uniform profile throughout entire unit length and with precisely formed arris slightly eased to prevent snipping, and matched at joints between units.

Retain subparagraphs below for polished or honed granite and marble. Delete if no moldings or if moldings are simple, flat shapes with chamfered or eased edges.

1. Produce moldings with machines having abrasive shaping wheels made to reverse contour of molding shape; do not sculpt moldings.

2. Miter moldings at corners, unless otherwise indicated, with edges of miters slightly eased at outside corners.

D. Flat Trim:

1. Nominal Thickness: [3/4 inch (20 mm)] [7/8 inch (22 mm)] [1-1/4 inches (32 mm)] [1-1/2 inches (40 mm)] unless otherwise indicated.

2. Edge Detail: [Straight, slightly eased at corners] [3/8-inch (10-mm) bevels] [3/4-inch (20-mm) radii] [3/8-inch (10-mm) radii] [As indicated].

3. Joints: [1/16-inch- (1.5-mm-) wide grouted joints] [1/8-inch- (3-mm-) wide grouted joints] [1/8-inch- (3-mm-) wide, sealant-filled joints] [Bonded joints, 1/32 inch (0.8 mm) or less in width].

E. Molded Trim:

1. Profile: Match [profiles indicated on Drawings] [existing].

2. Joints: [1/16-inch- (1.5-mm-) wide grouted joints] [1/8-inch- (3-mm-) wide grouted joints] [1/8-inch- (3-mm-) wide, sealant-filled joints] [Bonded joints, 1/32 inch (0.8 mm) or less in width].

2.13 MIXES

A. Spotting Plaster: Stiff mix of molding plaster and water.

B. Mortar, General: Comply with referenced standards and with manufacturers' written instructions for mix proportions, mixing equipment, mixer speeds, mixing containers, mixing time, and other procedures needed to produce mortar of uniform quality and with optimum performance characteristics.

1. Do not use admixtures, including pigments, air-entraining agents, accelerators, retarders, water-repellent agents, antifreeze compounds, or other admixtures unless otherwise indicated. Do not use calcium chloride.

2. Combine and thoroughly mix cementitious materials, water, and aggregates in a mechanical batch mixer unless otherwise indicated. Discard mortar when it has reached initial set.

C. Setting Mortar: Comply with ASTM C 270, Proportion Specification, [Type N] [Type O].

D. Pointing Mortar: Comply with ASTM C 270, Proportion Specification, [Type N] [Type O]. Provide pointing mortar mixed to match Architect's sample and complying with the following:

Retain "Pigmented Pointing Mortar" or "Colored-Aggregate Pointing Mortar" Subparagraph below. Ratio in first applies only to pigment types included in "Pointing Mortar Materials" Article. Other pigments, if inserted, may require different limitations.

1. Pigmented Pointing Mortar: Select and proportion pigments with other ingredients to produce color required. Do not exceed pigment-to-cement ratio of 1:10, by weight.

2. Colored-Aggregate Pointing Mortar: Produce color required by combining colored aggregates with portland cement of selected color.

E. Grout: Comply with mixing requirements of referenced ANSI standards and with manufacturer's written instructions.

PART 3 -  EXECUTION

3.1 SETTING STONE, GENERAL

A. Do necessary field cutting as stone is set. Use power saws with diamond blades to cut stone. Cut lines straight and true, with edges eased slightly to prevent snipping.

B. Contiguous Work: Provide reveals and openings as required to accommodate contiguous work.

C. Set stone to comply with requirements indicated. Install anchors, supports, fasteners, and other attachments indicated or necessary to secure stone in place. Shim and adjust anchors, supports, and accessories to set stone accurately in locations indicated, with edges and faces aligned according to established relationships and indicated tolerances.

D. Provide expansion, control, and pressure-relieving joints of widths and at locations indicated.

1. Sealing of expansion and other joints is specified in Section 079200 "Joint Sealants."

2. Keep expansion joints free of plaster, mortar, grout, and other rigid materials.

3.2 CONSTRUCTION TOLERANCES

Tolerances in "Variation from Plumb," "Variation from Level," "Variation in Joint Width," and "Variation in Plane between Adjacent Stone Units (Lipping)" paragraphs below are more stringent than those published by the Brick Industry Association for unit masonry and are generally appropriate for smooth-finished stone. Revise tolerances for natural-cleft, thermal, and similar finishes.

A. Variation from Plumb: For vertical lines and surfaces, do not exceed 1/8 inch in 96 inches (3 mm in 2400 mm), 1/4 inch (6 mm) maximum.

B. Variation from Level: For lintels, sills, chair rails, horizontal bands, horizontal grooves, and other conspicuous lines, do not exceed 1/8 inch in 10 feet (3 mm in 3 m), 1/4 inch in 20 feet (6 mm in 6 m), 3/8 inch (10 mm) maximum.

C. Variation in Joint Width: Do not vary from average joint width more than plus or minus 1/16 inch (1.5 mm) or one-fourth of nominal joint width, whichever is less.

D. Variation in Plane between Adjacent Stone Units (Lipping): Do not exceed 1/32-inch (0.8-mm) difference between planes of adjacent units.

3.3 INSTALLATION

Retain one of first two "Stone Trim" paragraphs below. Retain first for trim applied over interior stone facing. Retain second for trim set adjacent to or slightly overlapping rather than applied over stone facing.

A. Stone Trim: At locations with stone paneling, set units by adhering to stone paneling with water-cleanable epoxy adhesive. Hold adhesive back from exposed edges of joints to allow for grouting.

B. Stone Trim: At locations with stone paneling, set units firmly against setting spots. Locate setting spots at anchors and spaced not more than 18 inches (450 mm) apart unless otherwise indicated. Provide no fewer than two anchors per piece for stone trim up to 48 inches (1200 mm) in length, plus one additional anchor for each additional 24 inches (600 mm) of length.

Retain one of two "Stone Trim" paragraphs below for stone trim on gypsum board partitions and column covers. For best work, retain first paragraph and use plywood strips, slightly narrower than stone units, in place of gypsum board.

C. Stone Trim: At locations without stone paneling, adhere units to plywood backing with full spread of water-cleanable epoxy adhesive. Hold adhesive back from exposed edges of joints to allow for grouting.

Retain "Stone Trim" Paragraph below for cost saving and for fire-rated partitions; reinforce gypsum board behind base and trim with metal bridging or fire-retardant wood blocking.

D. Stone Trim: At locations without stone paneling, adhere units to gypsum board with full spread of water-cleanable epoxy adhesive. Hold adhesive back from exposed edges of joints to allow for grouting.

Delete paragraph below if all joints are filled with sealant.

E. [Grout] [Point] joints after setting stone.

3.4 GROUTING JOINTS

A. Grout stone to comply with ANSI A108.10.

Retain two subparagraphs below if retaining both sanded and unsanded grouts in "Grout" Article. Do not use sanded grout for joints with polished stone, especially softer stones such as marble; sand can scratch the stone when grout is spread and wiped.

1. Use sanded grout mixture for joints wider than 1/8 inch (3 mm).

2. Use unsanded grout mixture for joints 1/8 inch (3 mm) and narrower.

B. Remove temporary shims before grouting.

C. Tool joints uniformly and smoothly with plastic tool.

3.5 POINTING JOINTS WITH MORTAR

A. Prepare stone-joint surfaces for pointing with mortar by removing temporary shims, dust, and mortar particles. Where setting spots occur at joints, rake out excess setting mortar or plaster to a depth of not less than 1/2 inch (13 mm).

B. Point stone joints by placing pointing mortar in layers of not more than 3/8 inch (10 mm). Compact each layer thoroughly and allow it to become thumbprint hard before applying next layer. Apply mortar first to areas where depths are greater than surrounding areas until a uniform depth is formed.

C. Tool joints when pointing mortar is thumbprint hard. Use a round jointer having a diameter 1/8 inch (3 mm) larger than width of joint.

3.6 ADJUSTING AND CLEANING

A. In-Progress Cleaning: Clean interior stone trim as work progresses. Remove adhesive, grout, mortar, and sealant smears immediately.

Revise the words "six days" in first paragraph below to minimum needed for curing mortar and sealants as applicable.

B. Clean interior stone trim no fewer than six days after completion of grouting and pointing, using clean water and soft rags or stiff-bristle fiber brushes. Do not use wire brushes, acid-type cleaning agents, cleaning compounds with caustic or harsh fillers, or other materials or methods that could damage stone.

C. Sealer Application: Apply stone sealer to comply with stone producer's and sealer manufacturer's written instructions and recommendations.

END OF SECTION 097519

	STONE TRIM
	097519 - 1



