Copyright 2011 AIA
MasterSpec Professional
11/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 101300 - DIRECTORIES
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

Revise list below to suit Project.

1. [Nonilluminated] [Illuminated], message-strip directories.

2. [Nonilluminated] [Illuminated], changeable-letter directories.

1.2 PERFORMANCE REQUIREMENTS

A. Structural Performance: Directories shall withstand the effects of gravity loads and the following loads and stresses within limits and under conditions indicated according to [ASCE/SEI 7] <Insert requirement>.

Retain subparagraph below if design loads are not indicated on Drawings.

1. Seismic Loads: <Insert loads>.

1.3 ACTION SUBMITTALS

A. Product Data: For each type of product indicated.

B. Shop Drawings: For directories. Include plans, elevations, sections, details, and attachments to other work.

1. Wiring Diagrams: For power, signal, and control wiring.

C. Samples: For each exposed product and for each color and texture specified.

D. Message-Strip Schedule: Layout of each directory and each message strip showing letter size, font, spacing, indents, text copy, and graphics.

Paragraph below is defined in Section 013300 "Submittal Procedures" as a "Delegated-Design Submittal." Retain if Work of this Section is required to withstand specific design loads and design responsibilities have been delegated to Contractor or if structural data are required as another way to verify compliance with performance requirements. Professional engineer qualifications are specified in Section 014000 "Quality Requirements."

E. Delegated-Design Submittal: For directories indicated to comply with performance requirements and design criteria, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance Data: For illuminated directories to include in maintenance manuals.

1.5 MAINTENANCE MATERIAL SUBMITTALS

A. Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise subparagraph below to suit Project. Replace percentage with a specific number of message strips if required.

1. Message Strips: Full-size, blank strips equal to [10] <Insert number> percent of amount installed for each size indicated, but no fewer than [20] <Insert number> strips.

1.6 QUALITY ASSURANCE

Retain first paragraph below if components of directories are flammable and if testing is required by authorities having jurisdiction.

A. Surface-Burning Characteristics: As determined by testing identical products according to ASTM E 84 by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

1. Flame-Spread Index: [25] <Insert value> or less.

2. Smoke-Developed Index: [50] [450] <Insert value> or less.

Retain first paragraph below if including illuminated directories.

B. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

C. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. Aluminum: Alloy and temper recommended by aluminum producer and manufacturer for type of use and finish indicated; ASTM B 209 (ASTM B 209M) for sheet; ASTM B 221 (ASTM B 221M), Alloy 6063 for extruded shapes.

B. Stainless-Steel Sheet: ASTM A 666, Type 304.

C. Clear Float Glass: ASTM C 1036, Type I, Class 1, Quality q3, and 6 mm thick unless otherwise indicated.

D. Gray-Tinted Float Glass: ASTM C 1036; Type I (transparent glass, flat); Class 2 (tinted, heat absorbing, and light reducing); Quality q3 (glazing select); 6-mm-thick, gray-tinted monolithic glass with a maximum luminous transmittance of 14 percent.

E. Bronze-Tinted Float Glass: ASTM C 1036; Type I (transparent glass, flat); Class 2 (tinted, heat absorbing, and light reducing); Quality q3 (glazing select); 6-mm-thick, bronze-tinted monolithic glass with a maximum luminous transmittance of 44 percent.

Insert laminated glass for rear-illuminated, message-strip directories in first paragraph below if needed.

F. Clear Tempered Glass: ASTM C 1048, Kind FT, Condition A, Type I, Class 1 (clear), Quality q3, with exposed edges seamed before tempering, and 6 mm thick unless otherwise indicated.

G. Tinted Tempered Glass: ASTM C 1048, Kind FT, Condition A, Type I, Class 2 (tinted), tint as indicated, Quality q3, with exposed edges seamed before tempering, and 6 mm thick unless otherwise indicated.

H. Clear Acrylic Sheet: ASTM D 4802, Category A-1 (cell-cast sheet), with Finish 1 (smooth or polished), and 6 mm thick unless otherwise indicated; colorless sheet with visible light transmittance of 92 percent measured per ASTM D 1003.

I. Bronze-Tinted Acrylic Sheet: ASTM D 4802, Category A-1 (cell-cast sheet), with Finish 1 (smooth or polished), and 6 mm thick unless otherwise indicated.

J. Translucent Acrylic Sheet: ASTM D 4802, Category A-1 (cell-cast sheet), with Finish 1 (smooth or polished), and 6 mm thick unless otherwise indicated; white colored sheet of density required to produce uniform brightness and minimum halation effects.

K. Opaque Acrylic Sheet: ASTM D 4802, Category A-1 (cell-cast sheet), with Finish 1 (smooth or polished), and 6 mm thick unless otherwise indicated; colors as indicated.

L. Fasteners: Provide screws, bolts, and other fastening devices made from same material as items being fastened. Provide types, sizes, and lengths to suit installation conditions. Use security fasteners where exposed to view.

2.2 MESSAGE-STRIP DIRECTORIES

Copy paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required. Use these designations on Drawings to identify each product.

A. Nonilluminated, Message-Strip Directory <Insert drawing designation>: Factory-fabricated unit consisting of changeable message strips held in place by retainer frame enclosed in manufacturer's standard [1-1/2-to-2-inch- (38-to-50-mm-)] <Insert dimension> deep perimeter frame; with aluminum-sheet rear cover panel and glazed cover.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. See Section 016000 "Product Requirements."

1. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Allenite Signs; division of Allen Marking Products, Inc.

b. APCO Graphics, Inc.

c. ASI-Modulex.

d. Best Sign Systems, Inc.

e. Desk & Door Nameplate Co.

f. Nelson-Harkins Industries.

g. Poblocki Sign Company.

h. Tablet & Ticket Co. (The).

i. Visiontron Corp.

j. Vomar Products, Inc.

k. <Insert manufacturer's name>.

Retain one of first four subparagraphs below or insert another design.

2. Profiled Frame: Removable, clear acrylic sheet held in place by perimeter frame; with [4-inch- (100-mm-)] <Insert dimension> high, molded-opaque-acrylic or molded-fiberglass profiled frame at top and bottom.

a. Profiled-Frame Shape: [Square] [Radiused] [Beveled] <Insert shape>.

b. Profiled-Frame Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert color>.

c. Header Panel: Provide copy on top profiled frame that complies with requirements indicated on [Drawings] [artwork supplied on electronic media by Architect] for size, style, spacing, content, height, location, material, and colors of graphics.

d. Divider Color: [Same as message strips] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert color>.

3. Reveal Frame and Nonsectional Cover: Glazing held in cover frame mounted on full-length, concealed continuous hinge to form reveal between outer edge of cover frame and inner edge of perimeter frame. Provide nonsectional, one-piece cover for access to message strips[ and header panel], equipped with cylinder lock.

a. Perimeter Frame: [Extruded aluminum] [Brass] [Bronze].

b. Perimeter Frame Profile: [Square] [Quarter round] [Beveled] <Insert shape>.

c. Perimeter Frame Corners: [Square] [Radius].

d. Cover Frame: Same material and finish as perimeter frame.

1) Hinge Location: [Side] [Top].

e. Glazing: [Clear float glass] [Clear tempered glass] [Bronze-tinted tempered glass] [Gray-tinted tempered glass] [Clear acrylic sheet].

f. Header Panel: Provide copy that complies with requirements indicated on [Drawings] [artwork supplied on electronic media by Architect] for size, style, spacing, content, height, location, material, and colors of graphics.

g. Divider Color: [Same as message strips] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert color>.

4. Sectional Frame and Cover: Individual, glazed section covers, each in its own frame, containing single column of message strips. Provide perimeter frame for entire unit[ and separate divider frame between header panel and message-strip columns]. Mount section covers with concealed pivot hinges or continuous hinges and equip with concealed lock.

a. Perimeter Frame: [Extruded aluminum] [Brass] [Bronze].

b. Perimeter Frame Profile: [Square] [Quarter round] [Beveled].

c. Perimeter Frame Corners: [Square] [Radius].

d. Cover Frame: Same material and finish as perimeter frame.

e. Glazing: [Clear float glass] [Clear tempered glass] [Bronze-tinted tempered glass] [Gray-tinted tempered glass] [Clear acrylic sheet].

f. Header Panel: Provide copy that complies with requirements indicated on [Drawings] [artwork supplied on electronic media by Architect] for size, style, spacing, content, height, location, material, and colors of graphics.

5. Modular Frame and Cover: Individual modular units containing single column of message strips. Provide each modular unit with its own perimeter frame and lift-off glazed cover that snaps into place, and equipped with concealed lock.

a. Perimeter Frame: [Extruded aluminum] [Brass] [Bronze].

b. Perimeter Frame Profile: [Square] [Quarter round] [Beveled] <Insert shape>.

c. Perimeter Frame Corners: [Square] [Radius].

d. Cover Frame: Same material and finish as perimeter frame.

e. Glazing: [Clear float glass] [Clear tempered glass] [Bronze-tinted tempered glass] [Gray-tinted tempered glass] [Clear acrylic sheet].

f. Header Panel: [At top of each message-strip column] [Separate modular unit for entire directory].

1) Provide copy that complies with requirements indicated on [Drawings] [artwork supplied on electronic media by Architect] for size, style, spacing, content, height, location, material, and colors of graphics.

6. Aluminum Finish: [Clear anodic] [Color anodic] [Baked enamel] [Powder coat].

a. Color and Gloss: [Light bronze] [Medium bronze] [Dark bronze] [Black] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert color and gloss>.

7. Changeable Message Strips: [Applied-copy] [Engraved] [Magnetic] [Film-insert] type; [7-inch- (175-mm-)] [14-inch- (350-mm-)] <Insert dimension> long strips.

Retain first six subparagraphs below with any of four types of message strips in subparagraph above.

a. Message-Strip Color: [Black] [Gray] [Brown] [White] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

b. Message-Strip Height: [1/2 inch (13 mm)] [1 inch (25 mm)] [As indicated on Drawings] <Insert dimension>.

c. Letter Height: [3/16 inch (5 mm)] [1/4 inch (6 mm)] [3/8 inch (9.5 mm)] <Insert dimension>.

d. Letter Style: [Helvetica Medium] [Clarendon Medium] [Optima Bold] [As selected by Architect] <Insert style>.

e. Letter Color: [White] [Black] [Gray] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

f. Letter Case: [All capitals] [Initial capitals] [Capitals and lowercase] [As typed].

8. Graphics Panel: Screen-printed graphics or vinyl graphics laminated to opaque acrylic sheet; held in place by interchangeable, interlocking plastic carrier.

B. Rear-Illuminated, Message-Strip Directory <Insert drawing designation>: Factory-fabricated unit consisting of changeable message strips held in place by retainer frame enclosed in manufacturer's standard [4-to-6-inch- (100-to-150-mm-)] <Insert dimension> deep perimeter frame; with glazed cover, aluminum-sheet rear cover panel, and concealed illumination system.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. See Section 016000 "Product Requirements."

1. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. APCO Graphics, Inc.

b. ASI-Modulex, Inc.

c. Nelson-Harkins Industries.

d. Poblocki Sign Company.

e. Tablet & Ticket Co. (The).

f. Vomar Products, Inc.

g. <Insert manufacturer's name>.

Retain one of first two subparagraphs below or insert another design.

2. Reveal Frame and Nonsectional Cover: Glazing held in cover frame mounted on full-length, concealed continuous hinge to form reveal between outer edge of cover frame and inner edge of perimeter frame. Provide nonsectional, one-piece cover for access to message strips[ and header panel], equipped with concealed lock.

a. Perimeter Frame: [Extruded aluminum] [Brass] [Bronze] [Stainless steel].

b. Perimeter Frame Profile: [Square] [Quarter round] [Beveled] <Insert shape>.

c. Perimeter Frame Corners: [Square] [Radius].

d. Cover Frame: Same material and finish as perimeter frame.

1) Hinge Location: [Side] [Top].

e. Glazing: [Gray-tinted float glass] [Gray-tinted tempered glass] [Bronze-tinted acrylic sheet].

f. Header Panel: Provide copy that complies with requirements indicated on [Drawings] [artwork supplied on electronic media by Architect] for size, style, spacing, content, height, location, material, and colors of graphics.

g. Divider Color: Same as message strips.

3. Floating-Glass Cover: Unframed glazing mounted on full-length, concealed continuous hinge [to form reveal between outer edge of glazing and inner edge] [so glazing conceals front] of perimeter frame. Provide nonsectional, one-piece cover for access to message strips[ and header panel], equipped with concealed lock.

a. Perimeter Frame: [Extruded aluminum] [Brass] [Bronze] [Stainless steel] with square profile and corners.

b. Glazing: [Gray-tinted float glass] [Gray-tinted tempered glass] [Bronze-tinted acrylic sheet].

c. Hinge Location: [Side] [Top].

d. Header Panel: Provide copy that complies with requirements indicated on [Drawings] [artwork supplied on electronic media by Architect] for size, style, spacing, content, height, location, material, and colors of graphics.

e. Divider Color: Same as message strips.

4. Aluminum Finish: [Clear anodic] [Color anodic] [Plated] <Insert finish>.

Retain one color option in first subparagraph below with color anodic finish in subparagraph above. First five options below may vary in color range and availability among manufacturers.

a. Color and Gloss: [Light bronze] [Medium bronze] [Dark bronze] [Black] [Gold] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

Retain first subparagraph below if retaining "Plated" option in "Aluminum Finish" Subparagraph above.

b. Plating: [Satin bronze] [Mirror bronze] [Bright chrome] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert finish>.

5. Stainless-Steel Finish: [No. 4] [No. 8].

6. Changeable Message Strips: [Engraved] [Film-insert] type; [7-inch- (175-mm-)] [14-inch- (350-mm-)] <Insert dimension> long strips; copy size, style, and case as indicated on Drawings.

a. Message-Strip Height: [3/8 inch (9.5 mm)] [1/2 inch (13 mm)] [5/8 inch (16 mm)] [3/4 inch (19 mm)] [1 inch (25 mm)] [As indicated on Drawings] <Insert dimension>.

Retain first two subparagraphs below if required. Indicate graphics and header panel copy content on Drawings.

7. Graphics Panel: Same material and color as changeable message strips.

8. Header Panel: Same material and color as changeable message strips.

Revise subparagraph below to suit Project.

9. Rear-Illumination System: Provide removable and accessible fluorescent-strip fixture system with reflective interior surfaces for uniform illumination of message strips[ and header panel] with minimum halation and without light leaks. Include lamps and internal wiring with single concealed electrical connection to building system. Coordinate electrical characteristics with power supply provided.

a. Ballasts: Low-temperature, high-power-factor, low-energy, fluorescent lamp ballasts that comply with Certified Ballast Manufacturers Association standards and carry its label.

2.3 CHANGEABLE-LETTER DIRECTORIES

Copy paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required. Use these designations on Drawings to identify each product.

Revise paragraph below if lift-off back, instead of accessible front, is required.

A. Enclosed-Face, Changeable-Letter Directory <Insert drawing designation>: Factory-fabricated unit consisting of manufacturer's standard, 2-inch- (50-mm-) deep perimeter frame with [fixed] [removable] letterboard on back inside surface and with [glazed doors] [lift-off, glazed cover].

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. See Section 016000 "Product Requirements."

1. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. A-1 Visual Systems.

b. AARCO Products, Inc.

c. ADP Lemco, Inc.

d. APCO Graphics, Inc.

e. ASI-Modulex.

f. Best-Rite Manufacturing.

g. Best Sign Systems, Inc.

h. Claridge Products and Equipment, Inc.

i. Ghent Manufacturing, Inc.

j. Marsh Industries, Inc.; Visual Products Group.

k. Nelson-Harkins Industries.

l. Peter Pepper Products, Inc.

m. Poblocki Sign Company.

n. Tablet & Ticket Co. (The).

o. Visiontron Corp.

p. Vomar Products, Inc.

q. <Insert manufacturer's name.>

Retain one of two perimeter frame subparagraphs below.

2. Aluminum Perimeter Frame: Extruded aluminum.

a. Perimeter Frame Profile: [Square] [Half round] [Quarter round] <Insert shape>.

b. Perimeter Frame Corners: [Square] [Radius].

c. Finish: [Clear anodic] [Color anodic] [Baked enamel] [Powder coat].

d. Color and Gloss: [Light bronze] [Medium bronze] [Dark bronze] [Black] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

3. Wood Perimeter Frame: [Red oak] [Walnut] with [natural lacquered] [oiled] [stained] finish.

a. Perimeter Frame Profile: [Square] [Half round] [Quarter round] <Insert shape>.

b. Perimeter Frame Corners: [Square] [Radius].

Retain one of first three subparagraphs below. Sliding doors are typical for directories 36 inches (914 mm) wide or wider; one hinged door is typical for narrower directories.

4. Glazed, Sliding Doors: Unframed, clear-tempered-glass doors supported on nylon or ball-bearing rollers in extruded-aluminum top and bottom track, with plastic top guide and rubber bumpers. Equip each door with ground finger pull and adjustable cylinder lock with two keys.

a. Number of Doors: [Two] [Three] [As indicated on Drawings] <Insert number>.

5. Glazed, Hinged Doors: Clear [acrylic sheet] [tempered glass] set in door frame equipped with full-height continuous hinge and cylinder lock with two keys.

a. Door Frame: Same material and finish as perimeter frame.

b. Number of Doors: [One] [Two] [Three] [As indicated on Drawings] <Insert number>.

6. Lift-off Cover: Clear [acrylic sheet] [tempered glass] set in frame that snaps into place. Fabricate frame from same material and finish as perimeter frame.

Retain first subparagraph below if required.

7. Header Panel: [Nonilluminated; with opaque,] [Illuminated; with translucent,] acrylic sheet panel set within overall perimeter frame; with matching frame that separates header panel from letterboard.

a. Graphic Content and Style: Provide header panel copy that complies with requirements indicated on Drawings.

b. Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

Retain first subparagraph below if required. Illumination systems are available for directory cabinets whether or not they have header panels.

8. Illumination System: Concealed top-lighting system consisting of fluorescent-strip fixtures. Include lamps and internal wiring with single, concealed electrical connection to building system. Coordinate electrical characteristics with power supply provided.

a. Ballasts: Low-temperature, high-power-factor, low-energy, fluorescent lamp ballasts that comply with Certified Ballast Manufacturers Association standards and carry its label.

First option in first subparagraph below is most common. Revise if nongrooved cork surface or steel backing for magnetic letters is required.

9. Letterboard: Manufacturer's standard [felt] [vinyl] [rubber]-covered panel material, with grooves spaced at 1/4 inch (6 mm) o.c. to receive changeable letters.

a. Color: [Black] [Red] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

10. Letters: Molded plastic with tabs for engaging grooves in letterboard. Provide manufacturer's standard assortment of not less than [300] <Insert number> characters for each size, style, color, and case indicated on Drawings; include letters, numbers, and characters. Package letters in compartmentalized carrying box.

2.4 FABRICATION

A. Message-Strip Directories: Provide message strips for each carrier in entire directory.

B. Provide hold-open arms for doors of top-hinged directories.

2.5 ALUMINUM FINISHES

Retain finishes in this article to suit Project. If retaining more than one, indicate location of each on Drawings or by inserts.

Retain one of two options in first paragraph below. Class II finish is standard with many manufacturers; Class I finish is heavy anodized. Verify availability with manufacturers selected.

A. Clear Anodic Finish: AAMA 611, [AA-M12C22A41, Class I, 0.018 mm] [AA-M12C22A31, Class II, 0.010 mm] or thicker.

Retain one of two options in first paragraph below. Verify availability with manufacturers selected.

B. Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44, Class I, 0.018 mm] [AA-M12C22A32/A34, Class II, 0.010 mm] or thicker.

Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

C. Baked-Enamel or Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General: Install directories in locations and at mounting heights indicated on Drawings. Keep perimeter lines straight, level, and plumb. Provide grounds, clips, backing materials, adhesives, brackets, anchors, trim, and accessories necessary for complete installation.

B. Recessed Directories: Attach directories to wall framing with fasteners at not more than 16 inches (400 mm) o.c. Attach aluminum trim over edges of recessed directories and conceal grounds and clips.

C. Surface-Mounted Directories: Attach directories to wall surfaces with concealed clips, hangers, or grounds fastened at not more than 16 inches (400 mm) o.c. Secure both top and bottom of directories to walls.

D. Comply with requirements specified elsewhere for connecting illuminated directories. After installation is complete, install new fluorescent lamps.

E. Adjust directory doors to operate smoothly without warp or bind and so that contact points meet accurately. Lubricate operating hardware as recommended by manufacturer.

F. Touch up factory-applied finishes to restore damaged or soiled areas.

END OF SECTION 101300

	DIRECTORIES
	101300 - 1



