Copyright 2010 AIA
MasterSpec Professional
02/10

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102116 - SHOWER AND DRESSING COMPARTMENTS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Shower compartments fabricated from [steel] [stainless steel] [solid phenolic] [solid polymer].

2. Dressing compartments fabricated from [steel] [stainless steel] [solid phenolic] [solid polymer].

3. Shower receptors.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product indicated.

B. LEED Submittals:

"Laboratory Test Reports for Credit IEQ 4" Subparagraph below applies to LEED for Schools.

1. Laboratory Test Reports for Credit IEQ 4: For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

C. Shop Drawings: For shower and dressing compartments. Include plans, elevations, sections, details, and attachments to other work.

1.3 INFORMATIONAL SUBMITTALS

Paragraph below assumes manufacturer's standard-size Samples are acceptable. Revise to suit Project.

A. Samples: For each exposed product and for each color and texture specified.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.5 QUALITY ASSURANCE

Retain first paragraph below if Owner or authorities having jurisdiction require that shower and dressing compartments made of combustible materials comply with surface-burning requirements. Metal compartments are inherently noncombustible.

A. Surface-Burning Characteristics: As determined by testing identical products according to ASTM E 84, or another standard acceptable to authorities having jurisdiction, by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

1. Flame-Spread Index: [25] [75] [200] or less.

2. Smoke-Developed Index: 450 or less.

B. Regulatory Requirements: Comply with applicable provisions in [the U.S. Architectural & Transportation Barriers Compliance Board's ADA-ABA Accessibility Guidelines] [and] [ICC/ANSI A117.1] for shower and dressing compartments designated as accessible.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. Steel Sheet: ASTM A 653/A 653M, either hot-dip galvanized or galvannealed; mill phosphatized and selected for smoothness.

Retain "Adhesives" Paragraph below if required for LEED for Schools Credit IEQ 4.

B. Adhesives: Manufacturer's standard product that complies with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

2.2 STEEL COMPARTMENTS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. Retain one of first two paragraphs and list of manufacturers below. See Section 016000 "Product Requirements."

A. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. All American Metal Corp.

2. General Partitions Mfg. Corp.

3. Global Steel Products Corp.

4. Metpar Corp.

5. Rockville Partitions Incorporated.

6. Sanymetal; a Crane Plumbing company.

7. <Insert manufacturer's name>.

C. Configuration: [Shower compartment] [Shower and dressing compartments] [Shower compartment with two dressing compartments] [As shown on Drawings].

D. Enclosure Style: [Overhead braced] [Floor and ceiling anchored] <Insert style>.

E. Panel and Pilaster Construction: Seamless metal facing sheets, pressure laminated to core material; with continuous, interlocking molding strip or lapped-and-formed edge closures and with corners secured by welding or clips and exposed welds ground smooth. Exposed surfaces shall be free of pitting, seam marks, roller marks, stains, discolorations, telegraphing of core material, or other imperfections.

1. Core Material: Manufacturer's standard, sound-deadening honeycomb of resin-impregnated kraft paper in thickness required to provide finished thickness of 1 inch (25 mm) for panels and 1-1/4 inches (32 mm) for pilasters.

2. Grab-Bar Reinforcement: Provide concealed internal reinforcement for grab bars mounted on compartments.

3. Tapping Reinforcement: Provide concealed reinforcement for tapping (threading) at locations where machine screws are used for attaching items to compartments.

Retain first paragraph below if doors are required; delete if curtains are required.

F. Door Construction: Match panels; 1-inch (25-mm) finished thickness.

G. Facing Sheets and Closures: Hot-dip galvanized-steel sheet in manufacturer's standard, nominal base-metal (uncoated) thicknesses.

H. Pilaster [Shoes] [and] [Sleeves (Caps)]: Formed from stainless-steel sheet, not less than 3 inches (76 mm) high, finished to match hardware.

I. Brackets (Fittings):

Retain first subparagraph below for shower compartments; verify availability with manufacturers if inserting material other than aluminum. Retain first, second, or third subparagraphs for dressing compartments; if retaining more than one, indicate locations of each type of fitting on Drawings or by inserts.

1. Full-Height (Continuous) Type: Manufacturer's standard design; [clear-anodized aluminum] <Insert material>.

2. Stirrup Type: Ear or U-brackets; [clear-anodized aluminum] [stainless steel] [chrome-plated brass].

3. Dressing-Compartment Brackets: Match toilet-compartment brackets specified in Section 102113 "Toilet Compartments."

J. Steel-Sheet Finish: Manufacturer's standard baked-on finish with [one color] [two colors] in each room.

1. Color: [As indicated by manufacturer's designations] [As selected by Architect from manufacturer's full range] [Match steel toilet compartments specified in Section 102113 "Toilet Compartments"] <Insert color>.

2.3 STAINLESS-STEEL COMPARTMENTS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. Retain one of first two paragraphs and list of manufacturers below. See Section 016000 "Product Requirements."

A. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. All American Metal Corp.

2. General Partitions Mfg. Corp.

3. Global Steel Products Corp.

4. Metpar Corp.

5. Rockville Partitions Incorporated.

6. Sanymetal; a Crane Plumbing company.

7. <Insert manufacturer's name>.

C. Configuration: [Shower compartment] [Shower and dressing compartments] [Shower compartment with two dressing compartments] [As shown on Drawings].

D. Enclosure Style: [Overhead braced] [Floor and ceiling anchored] <Insert style>.

E. Panel and Pilaster Construction: Seamless metal facing sheets, pressure laminated to core material; with continuous, interlocking molding strip or lapped-and-formed edge closures and with corners secured by welding or clips and exposed welds ground smooth. Exposed surfaces shall be free of pitting, seam marks, roller marks, stains, discolorations, telegraphing of core material, or other imperfections.

1. Core Material: Manufacturer's standard, sound-deadening honeycomb of resin-impregnated kraft paper in thickness required to provide finished thickness of 1 inch (25 mm) for panels and 1-1/4 inches (32 mm) for pilasters.

2. Grab-Bar Reinforcement: Provide concealed internal reinforcement for grab bars mounted on compartments.

3. Tapping Reinforcement: Provide concealed reinforcement for tapping (threading) at locations where machine screws are used for attaching items to compartments.

Retain first paragraph below if doors are required; delete if curtains are required.

F. Door Construction: Match panels; 1-inch (25-mm) finished thickness.

G. Facing Sheets and Closures: Stainless-steel sheet in manufacturer's standard, nominal thicknesses.

H. Pilaster [Shoes] [and] [Sleeves (Caps)]: Formed from stainless-steel sheet, not less than 3 inches (76 mm) high, finished to match hardware.

I. Brackets (Fittings):

Retain first subparagraph below for shower compartments; verify availability with manufacturers if inserting material other than aluminum. Retain first, second, or third subparagraphs for dressing compartments; if retaining more than one, indicate locations of each type of fitting on Drawings or by inserts.

1. Full-Height (Continuous) Type: Manufacturer's standard design; [clear-anodized aluminum] <Insert material>.

2. Stirrup Type: Ear or U-brackets; [clear-anodized aluminum] [stainless steel] [chrome-plated brass].

3. Dressing-Compartment Brackets: Match toilet-compartment brackets specified in Section 102113 "Toilet Compartments."

Textured finishes are available from some manufacturers, but textures vary.

J. Stainless-Steel Finish: [No. 4, bright, directional polish] [Manufacturer's standard textured finish] [Match stainless-steel toilet-compartment finish, specified in Section 102113 "Toilet Compartments,"] <Insert finish> on exposed faces. Protect exposed surfaces from damage by applying strippable, temporary protective covering before shipment.

2.4 PHENOLIC-CORE COMPARTMENTS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. Retain one of first two paragraphs and list of manufacturers below. See Section 016000 "Product Requirements."

A. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. Bobrick Washroom Equipment, Inc.

2. General Partitions Mfg. Corp.

3. Global Steel Products Corp.

4. Metpar Corp.

5. Rockville Partitions Incorporated.

6. Sanymetal; a Crane Plumbing company.

7. Shanahan's Limited.

8. Young Group Ltd. (The); Fabricated Products Division; DesignRite Partitions.

9. <Insert manufacturer's name>.

C. Configuration: [Shower compartment] [Shower and dressing compartments] [Shower compartment with two dressing compartments] [As shown on Drawings].

D. Enclosure Style: [Overhead braced] [Floor and ceiling anchored] <Insert style>.

E. Panel and Pilaster Construction: Solid phenolic material consisting of solid phenolic-core panel with melamine facing on both sides fused to substrate during panel manufacture (not separately laminated) and with eased and polished edges. Provide minimum 3/4-inch- (19-mm-) thick pilasters and minimum 1/2-inch- (13-mm-) thick panels.

Retain first paragraph below if doors are required; delete if curtains are required.

F. Door Construction: Match panels; 3/4-inch (19-mm) minimum thickness.

G. Pilaster [Shoes] [and] [Sleeves (Caps)]: Formed from stainless-steel sheet, not less than 3 inches (76 mm) high, finished to match hardware.

H. Brackets (Fittings):

Retain first subparagraph below for shower compartments; verify availability with manufacturers if inserting material other than aluminum. Retain first, second, or third subparagraphs for dressing compartments; if retaining more than one, indicate locations of each type of fitting on Drawings or by inserts.

1. Full-Height (Continuous) Type: Manufacturer's standard design; [clear-anodized aluminum] <Insert material>.

2. Stirrup Type: Ear or U-brackets; [clear-anodized aluminum] [stainless steel] [chrome-plated brass].

3. Dressing-Compartment Brackets: Match toilet-compartment brackets specified in Section 102113 "Toilet Compartments."

I. Phenolic-Core-Panel Finish:

1. Facing Sheet Finish: [One color and pattern] [Two colors and patterns] in each room.

Solid "through-color" phenolic-core panels in last option in subparagraph below are available from Global Steel Products and perhaps from other manufacturers.

2. Color and Pattern: [As indicated by manufacturer's designations,] [As selected by Architect from manufacturer's full range,] [Match phenolic-core toilet compartments specified in Section 102113 "Toilet Compartments"] <Insert color and pattern> with manufacturer's standard [dark-color core] [through-color core matching face sheet].

2.5 SOLID-POLYMER COMPARTMENTS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. Retain one of first two paragraphs and list of manufacturers below. See Section 016000 "Product Requirements."

A. Manufacturers: Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. Comtec Industries/Capitol Partitions.

2. General Partitions Mfg. Corp.

3. Global Steel Products Corp.

4. Metpar Corp.

5. Rockville Partitions Incorporated.

6. Santana Products, Inc.

7. Sanymetal; a Crane Plumbing company.

8. <Insert manufacturer's name>.

C. Configuration: [Shower compartment] [Shower and dressing compartments] [Shower compartment with two dressing compartments] [As shown on Drawings].

D. Enclosure Style: [Overhead braced] [Floor and ceiling anchored] <Insert style>.

Most formulations of HDPE do not comply with Class A or B flame-spread and smoke-developed index requirements; verify properties with manufacturers.

E. Panel and Pilaster Construction: Solid HDPE panel material, not less than 1 inch (25 mm) thick, seamless, with eased edges and with homogenous color and pattern throughout thickness of material.

Retain first subparagraph below if required for doors. If retaining, retain option for integral hinge in "Hinges" Subparagraph in "Accessories" Article.

1. Integral Hinges: Configure doors and pilasters to receive integral hinges.

2. Heat-Sink Strip: Manufacturer's standard, continuous, [clear-anodized extruded-aluminum] [or] [stainless-steel] strip fastened to exposed bottom edges of solid-polymer components to prevent burning.

3. Color and Pattern: [One color and pattern] [Two colors and patterns] in each room; [as indicated by manufacturer's designations] [as selected by Architect from manufacturer's full range] [match solid-polymer toilet compartments specified in Section 102113 "Toilet Compartments"] <Insert color and pattern>.

Retain first paragraph below if doors are required; delete if curtains are required.

F. Door Construction: Match panels.

G. Pilaster [Shoes] [and] [Sleeves (Caps)]: Manufacturer's standard design; [polymer] [or] [stainless steel].

1. Polymer Color and Pattern: [Match pilaster] [Contrast with pilaster, as indicated by manufacturer's designations] [Contrast with pilaster, as selected by Architect from manufacturer's full range] [Match solid-polymer toilet compartments specified in Section 102113 "Toilet Compartments"] <Insert color and pattern>.

H. Brackets (Fittings):

Retain first subparagraph below for shower compartments; verify availability with manufacturers if inserting material other than polymer or aluminum. Retain first, second, or third subparagraphs for dressing compartments; if retaining more than one, indicate locations of each type of fitting on Drawings or by inserts.

1. Full-Height (Continuous) Type: Manufacturer's standard design; [polymer or clear-anodized extruded aluminum] [polymer] [clear-anodized extruded aluminum] <Insert material>.

a. Polymer Color and Pattern: [Match panel] [Contrast with panel, as indicated by manufacturer's designations] [Contrast with panel, as selected by Architect from manufacturer's full range] [Match solid-polymer toilet compartments specified in Section 102113 "Toilet Compartments"] <Insert color and pattern>.

2. Stirrup Type: Ear or U-brackets; [clear-anodized aluminum] [stainless steel] [chrome-plated brass].

3. Dressing-Compartment Brackets: Match toilet-compartment brackets specified in Section 102113 "Toilet Compartments."

2.6 SHOWER RECEPTORS

Retain this article if shower receptors are prefabricated units; delete if receptors are fabricated on-site or if entire floor is drained.

A. General: Manufacturer's standard, prefabricated, terrazzo receptor complete with integral drain.

1. Curb: Not less than 2 inches (51 mm) and not more than 9 inches (229 mm) deep when measured from the top of the curb to the top of the drain; with curb threshold not less than 1 inch (25 mm) below the sides and back of the receptor[; and with a ramped entrance surface for accessible compartments].

2. Floor: Finished, sloping uniformly toward the drain and not less than 1/4 unit vertical in 12 units horizontal and not more than 1/2 inch (13 mm).

3. Drain Strainer: Manufacturer's standard, removable [brass strainer] [chrome strainer] [stainless-steel strainer] [plastic strainer, matching the receptor] <Insert description>.

4. Drain Gasket: Manufacturer's standard gasket, sized to fit waste pipe.

Details of waterstop vary among manufacturers.

5. Waterstop: Manufacturer's standard, continuous [galvanized-steel flange] [or] [rabbeted groove] to receive panels and create a waterstop when panels are in place.

B. Finish: Manufacturer's standard finish on exposed surfaces, [matching the enclosure panels] [contrasting with the enclosure panels, as indicated by manufacturer's designations] [contrasting with the enclosure panels, as selected by Architect from manufacturer's full range] <Insert finish> and with slip-resistant floor surface texture.

2.7 ACCESSORIES

Retain first paragraph below if doors are required; before revising, verify requirements of authorities having jurisdiction for accessible facilities.

A. Door Hardware and Accessories: Manufacturer's standard design, heavy-duty, operating hardware and accessories.

1. Material: [Clear-anodized aluminum] [Stainless steel] [Chrome-plated brass].

Hinge types vary in availability among manufacturers; most offer paired hinges in first subparagraph below. Paired hinges can be either preset or adjustable; for preset hinges, insert requirements for hold-open angle. Retain last option below only if retaining solid-polymer doors and pilasters configured to receive integral hinges.

2. Hinges: Manufacturer's standard, [paired, self-closing type that can be adjusted to hold doors open at any angle up to 90 degrees] [continuous, cam type that swings to a closed or partially open position] [continuous, spring-loaded type] [integral hinge for solid-polymer doors] <Insert requirement>.

Recessed latch-operating devices that are accessible to people with disabilities are available. Verify requirements of authorities having jurisdiction.

3. Latch and Keeper: Manufacturer's standard, [recessed] [surface-mounted] latch unit designed for emergency access and with combination rubber-faced door strike and keeper. Provide units that comply with regulatory requirements for accessibility[ at each compartment, accessible or not] [ at compartments designated as accessible].

4. Clothing Hooks: Manufacturer's standard clothing hooks in each dressing compartment[; include one combination hook and rubber-tipped bumper at in-swinging doors, sized to prevent door from hitting wall panel or compartment-mounted accessories].

5. Door Bumper: Manufacturer's standard, rubber-tipped bumper at out-swinging doors.

6. Door Pull: Manufacturer's standard unit at out-swinging doors that complies with regulatory requirements for accessibility. Provide units on both sides of doors at compartments designated as accessible.

Retain first paragraph below for shower compartments and overhead-braced dressing compartments. Generally, clear-anodized finish is provided.

B. Overhead Bracing: Manufacturer's standard, continuous, extruded-aluminum head rail or cap with antigrip profile; in manufacturer's standard finish.

Retain one of first two paragraphs below for openings into shower compartments and for curtained openings into dressing compartments. Generally, clear-anodized finish is provided.

C. Head Rail with Hooks: Manufacturer's standard, continuous, extruded-aluminum head rail or cap with curtain hooks running in concealed track; with antigrip profile; in manufacturer's standard finish.

D. Curtain Rod with Hooks: Manufacturer's standard, 1-inch- (25-mm-) diameter, [stainless-steel] <Insert material> curtain rod with matching hooks.

E. Curtain: Flame-resistant, [polyester-reinforced vinyl fabric] [manufacturer's standard fabric] <Insert fabric> that is stain resistant, self-sanitizing, antistatic, and antimicrobial; launderable to a temperature of not less than [90 deg F (32 deg C)] <Insert temperature>.

1. Flame Resistance: Passes NFPA 701 tests when tested by a testing and inspecting agency acceptable to authorities having jurisdiction.

2. Labeling: Identify fabrics with appropriate markings of applicable testing and inspecting agency.

3. Curtain Grommets: Two-piece, rolled-edge, rustproof, nickel-plated brass; spaced not more than 6 inches (152 mm) o.c.; machined into top hem.

4. Length: Where curtain extends to a floor surface, size so that bottom hem clears finished floor by not more than 1 inch (25 mm) and not less than 1/2 inch (13 mm) above floor surface. Where curtains extend to a shower-receptor curb, size so that bottom hem hangs above curb line and clears curb line by not more than 1/2 inch (13 mm).

5. Color and Pattern: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and pattern>.

F. Soap Holder: [Surface-mounted] [Recessed], seamless [stainless-steel] <Insert material> soap dish.

Seats in first paragraph below vary among manufacturers. Verify availability before specifying.

G. Seats: Manufacturer's standard, [panel-mounted] [wall-mounted] [or] [floor-mounted] benches.

1. Material: [Wood] [Solid phenolic] [Molded plastic] [Stainless steel] <Insert material>.

2. Operation: [Fixed] [Folding].

3. Finish: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] [Match enclosure panels] <Insert finish>.

H. Anchorages and Fasteners: Manufacturer's standard, exposed fasteners of stainless steel, chrome-plated steel, or solid brass, finished to match the items they are securing; with theft-resistant-type heads. Provide sex-type bolts for through-bolt applications. For concealed anchors, use stainless steel, hot-dip galvanized steel, or other rust-resistant, protective-coated steel.

2.8 FABRICATION

A. Overhead-Braced Compartments: Provide manufacturer's standard, corrosion-resistant supports, leveling method, and anchors at pilasters and walls to suit floor and wall conditions. Provide shoes at pilasters to conceal supports and leveling method.

B. Floor-and-Ceiling-Anchored Compartments: Provide manufacturer's standard, corrosion-resistant anchoring assemblies at pilasters and walls with leveling adjustment at tops and bottoms of pilasters. Provide shoes and sleeves (caps) at pilasters to conceal anchorage.

C. Door Sizes and Swings: Unless otherwise indicated, provide 24-inch- (610-mm-) wide, in-swinging doors for standard shower and dressing compartments, and 36-inch- (914-mm-) wide, out-swinging doors with a minimum 32-inch- (813-mm-) wide, clear opening for compartments designated as accessible.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General: Comply with manufacturer's written installation instructions. Install compartments rigid, straight, level, and plumb. Secure compartments in position with manufacturer's recommended anchoring devices.

B. Clearances for Dressing Compartment: Maximum 1/2 inch (13 mm) between pilasters and panels; 1 inch (25 mm) between panels and walls.

Retain first paragraph below for stirrup brackets. For solid-polymer compartments, three brackets are recommended for panel-to-wall and panel-to-pilaster connections to prevent sagging of panel.

C. Stirrup Brackets: Secure panels to walls and to pilasters with no fewer than [two brackets attached] [three brackets attached at midpoint and] near top and bottom of panel. Locate wall brackets so holes for wall anchors occur in masonry or tile joints. Align brackets at pilasters with brackets at walls.

D. Curtains: Install curtains to specified length and verify that they hang vertically without stress points or diagonal folds.

E. Shower Receptors: Install prefabricated shower receptors with drain gasket compression fit to outside diameter of waste pipe.

3.2 ADJUSTING

A. Curtain Adjustment: After hanging curtains, test and adjust each track or rod to produce unencumbered, smooth operation. Steam and dress down curtains as required to produce crease- and wrinkle-free installation. Remove and replace curtains that are stained or soiled or that have stress points or diagonal folds.

B. Hardware Adjustment: Adjust and lubricate hardware according to hardware manufacturer's written instructions for proper operation. Set hinges on in-swinging doors to hold doors open approximately 30 degrees from closed position when unlatched. Set hinges on out-swinging doors to return doors to fully closed position.

END OF SECTION 102116

	SHOWER AND DRESSING COMPARTMENTS
	102116 - 1



