Copyright 2011 AIA
MasterSpec Professional
11/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102239 - FOLDING PANEL PARTITIONS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Manually operated, acoustical panel partitions.

2. Electrically operated, acoustical panel partitions.

3. Manually operated, fire-rated panel partitions.

1.2 PREINSTALLATION MEETINGS

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

1.3 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

1. Product Data for Credit MR 4: For products having recycled content, documentation indicating percentages by weight of postconsumer and preconsumer recycled content. Include statement indicating cost for each product having recycled content.

"Certificates for Credit MR 7" Subparagraph below applies to LEED-NC, LEED-CI, and LEED for Schools.

2. Certificates for Credit MR 7: Chain-of-custody certificates certifying that operable panel partitions comply with forest certification and chain-of-custody requirements. Include statement indicating cost for each certified wood product.

"Product Data for Credit IEQ 4.4" Subparagraph below applies to LEED-NC and LEED-CI.

3. Product Data for Credit IEQ 4.4: For composite wood products, documentation indicating that products contain no urea formaldehyde.

"Laboratory Test Reports for Credit IEQ 4.4" Subparagraph below applies to LEED for Schools.

4. Laboratory Test Reports for Credit IEQ 4.4: For composite wood products used in operable panel partitions, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

C. Shop Drawings: For operable panel partitions.

1. Include plans, elevations, sections, details, and attachments to other work.

2. Indicate stacking and operating clearances. Indicate location and installation requirements for hardware and track, blocking, and direction of travel.

3. Include diagrams for power, signal, and control wiring.

D. Samples: For each exposed product and for each color and texture specified.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

E. Delegated-Design Submittal: For operable panel partitions.

1. Include design calculations for seismic restraints.

1.4 INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below for situations where limited space necessitates maximum utilization for efficient installation of different components or if coordination is required for installation of products and materials by separate installers.

A. Coordination Drawings: Reflected ceiling plans, drawn to scale and coordinated with each other, using input from installers of the items involved.

Revise "Setting Drawings" Paragraph below to suit Project.

B. Setting Drawings: For embedded items and cutouts required in other work[, including support-beam, mounting-hole template].

Retain "Seismic Qualification Certificates" Paragraph below if required by seismic criteria applicable to Project. See ASCE/SEI 7 for certification requirements for equipment and components.

C. Seismic Qualification Certificates: For operable panel partitions, tracks, accessories, and components, from manufacturer.

D. Product certificates.

E. Product test reports.

F. Sample warranty.

1.5 CLOSEOUT SUBMITTALS

A. Operation and maintenance data.

1.6 QUALITY ASSURANCE

Retain "Manufacturer Qualifications" and "Vendor Qualifications" paragraphs below if required for LEED for wood products. Before retaining, verify that manufacturers comply.

A. Manufacturer Qualifications: A qualified manufacturer that is certified for chain of custody by an FSC-accredited certification body.

B. Vendor Qualifications: A vendor that is certified for chain of custody by an FSC-accredited certification body.

C. Installer Qualifications: An entity that employs installers and supervisors who are trained and approved by manufacturer.

1.7 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of operable panel partitions that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods.

1. Warranty Period: [Two] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

A. Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design seismic bracing of tracks to structure above.

Retain "Seismic Performance" Paragraph below with "Seismic Qualification Certificates" Paragraph in "Informational Submittals" Article for projects requiring seismic design. Delete paragraph if performance requirements are indicated on Drawings. Model building codes and ASCE/SEI 7 establish criteria for buildings subject to earthquake motions. Coordinate requirements with structural engineer.

B. Seismic Performance: Operable panel partitions shall withstand the effects of earthquake motions determined according to [ASCE/SEI 7] <Insert requirement>.

Retain subparagraph below to define the term "withstand" as it applies to this Project. Definition varies with type of building and occupancy and is critical to valid certification.

1. The term "withstand" means "the partition panels will remain in place without separation of any parts from the system when subjected to the seismic forces specified."

C. Acoustical Performance: Provide operable panel partitions tested by a qualified testing agency for the following acoustical properties according to test methods indicated:

If retaining one of or both "Sound-Transmission Requirements" and "Noise-Reduction Requirements" subparagraphs below, indicate ratings in Part 2 or on Drawings.

1. Sound-Transmission Requirements: Operable panel partition assembly tested for laboratory sound-transmission loss performance according to ASTM E 90, determined by ASTM E 413, and rated for not less than the STC indicated.

Retain "Noise-Reduction Requirements" Subparagraph below only if NRC rating is required. Verify, with manufacturer, that values for NRC and STC are possible and practical for required products.

2. Noise-Reduction Requirements: Operable panel partition assembly, identical to partition tested for STC, tested for sound-absorption performance according to ASTM C 423, and rated for not less than the NRC indicated.

The IBC requires that textiles used as wall finishes are tested according to ASTM E 84 or UL 723 and that automatic sprinklers are present, or that textiles are tested as intended for use according to NFPA 265 Method B Protocol or to NFPA 286. Finishes other than textiles and having a thickness of 0.036 inch (0.9 mm) or more must be tested according to ASTM E 84, UL 723, or NFPA 286.

D. Fire-Test-Response Characteristics: Provide panels with finishes complying with one of the following as determined by testing identical products by UL or another testing and inspecting agency acceptable to authorities having jurisdiction:

1. Surface-Burning Characteristics: Comply with ASTM E 84 or UL 723; testing by a qualified testing agency.

Option in "Flame-Spread Index" Subparagraph below applies to textile-finished (fabric and carpet) wall surfaces.

a. Flame-Spread Index: [25] <Insert value> or less.

b. Smoke-Developed Index: [450] <Insert value> or less.

2. Fire Growth Contribution: Complying with acceptance criteria of local code and authorities having jurisdiction when tested according to [NFPA 265 Method B Protocol] [or] [NFPA 286].

Retain "Fire Resistance" Paragraph below for fire-rated operable panels.

E. Fire Resistance: Provide fire-rated operable panel partition assemblies[ including pass doors] complying with NFPA 80, based on testing according to UL 10B for fire-rated door assemblies.

1. Pass doors in fire-rated operable panel partition assemblies shall meet positive-pressure requirements.

F. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

2.2 OPERABLE ACOUSTICAL PANELS

A. Operable Acoustical Panels: Partition system, including panels, seals, finish facing, suspension system, operators, and accessories.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Advanced Equipment Corporation.

b. FolDoor.

c. Hufcor Inc.

d. KWIK-WALL Company.

e. Moderco Inc.

f. Modernfold, Inc.

g. Panelfold Inc.

h. <Insert manufacturer's name>.

B. Panel Operation: [Manually operated, individual] [Manually operated, paired] [Manually operated, continuously hinged] [Electrically operated, continuously hinged] panels.

C. Panel Construction: As required to support panel from suspension components and with reinforcement for hardware attachment. Fabricate panels with tight hairline joints and concealed fasteners. Fabricate panels so finished in-place partition is rigid; level; plumb; aligned, with tight joints and uniform appearance; and free of bow, warp, twist, deformation, and surface and finish irregularities.

D. Dimensions: Fabricate operable acoustical panel partitions to form an assembled system of dimensions indicated and verified by field measurements.

E. STC: Not less than [38] [41] [45] [47] [50] [52] [54] <Insert number>.

If retaining "NRC" Paragraph below, verify that NRC rating is suitable for Project. NRC ratings are not available from all manufacturers.

F. NRC: Not less than [0.50] [0.60] [0.65] [0.90] <Insert number>.

Consider deleting "Panel Materials" Paragraph below if products are adequately specified by performance requirements.

G. Panel Materials:

Retain "Certified Wood" Subparagraph below if wood components of operable panel partitions are required to be certified for LEED. An alternative method of dealing with this credit requirement is to retain requirement in Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.23 "Sustainable Design Requirements - LEED for Schools" that gives Contractor the option and responsibility to determine how the credit requirement will be met. Manufacturers certifying products as "FSC Mixed Credit" do not have to use 100 percent certified wood in the products, but in their total production must use an amount equal to or greater than the percentage of their production that is labeled "FSC Mixed Credit."

1. Certified Wood: Wood for operable panel partitions shall be certified as "FSC Pure"[ or "FSC Mixed Credit"] according to FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship," and to FSC STD-40-004, "FSC Standard for Chain of Custody Certification."

Retain "Recycled Content of Operable Panel Partitions" Subparagraph below to specify recycled content if applying for LEED Credit MR 4. An alternative method of dealing with these credit requirements is to retain requirement in Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.23 "Sustainable Design Requirements - LEED for Schools" that gives Contractor the option and responsibility to determine how these credit requirements will be met.

2. Recycled Content of Operable Panel Partitions:

Retain "Recycled Content of Steel," "Recycled Content of Aluminum," and "Recycled Content of Gypsum Panel Products" subparagraphs below to suit Project; verify available recycled content with manufacturer.

The USGBC allows a default value of 25 percent to be used for steel, without documentation; higher percentages can be claimed if they are supported by appropriate documentation. The Steel Recycling Institute indicates that steel sheet typically has 23 percent postconsumer recycled content and 1.5 percent preconsumer recycled content.

a. Recycled Content of Steel: Postconsumer recycled content plus one-half of preconsumer recycled content not less than [25] <Insert number> percent by weight.

b. Recycled Content of Aluminum: Postconsumer recycled content plus one-half of preconsumer recycled content not less than <Insert number> percent by weight.

c. Recycled Content of Gypsum Panel Products: Postconsumer recycled content plus one-half of preconsumer recycled content not less than <Insert number> percent by weight.

3. Steel Frame: Steel sheet, manufacturer's standard thickness.

4. Steel Face/Liner Sheets: Tension-leveled steel sheet, manufacturer's standard thickness.

5. Aluminum: Alloy and temper recommended by aluminum producer and finisher for type of use, corrosion resistance, and finish indicated; manufacturer's standard strengths and thicknesses for type of use.

6. Gypsum Board: ASTM C 1396/C 1396M.

7. Cement Board: ASTM C 1288.

Retain option prohibiting urea formaldehyde in "Particleboard," "Medium-Density Fiberboard," and "Plywood" subparagraphs below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.4.

8. Particleboard: ANSI A208.1[, made with binder containing no urea formaldehyde].

9. Medium-Density Fiberboard: ANSI A208.2[, made with binder containing no urea formaldehyde].

10. Plywood: DOC PS 1[; made with adhesive containing no urea formaldehyde].

Retain "Composite Wood Products" Subparagraph below if required for LEED for Schools Credit IEQ 4.4.

11. Composite Wood Products: Products shall comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

H. Panel Closure: Manufacturer's standard.

I. Hardware: Manufacturer's standard as required to operate operable panel partition and accessories; with decorative, protective finish.

2.3 OPERABLE FIRE-RATED PANELS

A. Operable Fire-Rated Panels: Fire-rated, acoustical panel partition system, including panels, seals, finish facing, suspension system, operators, and accessories.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Advanced Equipment Corporation.

b. Hufcor, Inc.

c. Moderco Inc.

d. Modernfold, Inc.

e. Panelfold Inc.

f. <Insert manufacturer's name>.

B. Panel Operation: Manually operated, [individual] [paired] panels.

C. Panel Construction: As required to support panel from suspension components and with reinforcement for hardware attachment. Fabricate panels with tight hairline joints and concealed fasteners. Fabricate panels so finished in-place partition is rigid; level; plumb; aligned, with tight joints and uniform appearance; and free of bow, warp, twist, deformation, and surface and finish irregularities.

D. Dimensions: Fabricate operable fire-rated panel partitions to form an assembled system of dimensions indicated and verified by field measurements.

E. Fire Rating: [1] <Insert number> hour(s).

F. STC: Not less than [38] [41] [45] [47] [50] [52] [54] <Insert number>.

If retaining "NRC" Paragraph below, verify that NRC rating is suitable for Project. NRC ratings are not available from all manufacturers.

G. NRC: Not less than [0.50] [0.60] [0.65] [0.90] <Insert number>.

Consider deleting "Panel Materials" Paragraph below if products are adequately specified by performance requirements.

H. Panel Materials:

Retain "Recycled Content of Operable Panel Partitions" Subparagraph below to specify recycled content if applying for LEED Credit MR 4. An alternative method of dealing with these credit requirements is to retain requirement in Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.23 "Sustainable Design Requirements - LEED for Schools" that gives Contractor the option and responsibility to determine how these credit requirements will be met.

1. Recycled Content of Operable Panel Partitions:

Retain "Recycled Content of Steel" Subparagraph below to suit Project; verify available recycled content with manufacturer.

The USGBC allows a default value of 25 percent to be used for steel, without documentation; higher percentages can be claimed if they are supported by appropriate documentation. The Steel Recycling Institute indicates that steel sheet typically has 23 percent postconsumer recycled content and 1.5 percent preconsumer recycled content.

a. Recycled Content of Steel: Postconsumer recycled content plus one-half of preconsumer recycled content not less than [25] <Insert number> percent by weight.

2. Steel Frame: Steel sheet, manufacturer's standard thickness.

3. Steel Face/Liner Sheets: Tension-leveled steel sheet, manufacturer's standard thickness.

I. Panel Closure: Manufacturer's standard.

J. Hardware: Manufacturer's standard as required to operate fire-rated operable panel partition and accessories; with decorative, protective finish.

2.4 SEALS

A. General: Provide seals that produce operable panel partitions complying with performance requirements and the following:

1. Seals made from materials and in profiles that minimize sound leakage.

2. Seals fitting tight at contact surfaces and sealing continuously between adjacent panels and between operable panel partition perimeter and adjacent surfaces, when operable panel partition is extended and closed.

Retain one of two "Horizontal Bottom Seals" paragraphs below. Second offers better acoustical performance than continuous-contact seals; verify performance and availability with manufacturer.

B. Horizontal Bottom Seals: Manufacturer's standard continuous-contact seal exerting uniform constant pressure on floor.

C. Horizontal Bottom Seals: PVC-faced, mechanical, retractable, constant-force-contact seal exerting uniform constant pressure on floor when extended, ensuring horizontal and vertical sealing and resisting panel movement.

Retain one type of retractable bottom seal in "Mechanically Operated for Acoustical Panels," "Mechanically Operated for Fire-Rated Panels," and "Automatically Operated for Acoustical Panels" subparagraphs below. Verify availability with manufacturer and revise clearance dimensions as required to accommodate anticipated structure and floor deflections and irregularities.

1. Mechanically Operated for Acoustical Panels: Extension and retraction of bottom seal by operating handle or built-in operating mechanism, with operating range not less than [1-1/2 inches (38 mm)] [2 inches (50 mm)] [4 inches (102 mm)] [6 inches (152 mm)] between retracted seal and floor finish.

2. Mechanically Operated for Fire-Rated Panels: Extension and retraction of bottom seal by operating handle or built-in operating mechanism, with operating range not less than [1-1/2 inches (38 mm)] [2 inches (50 mm)] [4 inches (102 mm)] between retracted seal and floor finish.

3. Automatically Operated for Acoustical Panels: Extension and retraction of bottom seal automatically operated by movement of partition, with operating range not less than [1 inch (25 mm)] [1-1/2 inches (38 mm)] [2 inches (50 mm)] between retracted seal and floor finish.

2.5 PANEL FINISH FACINGS

Retain and revise this article to suit Project. If panels are unfinished for field finishing, consult manufacturers.

A. General: Provide finish facings for panels that comply with indicated fire-test-response characteristics and that are factory applied to operable panel partitions with appropriate backing, using mildew-resistant nonstaining adhesive as recommended by facing manufacturer's written instructions.

1. Color/Pattern: [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color/pattern>.

B. Vinyl-Coated Fabric Wall Covering: Manufacturer's standard, mildew-resistant, washable, vinyl-coated fabric wall covering; complying with CFFA-W-101-D for type indicated; Class A.

C. Carpet Wall Covering: Manufacturer's standard[ nonwoven, needle-punched carpet with fibers fused to backing], from same dye lot, treated to resist stains.

D. Fabric Wall Covering: [Manufacturer's standard fabric] [100 percent polyolefin woven fabric] <Insert fabric description>, from same dye lot, treated to resist stains.

E. High-Pressure Decorative Laminate: NEMA LD 3, Horizontal grade.

F. Wood Veneer: Laminated to [noncombustible] [fire-retardant-treated wood] core with moisture-resistant adhesive.

1. Species, Cut, and Finish: [As selected by Architect from manufacturer's full range] <Insert species, cut, and finish>.

G. Paint: Manufacturer's standard factory-painted finish.

H. Cap-Trimmed Edges: Protective perimeter-edge trim with tight hairline joints concealing edges of panel and finish facing.

I. Trimless Edges: Fabricate exposed panel edges so finish facing wraps uninterrupted around panel, covering edge and resulting in an installed partition with facing visible on vertical panel edges, without trim, for minimal sightlines at panel-to-panel joints.

2.6 SUSPENSION SYSTEMS

A. Tracks: Steel or aluminum [mounted directly to overhead structural support,] [with adjustable steel hanger rods for overhead support,] designed for operation, size, and weight of operable panel partition indicated. Size track to support partition operation and storage without damage to suspension system, operable panel partitions, or adjacent construction. Limit track deflection to no more than 0.10 inch (2.54 mm) between bracket supports. Provide a continuous system of track sections and accessories to accommodate configuration and layout indicated for partition operation and storage.

B. Carriers: Trolley system as required for configuration type, size, and weight of partition and for easy operation; with ball-bearing wheels.

C. Track Intersections, Switches, and Accessories: As required for operation, storage, track configuration, and layout indicated for operable panel partitions, and compatible with partition assembly specified. Fabricate track intersections and switches from steel or aluminum.

2.7 ELECTRIC OPERATORS

A. General: Factory-assembled electric operation system of size and capacity recommended and provided by operable panel partition manufacturer for partition specified; with electric motor and factory-prewired motor controls, speed reducer, chain drive, control stations, control devices, and accessories required for operation. Include wiring from control stations to motor. Coordinate operator wiring requirements and electrical characteristics with building electrical system.

B. Comply with NFPA 70.

C. Control Equipment: Comply with NEMA ICS 1, NEMA ICS 2, and NEMA ICS 6.

D. Control Stations: Two single-key-operated, constant-pressure control stations located remotely from each other on opposite sides and opposite ends of partition run. Wire in series to require simultaneous activation of both key stations to operate partition. Each three-position control station labeled "Open," "Close," and "[Off] [Stop]." Furnish two keys per station.

E. Obstruction-Detection Devices: Equip each motorized operable panel partition with indicated automatic safety sensor that causes operator to immediately [shut off motor] [stop and reverse direction].

F. Limit Switches: Adjustable switches, interlocked with motor controls and set to automatically stop operable panel partition at fully extended and fully stacked positions.

G. Emergency Release Mechanism: Quick disconnect-release of electric-motor drive system, permitting manual operation in event of operating failure.

H. Electric Interlock: Equip each motorized operable panel partition with electric interlocks at locations indicated, to prevent operation of operable panel partition under the following conditions:

1. On storage pocket door, to prevent operation if door is not in fully open position.

2. On partitions at location of convergence by another partition, to prevent operation if merging partitions are in place.

2.8 ACCESSORIES

A. Pass Doors: Swinging door built into and matching panel [materials,] [construction,] [acoustical qualities,] [fire rating,] finish and thickness, complete with frames and operating hardware. Hinges finished to match other exposed hardware.

Retain "Accessibility Standard" Subparagraph below to suit Project. Doors operated only by security personnel are exempt from cited standards.

1. Accessibility Standard: Fabricate doors to comply with applicable provisions in [ICC A117.1] [and] [the U.S. Architectural & Transportation Barriers Compliance Board's ADA-ABA Accessibility Guidelines for Buildings and Facilities] <Insert requirements of authorities having jurisdiction>.

2. Single Pass Door: [36 by 80 inches (914 by 2032 mm)] [36 by 84 inches (914 by 2134 mm)] <Insert dimensions>.

3. Double Pass Door: [72 by 80 inches (1829 by 2032 mm)] [72 by 84 inches (1829 by 2134 mm)] <Insert dimensions>.

4. Pass-Door Hardware: Equip pass door with the following:

Features in eight subparagraphs below can be used with either a single or double pass door. Retain those required and revise to suit Project.

a. Door Seals: [Mechanically operated floor seal on panels containing pass doors] [Sweep floor seals].

b. [Panic] [Fire exit] hardware.

c. Concealed door closer.

d. Door Viewer: Installed with view in direction of swing.

e. Exit Sign: Recessed, self-illuminated.

Retain "Latchset" Subparagraph or one of two "Lock" subparagraphs below.

f. Latchset: Passage set.

g. Lock: Key-operated lock with cylinder[, keyed to master key system,] operable from both sides of door. Include two keys per lock.

h. Lock: Deadlock to receive cylinder, operable from both sides of door. See [Section 087100 "Door Hardware"] [Section 087111 "Door Hardware (Descriptive Specification)"] for lock cylinder and keying requirements.

B. Storage Pocket Door: Full height at end of partition runs to conceal stacked partition; of same materials, finish, construction, thickness, and acoustical qualities as panels; complete with operating hardware[ and acoustical seals at soffit, floor, and jambs]. Hinges in finish to match other exposed hardware.

Retain one of three subparagraphs below.

1. Manufacturer's standard method to secure storage pocket door in closed position.

2. Rim Lock: Key-operated lock cylinder[, keyed to master key system,] to secure storage pocket door in closed position. Include two keys per lock.

Lock in "Rim Lock" Subparagraph below may be required for certain situations such as schools.

3. Rim Lock: Deadlock to receive cylinder, to secure storage pocket door in closed position. See [Section 087100 "Door Hardware"] [Section 087111 "Door Hardware (Descriptive Specification)"] for lock cylinder and keying requirements.

C. Windows: [Manufacturer's standard] [As indicated] <Insert requirement>.

D. Work Surfaces: Quantities, placement, and size indicated.

Retain applicable requirements in "Surface" and "Surface Color" subparagraphs below, or revise or insert others to suit Project. See Evaluations.

1. Surface: [Porcelain steel marker/projection surface] [Self-healing, tackable, vinyl-coated fabric wall covering, complying with CFFA-W-101-D, Type II, and indicated fire-test-response characteristics; laminated to natural cork tackboard] <Insert description>.

2. Surface Color: [Matching Architect's sample] [As selected by Architect from manufacturer's full range] <Insert manufacturer's designation for color>.

E. Chalk Tray[ and Eraser Pocket]: Manufacturer's standard.

F. Chair Rails: <Insert material, finish, dimensions, and other characteristics not indicated on Drawings>. [Recessed] [Surface mounted] in locations indicated on Drawings.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General: Comply with ASTM E 557 except as otherwise required by operable panel partition manufacturer's written installation instructions.

B. Install operable panel partitions and accessories after other finishing operations, including painting, have been completed in area of partition installation.

C. Broken, cracked, chipped, deformed, or unmatched panels are not acceptable.

D. Broken, cracked, deformed, or unmatched gasketing or gasketing with gaps at butted ends is not acceptable.

E. Light-Leakage Test: Illuminate one side of partition installation and observe vertical joints and top and bottom seals for voids. Adjust partitions for alignment and full closure of vertical joints and full closure along top and bottom seals.

3.2 ADJUSTING

A. Adjust [pass doors] [and] [storage pocket doors] to operate smoothly and easily, without binding or warping.

B. Verify that safety devices are properly functioning.

3.3 DEMONSTRATION

A. Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain operable panel partitions.

END OF SECTION 102239

	FOLDING PANEL PARTITIONS
	102239 - 1



