Copyright 2012 AIA
MasterSpec Professional
02/12

Copyright 2012 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102819 - TUB AND SHOWER DOORS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes frameless shower doors and enclosures.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

"Product Data for Credit IEQ 4.1" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for sealants.

1. Product Data for Credit IEQ 4.1: For sealants, documentation including printed statement of VOC content.

C. Shop Drawings: For tub and shower doors and enclosures.

D. Samples: For each type of exposed finish.

1.3 INFORMATIONAL SUBMITTALS

A. Sample warranties.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.5 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of tub and shower doors and enclosures that fail in materials or workmanship within specified warranty period without monetary limitation.

Verify available warranties and warranty periods for units and components.

1. Warranty Period: [Two] [Three] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 FRAMELESS ENCLOSURES

Verify that units in this article are acceptable to authorities having jurisdiction. Some jurisdictions do not permit frameless doors and enclosures.

A. Frameless glass panels with mounting and operating hardware of types and sizes required to support imposed loads.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Agalite; Hartung Glass Industries.

b. Alumax; Sapa Extrusions, Inc.

c. American Shower Door.

d. Artistcraft Shower Doors.

e. Basco Corporation.

f. Cardinal Shower Enclosures; Hoskin & Muir, Inc.

g. Century Bathworks.

h. Fleurco Shower Doors.

i. Kohler Co.

j. Southeastern Aluminum Products, Inc.

k. <Insert manufacturer's name>.

B. Hardware and Trim: Manufacturer's standard units as indicated and as required for complete installation.

1. Materials:

Retain "Aluminum," "Brass," or "Stainless Steel" Subparagraph below.

a. Aluminum:

1) Finish: [Clear anodic] [Color anodic] [Baked enamel or powder coat] <Insert finish>.

Retain "Color" Subparagraph below for color-anodized or baked-enamel or powder-coated aluminum.

2) Color: [Silver] [Gold] [Black] <Insert color>.

b. Brass:

1) Finish: [Chrome] [Satin chrome] [Brass] [Satin brass] [Oil-rubbed brass] [Bronze] [Brushed bronze] [Oil-rubbed bronze] <Insert finish>.

c. Stainless Steel:

1) Finish: [No. 4 directional satin finish] [No. 6 dull satin finish] [No. 7 reflective, directional finish] <Insert finish>.

C. Bypass Doors: Sliding units suspended from extruded-aluminum header track by fully adjustable, sealed, heavy-duty ball-bearing rollers. Self-draining sill tracks with nylon panel guides. Molded jamb bumpers with concealed fasteners.

1. Door Pulls: Full-door-width, single-sided towel bars.

Retain "Safety Clip System" Subparagraph below for safety device available from some manufacturers on some systems.

2. Safety Clip System: Manufacturer's standard safety device designed to prevent doors from falling off sliding track.

D. Swinging Doors: Hinged for [90 degrees out] [135 degrees] [180 degrees] swing. Self-centering when doors are within 15 degrees of closed position. Soft bulb seal or wipes; affixed to door to direct water back into enclosure and provide a tight water seal.

1. Hinges: [Side hinged] [Top-and-bottom pivots].

2. Door Pulls: [Single-sided towel bar] [Back-to-back towel bars] [Knobs].

Retain "Towel Bar Length" Subparagraph below for towel bars.

a. Towel Bar Length: [18 inches (457 mm)] [24 inches (610 mm)] [30 inches (762 mm)] <Insert dimension>.

E. Fixed Panels: [Side] [Top-and-bottom] mounts; match hinges in material and finish.

F. Ventilating Transom: Rotating clamps centered on transom.

Retain one of two "Glazing" paragraphs below. Retain first paragraph if glazing is included in Section 088000 "Glazing."

G. Glazing: Comply with requirements in Section 088000 "Glazing."

Retain "Glazing" Paragraph below if requirements are specified in this Section.

H. Glazing: Safety glazing materials complying with 16 CFR 1201, Category II, with permanently etched identification acceptable to authorities having jurisdiction.

Retain first option in "Glass Nominal Thickness" Subparagraph below to allow manufacturer to determine glass thickness. Thicknesses below correspond to 5/16, 3/8, and 1/2 inch, respectively.

1. Glass Nominal Thickness: [As determined by manufacturer based on panel size] [8 mm] [10 mm] [12 mm] <Insert thickness>.

2. Clear Glass: ASTM C 1048, Type I, Quality-Q3, Class I (clear), Kind FT.

Coating in "Protective, Self-Cleaning, Glass Coating" Subparagraph below, offered by many manufacturers, is applied pyrolytically on the float line.

3. Protective, Self-Cleaning, Glass Coating: Clear float glass with a coating on first surface having both photocatalytic and hydrophilic properties that act to loosen dirt and to cause water to sheet evenly over the glass instead of beading.

I. Fasteners: Manufacturer's standard stainless-steel or other noncorrosive fasteners.

J. Sealant: Mildew-resistant, single-component, nonsag, neutral-curing silicone joint sealant; ASTM C 920, Type S, Grade NS, Class 25, for Use NT.

Retain subparagraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.1.

1. Sealant shall have a VOC content of 250 g/L or less.

K. Materials:

1. Aluminum: Alloy and temper recommended by manufacturer for type of use and finish indicated.

a. Sheet and Plate: ASTM B 209 (ASTM B 209M).

b. Extrusions: ASTM B 221 (ASTM B 221M).

2. Stainless-Steel Sheet: ASTM A 666, Type 302 or 304.

3. Stainless-Steel Bars and Shapes: ASTM A 276, Type 302 or 304.

4. Copper-Alloy Sheet and Shapes: ASTM B 36/B 36M.

5. Copper-Alloy Extrusions: ASTM B 455, alloy UNS No. C38500 (architectural brass).

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Prepare and install as recommended in manufacturer's written instructions unless more stringent requirements are contained in GANA's "Glazing Manual."

B. Clean substrates, removing projections, filling voids, and sealing joints.

C. Set units level, plumb, and true to line, without warp or rack of frames and panels, and anchor securely in place.

D. Fasten components securely in place, with provisions for thermal movement. Install with concealed fasteners unless otherwise indicated.

E. Install components to drain and return water to tub or shower.

F. Install doors to produce smooth operation and tight fit at contact points.

G. Repair, refinish, or replace components damaged during installation.

3.2 ADJUSTING AND CLEANING

A. Adjust operating parts and hardware for smooth, quiet operation and watertight closure. Lubricate hardware and moving parts.

B. Remove nonpermanent labels, and clean surfaces immediately after installation.

END OF SECTION 102819

	TUB AND SHOWER DOORS
	102819 - 1



