Copyright 2010 AIA
MasterSpec Professional
11/10

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 116653 - GYMNASIUM DIVIDERS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Walk-draw divider systems.

2. Fold-up divider systems.

3. Radius-fold divider systems.

4. Peak-fold divider systems.

5. Roll-up divider systems.

6. Top-roll divider systems.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. Shop Drawings: For gymnasium dividers.

C. Samples: For each exposed product and for each item and color specified.

1.3 CLOSEOUT SUBMITTALS

A. Operation and maintenance data.

1.4 QUALITY ASSURANCE

Consider retaining "Installer Qualifications" Paragraph below for custom-engineered equipment and automatic operation of equipment. If retaining either option below, verify availability with manufacturers.

A. Installer Qualifications: [Fabricator of products] [An entity that employs installers and supervisors who are trained and approved by manufacturer].

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 WALK-DRAW DIVIDER SYSTEMS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. AALCO Manufacturing.

2. ADP Lemco.

3. Arizona Courtlines, Inc.

4. Bison, Inc.

5. Douglas Industries, Inc.

6. Draper, Inc.

7. Institutional Products, Inc.

8. Jaypro Sports Construction Group.

9. Performance Sports Systems; a Gared Holdings company.

10. Porter Athletic, Inc.

11. Revere Plastics, Inc.

12. Spalding Equipment.

13. <Insert manufacturer's name>.

C. Divider Curtain System: Manually operated, [cable-suspended] [channel track] system, and as follows:

1. Top Hem: Double-thickness mesh or solid vinyl for grommets.

2. Outer Edge Hems: [Double turned and] [Triple turned and double] welded.

3. Bottom Pocket: Double thickness with proof coil chain in pocket.

Retain "Support Cable" Subparagraph below when choosing cable-suspended systems.

4. Support Cable: Manufacturer's standard galvanized-stranded-steel wire rope and fittings.

Retain "Channel Track" Subparagraph below when choosing channel track systems.

5. Channel Track: Galvanized steel or formed aluminum.

6. Grommets: Manufacturer's standard material, size, and spacing; for snaps or S-hooks.

2.2 FOLD-UP DIVIDER SYSTEMS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. AALCO Manufacturing.

2. ADP Lemco.

3. Arizona Courtlines, Inc.

4. Bison, Inc.

5. Draper, Inc.

6. Institutional Products, Inc.

7. Jaypro Sports Construction Group.

8. Performance Sports Systems; a Gared Holdings company.

9. Porter Athletic, Inc.

10. Revere Plastics, Inc.

11. Spalding Equipment.

12. <Insert manufacturer's name>.

C. Divider Curtain System: Electrically operated, upward folding, cable suspended, and as follows:

1. Top Hem: Double-thickness mesh or solid vinyl for continuous pipe batten.

2. Outer Edge Hems: Triple turned[ and welded].

3. Bottom Curtain Pocket: [6 inches (152 mm)] <Insert dimension> with manufacturer's standard pipe batten[ with padding].

4. Support Cables: Galvanized-stranded-steel wire rope, fittings, and grommets.

5. Support Chain and Fittings: Hardened alloy steel chain rated for lifting loads, with commercial-quality, corrosion-resistant steel connectors and hangers.

6. Curtain Battens and Drive Pipe: Steel pipe or tubing.

2.3 RADIUS-FOLD DIVIDER SYSTEMS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. AALCO Manufacturing.

2. ADP Lemco
3. Draper, Inc.

4. Jaypro Sports Construction Group.

5. Performance Sports Systems; a Gared Holdings company.

6. Porter Athletic, Inc.

7. Spalding Equipment.

8. <Insert manufacturer's name>.

C. Divider Curtain System: Electrically operated, upward folding, cable suspended, and as follows:

1. Top Hem: Double-thickness mesh or solid vinyl for pipe battens.

2. Outer Edge Hems: Triple turned[ and welded].

3. Bottom Curtain Pocket: [6 inches (152 mm)] <Insert dimension> with manufacturer's standard pipe batten[ with padding].

4. Support Cable: Galvanized-stranded-steel wire rope, fittings, and grommets.

5. Support Chain and Fittings: Hardened alloy steel chain rated for lifting loads, with commercial-quality, corrosion-resistant steel connectors and hangers.

6. Curtain Battens and Drive Pipe: Steel pipe or tubing.

a. Provide segmented or universal-joint-coupled pipe and hoist system design allowing divider to follow radius.

2.4 PEAK-FOLD DIVIDER SYSTEMS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. AALCO Manufacturing.

2. ADP Lemco.

3. Bison, Inc.

4. Draper, Inc.

5. Jaypro Sports Construction Group.

6. Performance Sports Systems; a Gared Holdings company.

7. Porter Athletic, Inc.

8. Spalding Equipment.

9. <Insert manufacturer's name>.

C. Divider Curtain System: Electrically operated, upward folding, cable suspended, and as follows:

1. Top Hem: Double-thickness mesh or solid vinyl for pipe battens.

2. Outer Edge Hems: Triple turned[ and welded].

3. Bottom Curtain Pocket: [6 inches (152 mm)] <Insert dimension> with manufacturer's standard pipe batten[ with padding].

4. Support Cable: Manufacturer's standard galvanized-stranded-steel wire rope, fittings, and grommets.

5. Support Chain and Fittings: Rated for lifting loads, with commercial-quality, corrosion-resistant steel connectors and hangers.

6. Curtain Battens: Steel pipe or tubing.

7. Provide drums of varying diameters to evenly retract curtains to sloped ceiling[, and hinged piping to fit to changes in ceiling slope]. Provide sheave assemblies or idler pulleys to guide cables, and other system components required to follow ceiling contour.

2.5 ROLL-UP DIVIDER SYSTEMS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. ADP Lemco.

2. Arizona Courtlines, Inc.

3. Bison, Inc.

4. Draper, Inc.

5. Institutional Products, Inc.

6. Jaypro Sports Construction Group.

7. Performance Sports Systems; a Gared Holdings company.

8. Porter Athletic, Inc.

9. Revere Plastics, Inc.

10. <Insert manufacturer's name>.

C. Divider Curtain System: Electrically operated with roll-up drive pipe, and as follows:

1. Top Hem: Double-thickness mesh or solid vinyl for continuous pipe batten.

2. Outer Edge Hems: Double turned[ and welded].

3. Belts: Manufacturer's standard width polyester or polyurethane webbing or fabric belts.

4. Support Chain and Fittings: Rated for lifting loads, with commercial-quality, corrosion-resistant steel connectors and hangers.

5. Curtain Battens and Drive Pipe: Steel pipe or tubing.

2.6 TOP-ROLL DIVIDER SYSTEMS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

B. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1. AALCO Manufacturing.

2. ADP Lemco.

3. Draper, Inc.

4. Jaypro Sports Construction Group.

5. Performance Sports Systems; a Gared Holdings company.

6. Spalding Equipment.

7. <Insert manufacturer's name>.

C. Divider Curtain System: Electrically operated, top-roll drive pipe, and as follows:

1. Outer Edge Hems: Double turned[ and welded].

2. Supports and Fittings: Corrosion-resistant steel clamps and hangers.

3. Curtain Battens and Drive Pipe: Steel pipe or tubing.

2.7 MATERIALS

Retain paragraphs in this article to suit Project.

A. Support Chain and Fittings: For chains used for overhead lifting, provide Grade 80 heat-treated alloy steel chains, complying with ASTM A 391/A 391M, with commercial-quality, hot-dip galvanized or zinc-plated steel connectors and hangers.

B. General-Purpose Chain: For chains not used for overhead lifting, provide carbon steel chain, complying with ASTM A 413/A 413M, Grade 30 proof coil chain or other grade recommended by gymnasium divider manufacturer. Provide coating type, chain size, number, and installation method complying with manufacturer's written instructions.

C. Anchors, Fasteners, Fittings, and Hardware: Manufacturer's standard corrosion-resistant or noncorrodible units; concealed.

2.8 ELECTRIC OPERATORS

Retain this article if dividers are electrically operated.

A. General: Factory-assembled electric operation system of size and capacity recommended and provided by gymnasium divider manufacturer for gymnasium dividers specified, with electric motors, thermal-overload protection, factory-prewired motor controls, control devices, and accessories required for proper operation. Include wiring from control stations to motors. Coordinate operator wiring requirements and electrical characteristics with building electrical system.

B. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

C. Limit Switches: Adjustable switches, interlocked with motor controls and set to automatically stop dividers at fully extended and fully retracted positions.

D. Key Switch Control System: NEMA ICS 6; Type 1 enclosure; momentary-contact, three-position switch-operated control. Provide one key per station.

If using multiple curtain configurations for different divider types, copy "Divider Curtains" Article below and revise for each divider type.

2.9 DIVIDER CURTAINS

A. [Upper ]Curtain, Mesh: Woven mesh of polyester yarn coated with vinyl, weighing not less than [7 oz./sq. yd. (237 g/sq. m)] [9 oz./sq. yd. (305 g/sq. m)] [13 oz./sq. yd. (440 g/sq. m)] <Insert value>.

1. Mesh Color: [Gray] [Black] [White] [Blue] [Red] [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities] <Insert color>.

Retain "Lower Curtain, Solid" Paragraph below if mesh does not extend to floor and if using solid (opaque) lower curtain.

B. Lower Curtain, Solid: Woven polyester fabric coated with vinyl, [18 oz./sq. yd. (610 g/sq. m)] [22 oz./sq. yd. (745 g/sq. m)] <Insert value>, [8-foot (2.4-m)] [10-foot (3.0-m)] <Insert dimension> height above floor.

1. Fabric Color(s): [One] [Two] <Insert number> color(s), [matching school colors] [as indicated by manufacturer's designations] [match Architect's sample] [as selected by Architect from full range of industry colors and color densities] <Insert color(s)>.

C. Hems: Folded and electronically welded.

D. Seams: Electronically welded.

E. Overall Curtain Height: [Floor to ceiling, within installation clearances required] [As indicated].

F. Bottom of Curtain: Approximately 2 inches (50 mm) above finished floor.

Retain "Divider Curtain Flame-Resistance Ratings" Paragraph below if authorities having jurisdiction consider divider curtains to be subject to fire tests. See the Evaluations for explanation of tests according to NFPA 701. Revise paragraph below if required by authorities having jurisdiction.

G. Divider Curtain Flame-Resistance Ratings: Passes NFPA 701, Test 2.

2.10 DIVIDER ACCESSORIES

"Safety Lock" Paragraph below is recommended by most manufacturers for all electrically operated dividers except for peak-fold systems that do not have a drive pipe. Verify availability with manufacturers.

A. Safety Lock: Locks drive system when speed exceeds manufacturer's recommended speed.

"Audible Motion Alarm" Paragraph below is recommended by major manufacturers for electrically operated dividers. Verify availability with manufacturers.

B. Audible Motion Alarm: Provide alarm with intermittent warning tone when curtain is raised or lowered.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General: Comply with manufacturer's written installation instructions.

B. Gymnasium Dividers and Components: Install level, plumb, square, and true; anchored securely to supporting structure; positioned at locations and elevations indicated; in proper relation to adjacent construction; and aligned with court layout.

Retain "Connections" Paragraph below for hardwired, electrically operated gymnasium dividers.

C. Connections: Connect automatic operators to building electrical system.

D. Adjust movable components of gymnasium dividers to operate safely, smoothly, easily, and quietly, free from binding, warp, distortion, uneven tension, nonalignment, misplacement, disruption, or malfunction, throughout entire operational range. Lubricate hardware and moving parts.

Retain "Limit Switch Adjustment" Paragraph below for electrically operated dividers.

E. Limit Switch Adjustment: Set and adjust upper and lower limit controls.

3.2 CLEANING

A. After completing gymnasium divider installation, inspect components. Remove spots, dirt, and debris and touch up damaged shop-applied finishes according to manufacturer's written instructions.

3.3 DEMONSTRATION

A. Train Owner's maintenance personnel to adjust, operate, and maintain gymnasium dividers.

END OF SECTION 116653

	GYMNASIUM DIVIDERS
	116653 - 1



