Copyright 2010 AIA
MasterSpec Professional
02/10

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 210518 - ESCUTCHEONS FOR FIRE-SUPPRESSION PIPING
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section title referenced in this Section is correct for this Project's Specifications; Section title may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Escutcheons.

2. Floor plates.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product indicated.

PART 2 -  PRODUCTS

2.1 ESCUTCHEONS

Retain one or both finish options in first paragraph below that match escutcheon types retained in Part 3.

A. One-Piece, Cast-Brass Type: With [polished, chrome-plated] [and] [rough-brass] finish and setscrew fastener.

B. One-Piece, Deep-Pattern Type: Deep-drawn, box-shaped brass with chrome-plated finish and spring-clip fasteners.

C. One-Piece, Stamped-Steel Type: With chrome-plated finish and spring-clip fasteners.

2.2 FLOOR PLATES

A. One-Piece Floor Plates: Cast-iron flange[ with holes for fasteners].

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Install escutcheons for piping penetrations of walls, ceilings, and finished floors.

B. Install escutcheons with ID to closely fit around pipe, tube, and insulation of piping and with OD that completely covers opening.

1. Escutcheons for New Piping:

a. Piping with Fitting or Sleeve Protruding from Wall: One-piece, deep-pattern type.

b. Chrome-Plated Piping: One-piece, cast-brass type with polished, chrome-plated finish.

c. Insulated Piping: One-piece, stamped-steel type.

Retain one of first two subparagraphs below.

d. Bare Piping at Wall and Floor Penetrations in Finished Spaces: One-piece, cast-brass type with polished, chrome-plated finish.

e. Bare Piping at Wall and Floor Penetrations in Finished Spaces: One-piece, stamped-steel type.

Retain one of first two subparagraphs below.

f. Bare Piping at Ceiling Penetrations in Finished Spaces: One-piece, cast-brass type with polished, chrome-plated finish.

g. Bare Piping at Ceiling Penetrations in Finished Spaces: One-piece, stamped-steel type.

Retain one of first two subparagraphs below.

h. Bare Piping in Unfinished Service Spaces: One-piece, cast-brass type with [polished, chrome-plated] [rough-brass] finish.

i. Bare Piping in Unfinished Service Spaces: One-piece, stamped-steel type.

Retain one of two subparagraphs below.

j. Bare Piping in Equipment Rooms: One-piece, cast-brass type with [polished, chrome-plated] [rough-brass] finish.

k. Bare Piping in Equipment Rooms: One-piece, stamped-steel type.

C. Install floor plates for piping penetrations of equipment-room floors.

D. Install floor plates with ID to closely fit around pipe, tube, and insulation of piping and with OD that completely covers opening.

Subparagraph below applies to both bare and insulated piping.

1. New Piping: One-piece, floor-plate type.

3.2 FIELD QUALITY CONTROL

A. Replace broken and damaged escutcheons and floor plates using new materials.

END OF SECTION 210518

	ESCUTCHEONS FOR FIRE-SUPPRESSION PIPING
	210518 - 1



