Copyright 2012 AIA
MasterSpec Professional
03/12

Copyright 2012 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 224100 - RESIDENTIAL PLUMBING FIXTURES
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Bathtubs.

2. Faucets.

3. Bidets.

4. Lavatories.

5. Showers.

6. Kitchen sinks.

7. Laundry trays.

8. Dishwasher air-gap fittings.

9. Disposers.

10. Water closets.

11. Toilet seats.

12. Supply fittings.

13. Waste fittings.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

Retain "Product Data for Prerequisite WE 1 ( and Credit WE 3) (, Credit WE 2, and Credit WE 3)" Subparagraph below for LEED-NC, LEED-CS, and LEED for Schools for fixtures and equipment that consume water.

1. Product Data for Prerequisite WE 1[ and Credit WE 3] [, Credit WE 2, and Credit WE 3]: Documentation indicating flow and water consumption requirements.

Retain "Product Data for Prerequisite WE 1" Subparagraph below for LEED-CI for fixtures and equipment that consume water.

2. Product Data for Prerequisite WE 1: Documentation indicating flow and water consumption requirements.

Retain "Product Data for Prerequisite WE 1( and Credit WE 2)" Subparagraph below for LEED-EB for fixtures and equipment that consume water.

3. Product Data for Prerequisite WE 1[ and Credit WE 2]: Documentation indicating flow and water consumption requirements.

Retain "Shop Drawings" Paragraph below if equipment includes wiring.

C. Shop Drawings: Include diagrams for power, signal, and control wiring.

1.3 INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below if coordination is required for installation of products and materials by separate installers.

A. Coordination Drawings: Counter cutout templates for mounting of counter-mounted plumbing fixtures.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance data.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 BATHTUBS

Copy "Bathtubs" Paragraph below and re-edit for each type of bathtub required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each bathtub.

A. Bathtubs <Insert drawing designation>: [Enameled cast iron] [Enameled steel] [FRP] [PMMA], with shower.

1. Enameled Cast-Iron Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Kohler Co.

2) <Insert manufacturer's name>.

2. Enameled-Steel Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Bootz Industries.

3) Briggs Plumbing Products, Inc.

4) Crane Plumbing, L.L.C.

5) Eljer, Inc.

6) <Insert manufacturer's name>.

3. FRP Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Clarion Bathware.

2) Florestone Products Co., Inc.

3) Praxis Industries, LLC.

4) Sterling.

5) Swan Corporation (The).

6) <Insert manufacturer's name>.

4. PMMA (Acrylic) Bathtubs:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Acryline USA, Inc.

2) American Standard America.

3) Aqua Bath Company, Inc.

4) Aqua Glass Corporation.

5) Aquatic Industries, Inc.

6) Clarion Bathware.

7) Crane Plumbing, L.L.C.

8) Florestone Products Co., Inc.

9) Kohler Co.

10) Lyons Industries, Inc.

11) Mansfield Plumbing Products LLC.

12) Praxis Industries, LLC.

13) St. Thomas Creations.

14) Swan Corporation (The).

15) <Insert manufacturer's name>.

5. Fixture:

Retain one of two "Standard" subparagraphs below.

a. Standard: ASME A112.19.1/CSA B45.2 for [enameled cast-iron] [enameled-steel] bathtubs.

b. Standard: ANSI Z124.1.2 for [FRP] [PMMA] bathtubs.

c. Bathing Surface: Slip resistant according to ASTM F 462.

d. Size: [48 by 30 inches (1219 by 762 mm)] [60 by 30 inches (1525 by 762 mm)] [66 by 30 inches (1680 by 762 mm)] <Insert dimensions> [with front apron] [drop-in type].

e. Color: [White] <Insert color>.

f. Drain Location: [Left] [Right] end.

g. Drain: NPS 1-1/2 (DN 40); chrome-plated-brass, pop-up waste and overflow.

6. Faucet: <Insert faucet designation from "Bathtub Faucets" Article>.

7. Supply Fittings: Included in faucet.

8. Tub Filler: Chrome-plated-brass diverter spout.

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B45.125.2.

b. Drain: Stainless steel or chrome-plated brass, removable strainer.

c. Overflow: Chrome-plated-brass escutcheon with toggle drain-plug device.

Retain one of two "Drain Piping" subparagraphs below.

d. Drain Piping: NPS 1-1/2 (DN 40) cast-brass overflow, P-trap, and waste.

e. Drain Piping: Schedule 40 [ABS] [or] [PVC], NPS 1-1/2 (DN 40) P-trap and waste.

2.2 BATHTUB FAUCETS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Bathtub Faucets" Paragraph below and re-edit for each type of single-handle, pressure-balance or thermostatic bathtub faucet required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each bathtub faucet.

B. Bathtub Faucets <Insert drawing designation>: Single handle, [pressure balance] [thermostatic].

1. Pressure-Balance Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Briggs Plumbing Products, Inc.

3) Central Brass Company.

4) Danze, Inc.

5) Ferguson Enterprises, Inc.

6) Gerber Plumbing Fixtures LLC.

7) Kohler Co.

8) Lawler Manufacturing Company, Inc.

9) Leonard Valve Company.

10) Matco-Norca.

11) Moen Incorporated.

12) Powers.

13) Speakman Company.

14) St. Thomas Creations.

15) Sterling.

16) Symmons Industries, Inc.

17) T & S Brass and Bronze Works, Inc.

18) Wolverine Brass, Inc.

19) Zurn Industries, LLC; AquaSpec Commercial Faucet Products.

20) <Insert manufacturer's name>.

2. Thermostatic Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Lawler Manufacturing Co., Inc.

3) Leonard Valve Company.

4) Powers.

5) <Insert manufacturer's name>.

3. Standards: ASME A112.18.1/CSA B125.1 and ASSE 1016.

4. Faucet:

a. Body Material: Solid brass.

b. Finish: Polished chrome plate.

c. Maximum Flow Rate: [2.5 gpm (9.5 L/min.)] <Insert value> unless otherwise indicated.

d. Mounting: [Concealed] [Exposed].

e. Operation: Single handle, [push-pull] [or] [twist or rotate] control, with hot- and cold-water indicators.

f. Antiscald Device: [Integral with mixing valve] [Separate unit] [Not required].

g. Check Stops: Check-valve type, integral with or attached to body; on hot- and cold-water supply connections.

h. Diverter: In-tub filler spout.

i. Supply Connections: NPS 1/2 (DN 15).

5. Shower Head:

a. Standard: ASME A112.18.1/CSA B125.1.

b. Type: [Ball joint with arm and flange] [Without ball joint, but with arm and flange] [Ball joint and head integral with mounting flange] [Integral with mounting flange] [Hand shower. Include wall-mounting device] <Insert type>.

Retain "Backflow-Prevention Device" Subparagraph below with hand-shower type.

c. Backflow-Prevention Device: ASSE 1014.

d. Shower Head Material: Metallic with chrome-plated finish.

e. Spray Pattern: [Adjustable] [Fixed].

f. Integral Volume Control: [Not required] [Required].

Retain "Shower-Arm, Flow-Control Fitting" Subparagraph below only if retaining option that includes arm and flange in "Type" Subparagraph above.

g. Shower-Arm, Flow-Control Fitting: [Not required] [1.5 gpm (5.7 L/min.)] [2.0 gpm (7.6 L/min.)].

Verify availability of feature in "Temperature Indicator" Subparagraph below if required.

h. Temperature Indicator: [Integral with faucet] [Not required].

6. Bathtub Filler Spout: Chrome-plated brass.

2.3 BIDETS

Copy "Bidets" Paragraph below and re-edit for each type of bidet required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each bidet.

A. Bidets <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Briggs Plumbing Products, Inc.

c. Capizzi.

d. Crane Plumbing, L.L.C.

e. Danze, Inc.

f. Duravit USA, Inc.

g. Eljer, Inc.

h. Ferguson Enterprises, Inc.; ProFlo Brand.

i. Gerber Plumbing Fixtures LLC.

j. Kohler Co.

k. Mansfield Plumbing Products LLC.

l. Peerless Pottery Sales, Inc.

m. St. Thomas Creations.

n. TOTO USA, INC.

o. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1 for vitreous-china bidets.

b. Type: With [spray] [flushing rim] [spray and flushing rim] [no spray].

c. Hole Punching: Number and spacing to match faucet.

d. Mounting: Floor.

e. Overflow: [Not required] [Required].

f. Color: [White] <Insert color>.

4. Faucet:

a. Standard: ASME A112.18.3.

b. Type: Manufacturer's standard with [vacuum breaker, diverter, and submerged spray] [over-rim filling], pop-up drain,[ NPS 1-1/4 (DN 32)] tailpiece, and chrome-plated finish. Include [manufacturer's standard] [cross] [lever] [wrist-blade] <Insert type> handles.

5. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

6. Waste Fittings: Comply with requirements in "Waste Fittings" Article, except faucet includes pop-up drain.

2.4 LAVATORIES

Copy "Lavatories" Paragraph below and re-edit for each type of counter-mounted lavatory required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

A. Lavatories <Insert drawing designation>: [Oval] [Rectangular] [Round], [enameled cast iron] [enameled steel] [PMMA] [solid surface] [stainless steel] [vitreous china], counter mounted.

1. Enameled Cast-Iron Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Commercial Enameling Company.

2) Eljer, Inc.

3) Kohler Co.

4) <Insert manufacturer's name>.

2. Enameled-Steel Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Bootz Industries.

3) Briggs Plumbing Products, Inc.

4) Crane Plumbing, L.L.C.

5) Eljer, Inc.

6) Mansfield Plumbing Products LLC.

7) <Insert manufacturer's name>.

3. PMMA (Acrylic) Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Benjamin Mfg. Co.

2) Royal Baths Manufacturing Co.

3) <Insert manufacturer's name>.

4. Solid-Surface Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Bradley Corporation.

3) DuPont.

4) Formica Corporation.

5) Franke Consumer Products, Inc.

6) InPro Corporation (IPC).

7) Rynone Manufacturing Corp.

8) Swan Corporation (The).

9) <Insert manufacturer's name>.

5. Stainless-Steel Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Acorn Engineering Company.

2) Elkay Manufacturing Co.

3) Intersan Manufacturing Company.

4) Just Manufacturing.

5) <Insert manufacturer's name>.

6. Vitreous-China Lavatories:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Barclay Products Limited.

3) Briggs Plumbing Products, Inc.

4) Crane Plumbing, L.L.C.

5) Duravit USA, Inc.

6) Eljer, Inc.

7) Gerber Plumbing Fixtures LLC.

8) Kohler Co.

9) Mansfield Plumbing Products LLC.

10) Peerless Pottery Sales, Inc.

11) St. Thomas Creations.

12) Sterling.

13) TOTO USA, INC.

14) <Insert manufacturer's name>.

7. Fixture:

Retain one of five "Standard" subparagraphs below.

a. Standard: ASME A112.19.1/CSA B45.2 for enameled [cast-iron] [-steel] lavatories.

b. Standard: ANSI Z124.3 for PMMA lavatories.

c. Standard: ANSI Z124.3 and ANSI/ICPA SS-1 for solid-surface lavatories.

d. Standard: ASME A112.19.3/CSA B45.4 for stainless-steel lavatories.

e. Standard: ASME A112.19.2/CSA B45.1 for vitreous-china lavatories.

f. Type: [Flat-rim with ledge] [Self-rimming].

Retain "Oval Nominal Size," ''Rectangular Nominal Size," or "Round Nominal Size" Subparagraph below.

g. Oval Nominal Size: [19 by 16 inches (483 by 406 mm)] [20 by 17 inches (508 by 432 mm)] <Insert dimensions>.

h. Rectangular Nominal Size: [18 by 15 inches (457 by 381 mm)] [19 by 16 inches (483 by 406 mm)] [20 by 18 inches (508 by 457 mm)] [24 by 20 inches (610 by 508 mm)] <Insert dimensions>.

i. Round Nominal Size: [18 inches (457 mm)] [19 inches (483 mm)] <Insert dimensions> in diameter.

j. Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

k. Faucet-Hole Location: [None] [Rim].

l. Color: [Not applicable] [White] <Insert color>.

8. Faucet: <Insert lavatory-faucet designation from "Lavatory Faucets" Article>.

9. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

10. Waste Fittings: Comply with requirements in "Waste Fittings" Article.

Copy "Lavatories" Paragraph below and re-edit for each type of vitreous-china, pedestal lavatory required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

B. Lavatories <Insert drawing designation>: [Oval] [Rectangular], vitreous china, pedestal.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Barclay Products Limited.

c. Briggs Plumbing Products, Inc.

d. Crane Plumbing, L.L.C.

e. Eljer, Inc.

f. Gerber Plumbing Fixtures LLC.

g. Kohler Co.

h. Mansfield Plumbing Products LLC.

i. Peerless Pottery Sales, Inc.

j. Sterling; a Kohler company.

k. St. Thomas Creations.

l. TOTO USA, INC.

m. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

Retain "Oval Nominal Size" or "Rectangular Nominal Size" Subparagraph below.

b. Oval Nominal Size: [20 by 17 inches (508 by 432 mm)] [24 by 18 inches (610 by 457 mm)] <Insert dimensions>.

c. Rectangular Nominal Size: [18 by 15 inches (457 by 381 mm)] [19 by 16 inches (483 by 406 mm)] [20 by 18 inches (508 by 457 mm)] [24 by 20 inches (610 by 508 mm)] <Insert dimensions>.

d. Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

e. Faucet-Hole Location: [Top] [Front wall] [Inclined panel].

f. Color: [White] <Insert color>.

4. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

5. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

6. Waste Fittings: Comply with requirements in "Waste Fittings" Article.

2.5 LAVATORY FAUCETS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Lavatory Faucets" Paragraph below and re-edit for each type of lavatory faucet valve required.

Insert number to complete drawing designation. Use these designations in lavatory description to identify required faucet for that lavatory.

B. Lavatory Faucets <Insert drawing designation>: [Single-control mixing] [Single-control nonmixing] [Two-handle mixing] valve.

1. General-Duty, Solid-Brass Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Bradley Corporation.

3) BrassTech Inc.

4) Central Brass Company.

5) Chicago Faucets.

6) Danze, Inc.

7) Delta Faucet Company.

8) Eljer, Inc.

9) Elkay Manufacturing Co.

10) Franke Consumer Products, Inc.

11) Gerber Plumbing Fixtures LLC.

12) Griffin Products, Inc.

13) GROHE America, Inc.

14) Hansgrohe Inc.

15) Houzer, Inc.

16) Hydrotek International, Inc.

17) Intersan Manufacturing Company.

18) Just Manufacturing.

19) Kohler Co.

20) Matco-Norca.

21) Moen Incorporated.

22) Price Pfister, Inc.

23) Rohl LLC.

24) Speakman Company.

25) T & S Brass and Bronze Works, Inc.

26) Watermark Designs, Ltd.

27) Wolverine Brass, Inc.

28) Zurn Plumbing Products Group.

29) <Insert manufacturer's name>.

2. General-Duty, Copper- or Brass-Underbody Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Briggs Plumbing Products, Inc.

3) Delta Faucet Company.

4) Eljer, Inc.

5) Ferguson Enterprises, Inc.

6) Gerber Plumbing Fixtures LLC.

7) Moen Incorporated.

8) Sterling.

9) WhiteRock Corp.

10) Wolverine Brass, Inc.

11) <Insert manufacturer's name>.

3. Standard: ASME A112.18.1/CSA B125.1.

4. General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture holes; coordinate outlet with spout and fixture receptor.

5. Body Material: [General-duty, solid brass] [General-duty, copper or brass underbody with brass cover plate].

6. Finish: [Polished chrome plate] [Polished brass].

7. Maximum Flow Rate: [2.2 gpm (8.3 L/min.)] [2.5 gpm (9.5 L/min.)] <Insert value>.

8. Centers: [3-3/8 inches (86 mm)] [4 inches (102 mm)] [6 inches (152 mm)] [8 inches (203 mm)] [Single hole] [Adjustable].

9. Mounting: [Deck, exposed] [Deck, concealed] [Back/wall, exposed] [Back/wall, concealed].

10. Valve Handle(s): [Lever] [Knob] [Knob, nonmetallic] [Cross, four arm] [Wrist blade, 4 inches (102 mm)] [Elbow, 6 inches (152 mm)] [Push button] [Not applicable].

11. Inlet(s): [NPS 3/8 (DN 10) tubing, plain end] [NPS 3/8 (DN 10) tubing, with NPS 1/2 (DN 15) male adaptor] [NPS 1/2 (DN 15) male shank] [NPS 1/2 (DN 15) female shank].

12. Spout: [Rigid] [Swing] [Rigid, gooseneck] [Swivel, gooseneck].

13. Spout Outlet: [Aerator] [Spray] [Plain end] [Spray, 0.5 gpm (1.9 L/min.)].

14. Operation: [Compression, manual] [Noncompression, manual] [Sensor] [Self-closing, metering].

15. Drain: [Pop up] [Stopper with chain] [Grid].

2.6 SHOWERS

Copy "Showers" Paragraph below and re-edit for each type of standard shower required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each shower.

A. Showers <Insert drawing designation>: Standard [FRP] [PMMA] with base and faucet.

1. FRP Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Aqua Glass Corporation.

2) Clarion Bathware.

3) Florestone Products Co., Inc.

4) LASCO Bathware.

5) MAAX.

6) Praxis Industries, LLC.

7) Sterling.

8) Swan Corporation (The).

9) <Insert manufacturer's name>.

2. PMMA (Acrylic) Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Acryline USA, Inc.

2) Aqua Bath Company, Inc.

3) Aqua Glass Corporation.

4) Aquatic Industries, Inc.

5) Clarion Bathware.

6) Crane Plumbing, L.L.C.

7) Jacuzzi, Inc.

8) Kohler Co.

9) LASCO Bathware.

10) MAAX.

11) Praxis Industries, LLC.

12) <Insert manufacturer's name>.

3. Standard: ANSI Z124.1.2.

4. Nominal Size: [36 by 34 inches (915 by 865 mm)] [42 by 36 inches (1065 by 915 mm)] [43 by 39 inches (1090 by 990 mm)] [48 by 34 inches (1219 by 865 mm)] <Insert dimensions>.

Retain first "Surround" Subparagraph below for FRP surround.

5. Surround: One piece[ or sealed, multiple piece].

Retain "Surround" Subparagraph below for PMMA (acrylic) surround.

6. Surround: One piece.

7. Bathing Surface: Slip resistant according to ASTM F 462.

8. Color: [White] <Insert color>.

9. Drain Location: [Left side] [Center] [Right side].

10. Faucet: <Insert faucet designation from "Shower Faucets" Article>.

Copy "Showers" Paragraph below and re-edit for each type of accessible shower required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each shower.

B. Showers <Insert drawing designation>: Accessible [FRP] [PMMA] with seat, grab bar, base, and faucet.

1. FRP Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Aqua Glass Corporation.

2) Clarion Bathware.

3) Florestone Products Co., Inc.

4) <Insert manufacturer's name>.
2. PMMA Showers:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Aqua Bath Company, Inc.

2) Aqua Glass Corporation.

3) Clarion Bathware.

4) Crane Plumbing, L.L.C.

5) <Insert manufacturer's name>.
3. Standards: ANSI Z124.1.2 and ICC/ANSI A117.1 for roll-in shower compartments.

4. Nominal Size: [60 by 36 inches (1525 by 915 mm)] [72 by 36 inches (1830 by 915 mm)] <Insert dimensions>.

Retain first "Surround" Subparagraph below for FRP surround.

5. Surround: One piece[ or sealed, multiple piece].

Retain "Surround" Subparagraph below for PMMA (acrylic) surround.

6. Surround: One piece.

7. Bathing Surface: Slip resistant according to ASTM F 462.

8. Color: [White] <Insert color>.

9. Drain Location: [Left side] [Center] [Right side].

10. Accessibility Options: Include grab bar and bench.

11. Faucet: Shower <Insert faucet designation from "Shower Faucets" Article>.

12. Drain: Grid, NPS 2 (DN 50).

2.7 SHOWER FAUCETS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Shower Faucets" Paragraph below and re-edit for each type of shower faucet required.

Insert number to complete drawing designation. Use these designations in shower description to identify required shower faucet.

B. Shower Faucets <Insert drawing designation>: [Single handle, pressure-balance, mixing valve] [Single handle, thermostatic, mixing valve] [Two handle, non-pressure-balance and non-thermostatic, mixing valve].

1. Single-Handle, Pressure-Balance Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Briggs Plumbing Products, Inc.

3) Central Brass Company.

4) Danze, Inc.

5) Ferguson Enterprises, Inc.

6) Gerber Plumbing Fixtures LLC.

7) Kohler Co.

8) Lawler Manufacturing Co., Inc.

9) Leonard Valve Company.

10) Matco-Norca.

11) Moen Incorporated.

12) Powers.

13) Speakman Company.

14) St. Thomas Creations.

15) Sterling.

16) Symmons Industries, Inc.

17) T & S Brass and Bronze Works, Inc.

18) Wolverine Brass, Inc.

19) Zurn Industries, LLC; AquaSpec Commercial Faucet Products.

20) <Insert manufacturer's name>.

2. Single-Handle, Thermostatic Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Lawler Manufacturing Co., Inc.

3) Leonard Valve Company.

4) Powers.

5) <Insert manufacturer's name>.

3. Two-Handle, Non-Pressure-Balance, and Non-Thermostatic Mixing Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Barclay Products Limited.

3) Briggs Plumbing Products, Inc.

4) Danze, Inc.

5) Ferguson Enterprises, Inc.

6) Gerber Plumbing Fixtures LLC.

7) Kohler Co.

8) Mansfield Plumbing Products LLC.

9) Matco-Norca.

10) Mustee, E. L. & Sons, Inc.

11) Paul Decorative Products.

12) Price Pfister, Inc.

13) St. Thomas Creations.

14) Sterling.

15) T & S Brass and Bronze Works, Inc.

16) Wolverine Brass, Inc.

17) Zurn Industries, LLC; AquaSpec Commercial Faucet Products.

18) <Insert manufacturer's name>.

4. Fixture:

Retain option in "Standard" Subparagraph below for pressure-balance and thermostatic faucets.

a. Standard: ASME A112.18.1/CSA B125.1[ and ASSE 1016].

b. General: Include hot- and cold-water indicators; check stops; and [fixed shower head, arm, and flange] [hand head complying with ASSE 1014 with arm, flange, hose, and bracket]. Coordinate faucet inlets with supplies.

c. Body Material: [Solid brass] <Insert material> [with nonmetallic trim].

d. Finish: [Polished chrome plate] [Polished brass] <Insert finish>.

e. Maximum Flow Rate: [2.5 gpm (9.5 L/min.)] <Insert value> unless otherwise indicated.

f. Mounting: [Exposed] [Concealed].

g. Backflow-Prevention Device for Hand-Held Shower: [Required] [Not required].

h. Operation: [Compression, manual] [Noncompression, manual].

i. Antiscald Device: [Integral with mixing valve] [Separate unit] [Not required].

j. Check Stops: Check-valve type, integral with or attached to body; on hot- and cold-water supply connections.

5. Supply Connections: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)].

6. Shower Head:

a. Type: [Ball joint] [Without ball joint] [Ball joint and head integral with mounting flange] [Integral with mounting flange] [Hand-held, slide-bar mounted] [Hand-held, hook mounted].

b. Shower Head Material: [Metallic] [Nonmetallic] [Combined, metallic and nonmetallic] with chrome-plated finish.

c. Spray Pattern: [Fixed] [Adjustable].

d. Integral Volume Control: [Required] [Not required].

e. Shower-Arm, Flow-Control Fitting: [Not required] [1.5 gpm (5.7 L/min.)] [2.0 gpm (7.6 L/min.)].

2.8 KITCHEN SINKS

Copy "Kitchen Sinks" Paragraph below and re-edit for each type of counter-mounted kitchen sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each kitchen sink.

A. Kitchen Sinks <Insert drawing designation>: [One] [Two] [Three] bowl, counter mounted, [enameled cast iron] [enameled steel] [PMMA] [solid surface] [stainless steel].

1. Enameled Cast-Iron Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Commercial Enameling Company.

2) Eljer, Inc.

3) Kohler Co.

4) <Insert manufacturer's name>.

2. Enameled-Steel Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Bootz Industries.

3) Briggs Plumbing Products, Inc.

4) Eljer, Inc.

5) Mansfield Plumbing Products LLC.

6) <Insert manufacturer's name>.

3. PMMA Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Gerber Plumbing Fixtures LLC.

2) Lyons Industries, Inc.

3) <Insert manufacturer's name>.

4. Solid-Surface Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Aristech Surfaces.

2) DuPont.

3) Formica Corporation.

4) Franke Consumer Products, Inc.

5) InPro Corporation (IPC).

6) Rynone Manufacturing Corp.

7) Swan Corporation (The).

8) Wilsonart International.

9) <Insert manufacturer's name>.

5. Stainless-Steel Kitchen Sinks:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Elkay Manufacturing Co.

2) Franke Consumer Products, Inc.

3) Griffin Products, Inc.

4) Houzer, Inc.

5) Just Manufacturing.

6) Kohler Co.

7) Revere Sink.

8) Sterling.

9) WhiteRock Corp.

10) <Insert manufacturer's name>.

6. Fixture:

Retain one of four "Standard" subparagraphs below.

a. Standard: ASME A112.19.1/CSA B45.2 for enameled [cast-iron] [-steel] kitchen sinks.

b. Standard: ANSI Z124.3 for PMMA kitchen sinks.

c. Standard: ANSI Z124.3 and ANSI/ICPA SS-1 for solid-surface kitchen sinks.

d. Standard: ASME A112.19.3/CSA B45.4 for stainless-steel kitchen sinks.

e. Overall Dimensions: <Insert dimensions>.

Retain "Metal Thickness" Subparagraph below for stainless-steel sinks.

f. Metal Thickness: [0.038 inch (1.0 mm)] [0.050 inch (1.3 mm)] <Insert dimension>.

Retain "Bowl" Subparagraph below for single-bowl sinks.

g. Bowl:

1) Dimensions: <Insert dimensions>.

2) Drain: 3-1/2-inch (89-mm) [crumb cup] [grid] [outlet for disposer].

a) Location: [Centered in bowl] [Near back of bowl] <Insert location>.

Retain "Left Bowl," "Right Bowl," and "Center Bowl" subparagraphs as required for multibowl sinks.

h. Left Bowl:

1) Dimensions: <Insert dimensions>.

2) Drain: 3-1/2-inch (89-mm) [crumb cup] [grid] [grid with offset waste] [outlet for disposer].

a) Location: [Centered in bowl] [Near back of bowl] <Insert location>.

i. Right Bowl:

1) Dimensions: <Insert dimensions>.

2) Drain: 3-1/2-inch (89-mm) [crumb cup] [grid] [grid with offset waste] [outlet for disposer].

a) Location: [Centered in bowl] [Near back of bowl] <Insert location>.

j. Center Bowl:

1) Dimensions: <Insert dimensions>.

2) Drain: [1-1/2-inch (38-mm)] [3-1/2-inch (89-mm)] [crumb cup] [grid] [grid with offset waste].

a) Location: [Centered in bowl] <Insert location>.

7. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

8. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

9. Waste Fittings: Comply with requirements in "Waste Fittings" Article, except include continuous waste for multibowl sinks.

a. Disposer: [Not required] <Insert disposer designation from "Disposers" Article>.

b. Dishwasher Air-Gap Fitting: [Not required] [Comply with requirements in "Dishwasher Air-Gap Fittings" Article].

2.9 LAUNDRY TRAYS

Copy "Laundry Trays" Paragraph below and re-edit for each type of plastic laundry tray required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each laundry tray.

A. Laundry Trays <Insert drawing designation>: Plastic.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Crane Plumbing, L.L.C.

b. Ferguson Enterprises, Inc.; ProFlo Brand.

c. Florestone Products Co., Inc.

d. Gerber Plumbing Fixtures LLC.

e. Mustee, E. L. & Sons, Inc.

f. Swan Corporation (The).

g. Zurn Industries, LLC; Light Commercial Specialty Plumbing Products.

h. <Insert manufacturer's name>.

3. Fixture:

a. Standard: IAPMO/ANSI Z124.6.

b. Style: Flat-rim ledge.

Retain "Plastic" option in "Material" Subparagraph below or retain one or more of other options if a specific material is required.

c. Material: [Plastic] [cast polymer] [FRP] [PMMA] [or] [PP].

d. Nominal Size: [24 by 21 inches (610 by 535 mm)] [25 by 22 inches (635 by 559 mm)] <Insert dimensions>.

e. Color: [White] <Insert color>.

f. Mounting: Freestanding on manufacturer's standard legs or separate, painted-steel stand.

4. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

5. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

6. Waste Fittings: Comply with requirements in "Waste Fittings" Article.

2.10 SINK FAUCETS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Sink Faucets" Paragraph below and re-edit for each type of sink faucet required.

Insert number to complete drawing designation. Use these designations in sink description to identify required faucet for that sink.

B. Sink Faucets <Insert drawing designation>: [Solid brass] [Copper underbody], [kitchen sink] [bar sink] [laundry tray].

1. General-Duty, Solid-Brass Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Bradley Corporation.

3) BrassTech Inc.

4) Briggs Plumbing Products, Inc.

5) Central Brass Company.

6) Chicago Faucets.

7) Danze, Inc.

8) Delta Faucet Company.

9) Eljer, Inc.

10) Elkay Manufacturing Co.

11) Fisher Manufacturing Co.

12) Franke Consumer Products, Inc.

13) Gerber Plumbing Fixtures LLC.

14) GROHE America, Inc.

15) Hansgrohe Inc.

16) Hydrotek International, Inc.

17) Intersan Manufacturing Company.

18) Just Manufacturing.

19) Kohler Co.

20) Moen Incorporated.

21) Price Pfister, Inc.

22) Rohl LLC.

23) Speakman Company.

24) T & S Brass and Bronze Works, Inc.

25) Watermark Designs, Ltd.

26) Wolverine Brass, Inc.

27) Zurn Plumbing Products Group.

28) <Insert manufacturer's name>.

2. General-Duty, Copper-Underbody Faucets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Briggs Plumbing Products, Inc.

3) Delta Faucet Company.

4) Eljer, Inc.

5) Ferguson Enterprises, Inc.

6) Moen Incorporated.

7) Sterling.

8) WhiteRock Corp.

9) Wolverine Brass, Inc.

10) <Insert manufacturer's name>.

3. Standard: ASME A112.18.1/CSA B125.1.

4. General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture holes; coordinate outlet with spout and fixture receptor.

5. Kitchen Sink Option: Separate hand spray complying with ASSE 1025.

6. Finish: [Polished chrome plate] [Polished brass] [Nonmetallic] [Polished or rough brass] [Rough brass] <Insert finish>.

Retain second option in "Maximum Flow Rate" Subparagraph below for laundry-tray faucets.

7. Maximum Flow Rate: [2.5 gpm (9.5 L/min.)] [4.0 gpm (15 L/min.)] <Insert value> unless otherwise indicated.

8. Mixing Valve: [Single control] [Two-lever handle].

9. Backflow-Prevention Device for Hand Spray: [Required] [Not required].

10. Centers: [4 inches (102 mm)] [6 inches (152 mm)] [8 inches (203 mm)] [Single hole] [Adjustable].

11. Mounting: [Deck] [Back/wall], [exposed] [concealed].

12. Handle(s): [Lever] [Knob] [Knob, nonmetallic] [Cross, four arm] [Wrist blade, 4 inches (102 mm)] [Elbow, 6 inches (152 mm)] [Not applicable].

13. Spout Type: [Rigid, solid brass] [Rigid, solid brass with wall brace] [Swing, round tubular] [Swing, shaped tube] [Swing, solid brass] [Rigid gooseneck] [Swivel gooseneck].

14. Spout Outlet: [Aerator] [Swivel aerator/spray] [Spray] [Plain end].

15. Drain: [Stopper with chain] [Grid] [Lift and turn].

2.11 DISHWASHER AIR-GAP FITTINGS

A. Dishwasher Air-Gap Fittings <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. B & K Industries, Inc.

b. Brass Craft Manufacturing; a subsidiary of Masco Corporation.

c. BrassTech Inc.

d. Dearborn Brass.

e. Geberit US.

f. Sioux Chief Manufacturing Company, Inc.

g. Watts Brass & Tubular; a division of Watts Water Technologies, Inc.

h. <Insert manufacturer's name>.

3. Standard: ASSE 1021.

4. Description: Device designed to prevent backflow of contaminated liquid into domestic dishwashers.

5. Material: Plastic body[ with chrome-plated-brass cover].

6. Hose Connections: 5/8-inch- (16-mm-) ID inlet and 7/8-inch- (22-mm-) ID outlet.

7. Capacity: At least 5 gpm (0.32 L/s); at inlet pressure of at least 5 psig (35 kPa) and at temperature of at least 140 deg F (60 deg C).

8. Mounting: Deck.

Retain "Hoses" Subparagraph below if hoses are required.

9. Hoses: Rubber and suitable for temperature of at least 140 deg F (60 deg C).

a. Inlet Hose: 5/8 inch (16 mm) ID and [48 inches (1219 mm)] <Insert dimension> long.

b. Outlet Hose: 7/8 inch (22 mm) ID and [48 inches (1219 mm)] <Insert dimension> long.

2.12 DISPOSERS

Copy "Disposers" Paragraph below and re-edit for each type of food-waste disposer required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each disposer.

A. Disposers <Insert drawing designation>: Continuous-feed household, food waste.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Anaheim Manufacturing, Inc.; a Subsidiary of Western Industries, Inc.

c. Franke Consumer Products, Inc.

d. InSinkErator.

e. KitchenAid.

f. Maytag.

g. WhiteRock Corp.

h. <Insert manufacturer's name>.

3. Standards: ASSE 1008 and UL 430, and listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. General: Include reset button; wall switch; corrosion-resistant chamber with jam-resistant, cutlery- or stainless-steel grinder or shredder; NPS 1-1/2 (DN 40) outlet; quick-mounting, stainless-steel sink flange; antisplash guard; and combination cover/stopper.

5. Model: [Not applicable] [Sound-insulated chamber] [Sound-insulated chamber and stainless-steel outer shell].

6. Motor: 115 V ac, 1725 rpm, [1/3] [1/2] [3/4] [1] hp with overload protection.

2.13 WATER CLOSETS

Copy "Water Closets" Paragraph below and re-edit for each type of gravity-tank, vitreous-china water closet required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each water closet.

A. Water Closets <Insert drawing designation>: Floor mounted, floor outlet, close coupled (gravity tank), vitreous china.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

List of manufacturers below is for gravity-tank, 1.6-gal./flush (6-L/flush) water closets.

a. American Standard America.

b. Barclay Products Limited.

c. Briggs Plumbing Products, Inc.

d. Capizzi.

e. Crane Plumbing, L.L.C.

f. Duravit USA, Inc.

g. Eljer, Inc.

h. Ferguson Enterprises, Inc.; ProFlo Brand.

i. Gerber Plumbing Fixtures LLC.

j. Kohler Co.

k. Lamosa Corp.; Sanitaryware.

l. Mansfield Plumbing Products LLC.

m. Peerless Pottery Sales, Inc.

n. St. Thomas Creations.

o. TOTO USA, INC.

p. Zurn Industries, LLC; Commercial Brass and Fixtures.

q. <Insert manufacturer's name>.

3. Bowl:

a. Standards: ASME A112.19.2/CSA B45.1, ASME A112.19.5, and ASSE 1037.

b. Bowl Type : [Siphon jet] <Insert type>.

c. Height: [Standard] [Child] [Handicapped/elderly].

d. Rim Contour: [Elongated] [Regular].

Retain first option in "Water Consumption" Subparagraph below unless special, low-consumption design is required.

e. Water Consumption: [Water saving] [Low].

f. Color: [White] <Insert color>.

4. Toilet Seat: <Insert toilet-seat designation from "Toilet Seats" Article>.

5. Supply Fittings:

a. Standard: ASME A112.18.1/CSA B125.1.

b. Supply Piping: Chrome-plated-brass pipe or chrome-plated-copper tube matching water-supply piping size. Include chrome-plated wall flange.

c. Stop: Chrome-plated-brass, one-quarter-turn, ball-type or compression stop with inlet connection matching water-supply piping type and size.

1) Operation: [Loose key] [Wheel handle] <Insert type>.

d. Riser:

1) Size: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

2) Material: [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless-steel flexible hose] riser.

Copy "Water Closets" Paragraph below and re-edit for each type of flushometer-tank, vitreous-china water closet required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each water closet.

B. Water Closets <Insert drawing designation>: Floor mounted, floor outlet, close coupled (flushometer tank), vitreous china.

1. Sloan's "Flushmate" Flushometer-Tank, 1.0-Gal./Flush (3.8-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Capizzi.

2) St. Thomas Creations.

3) <Insert manufacturer's name>.

2. Sloan's "Flushmate" Flushometer-Tank, 1.6-Gal./Flush (6-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) American Standard America.

2) Ferguson Enterprises, Inc.

3) Gerber Plumbing Fixtures LLC.

4) Kohler Co.

5) Mansfield Plumbing Products LLC.

6) St. Thomas Creations.

7) <Insert manufacturer's name>.

3. Geberit's "PF/2 Energizer" Flushometer-Tank, 1.6-Gal./Flush (6-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Crane Plumbing, L.L.C.

2) Eljer, Inc.

3) Peerless Pottery Sales, Inc.

4) <Insert manufacturer's name>.

4. Briggs's Vacuum-Assist, Flushometer-Tank, 1.6-Gal./Flush (6-L/Flush) Water Closets:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Briggs Plumbing Products, Inc.

2) <Insert manufacturer's name>.

5. Bowl:

a. Standards: ASME A112.19.2/CSA B45.1 and ASME A112.19.5.

b. Bowl Type: [Siphon jet] <Insert type>.

c. Height: [Standard] [Child] [Handicapped/elderly].

d. Rim Contour: [Elongated] [Regular].

Retain first option in "Water Consumption" Subparagraph below unless special, low-consumption design is required.

e. Water Consumption: [Water saving] [Low].

f. Color: [White] <Insert color>.

6. Toilet Seat: <Insert toilet-seat designation from "Toilet Seats" Article>.

7. Supply Fittings:

a. Standard: ASME A112.18.1/CSA B125.1.

b. Supply Piping: Chrome-plated-brass pipe or chrome-plated-copper tube matching water-supply piping size. Include chrome-plated wall flange.

c. Stop: Chrome-plated-brass, one-quarter-turn, ball-type or compression stop with inlet connection matching water-supply piping type and size.

1) Operation: [Loose key] [Wheel handle] <Insert type>.

d. Riser:

1) Size: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

2) Material: [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless-steel flexible hose] riser.

2.14 TOILET SEATS

Copy "Toilet Seats" Paragraph below and re-edit for each type of toilet seat required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each toilet seat.

A. Toilet Seats <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Bemis Manufacturing Company.

c. Centoco Manufacturing Corporation.

d. Church Seats.

e. Eljer, Inc.

f. Ferguson Enterprises, Inc.; ProFlo Brand.

g. Jones Stephens Corp.; Comfort Seat Brand.

h. Kohler Co.

i. Olsonite Seat Co.

j. Pressalit Inc.

k. Sanderson Plumbing Products, Inc.; Beneke Div.

l. Sperzel of Lexington.

m. <Insert manufacturer's name>.

3. Standard: IAPMO/ANSI Z124.5.

4. Material: Plastic.

5. Type: [Residential] [Commercial (Standard)].

6. Shape: [Regular rim (Open front)] [Regular rim (Closed front)] [Elongated rim (Open front)] [Elongated rim (Closed front)].

7. Configuration: [Closed] [Open] front [with] [without] cover.

8. Size: [Elongated] [Regular].

9. Hinge Type: [Check] [Self-sustaining] [Self-sustaining, check] [Self-raising].

10. Hinge Material: [Noncorroding metal] [Plastic].

11. Seat Cover: [Required] [Not required].

12. Color: [White] [Black] <Insert color>.

2.15 SUPPLY FITTINGS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

B. Standard: ASME A112.18.1/CSA B125.1.

C. [Bidet] [Lavatory] [Kitchen Sink] [and] [Laundry Tray] Supply Fittings:

1. Supply Piping: Chrome-plated-brass pipe or chrome-plated-copper tube matching water-supply piping size. Include chrome-plated wall flange.

2. Stops: Chrome-plated-brass, one-quarter-turn, ball-type or compression stop with inlet connection matching water-supply piping type and size.

a. Operation: [Loose key] [Wheel handle] <Insert type>.

3. Risers:

a. Size: NPS 3/8 (DN 10) for [bidets] [and] [lavatories].

b. Size: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)] for [kitchen sinks] [and] [laundry trays].

c. Material: [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless-steel flexible hose] riser.

2.16 WASTE FITTINGS

A. Standard: ASME A112.18.2/CSA B125.2.

B. Drain: Grid type with NPS 1-1/4 (DN 32) offset tailpiece for accessible lavatories.

C. Drain: Pop-up type with NPS 1-1/4 (DN 32) straight tailpiece as part of faucet for standard lavatories.

D. Drain: Grid type with NPS 1-1/2 (DN 40) offset tailpiece for accessible kitchen sinks.

E. Drain: Grid type with NPS 1-1/2 (DN 40) straight tailpiece for standard [kitchen sinks] [and] [laundry trays].

F. Trap:

1. Size: [NPS 1-1/4 (DN 32)] [NPS 1-1/2 by NPS 1-1/4 (DN 40 by DN 32)] [NPS 1-1/2 (DN 40)] for [bidets] [and] [lavatories].

2. Size: NPS 1-1/2 (DN 40) for [kitchen sinks] [and] [laundry trays].

Retain one of three "Material" subparagraphs below.

3. Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [one-piece, cast-brass trap with swivel 0.029-inch- (73-mm-) thick tubular brass wall bend] <Insert trap type>; and chrome-plated-brass or -steel wall flange.

4. Material: Stainless-steel, two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick stainless-steel tube to wall; and stainless-steel wall flange.

5. Material: ASTM F 409 [ABS] [or] [PVC] [one] [two]-piece trap and waste to wall and wall flange.

2.17 GROUT

A. Standard: ASTM C 1107/C 1107M, Grade B, post-hardening and volume-adjusting, dry, hydraulic-cement grout.

B. Characteristics: Nonshrink; recommended for interior and exterior applications.

C. Design Mix: 5000-psi (34.5-MPa), 28-day compressive strength.

D. Packaging: Premixed and factory packaged.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Install plumbing fixtures level and plumb according to roughing-in drawings.

B. Install floor-mounted water closets on closet flange attachments to drainage piping.

C. Install counter-mounting fixtures in and attached to casework.

D. Install pedestal lavatories on pedestals and secured to wood blocking in wall.

E. Install water-supply piping with stop on each supply to each fixture to be connected to water distribution piping. Attach supplies to supports or substrate within pipe spaces behind fixtures. Install stops in locations where they can be easily reached for operation.

1. Exception: Use ball, gate, or globe valves if supply stops are not specified with fixture. Comply with valve requirements specified in Section 220523 "General-Duty Valves for Plumbing Piping."

F. Install tanks for accessible, tank-type water closets with lever handle mounted on wide side of compartment.

G. Install toilet seats on water closets.

H. Install faucet flow-control fittings with specified flow rates and patterns in faucet spouts if faucets are not available with required rates and patterns. Include adapters if required.

I. Install shower flow-control fittings with specified maximum flow rates in shower arms.

J. Install traps on fixture outlets.

1. Exception: Omit trap on fixtures with integral traps.

2. Exception: Omit trap on indirect wastes unless otherwise indicated.

K. Install disposer in outlet of each sink indicated to have disposer. Install switch where indicated or in wall adjacent to sink if location is not indicated.

L. Install dishwasher air-gap fitting at each sink indicated to have air-gap fitting. Install [in sink deck] [on countertop at sink] <Insert location>. Connect inlet hose to dishwasher and outlet hose to disposer.

M. Set [bathtubs] [and] [shower receptors] in leveling bed of cement grout.

Indicate on Drawings those lavatories and sinks that are required to be accessible.

N. Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible [lavatories] [and] [sinks]. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

O. Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons if required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

P. Seal joints between plumbing fixtures, counters, floors, and walls using sanitary-type, one-part, mildew-resistant silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

3.2 CONNECTIONS

Coordinate piping installations and specialty arrangements with schematics on Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

B. Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

C. Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

D. Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible [lavatories] [and] [sinks]. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

3.3 ADJUSTING

A. Operate and adjust plumbing fixtures and controls. Replace damaged and malfunctioning fixtures, fittings, and controls.

B. Adjust water pressure at faucets to produce proper flow.

3.4 CLEANING AND PROTECTION

A. After completing installation of plumbing fixtures, inspect and repair damaged finishes.

B. Clean plumbing fixtures, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

C. Provide protective covering for installed plumbing fixtures and fittings.

D. Do not allow use of plumbing fixtures for temporary facilities unless approved in writing by Owner.

END OF SECTION 224100

	RESIDENTIAL PLUMBING FIXTURES
	224100 - 1



